Chapter

18

# German Aviation Gasoline 1939-1945

# Part 1

Photo 1. German Messerschmidt Bf-109-G-2 on display at RAF Hendon, London 2005.

Table of Contents

Summary	4

German Aviation Gasolines	5

The Uniqueness Of German Aircraft	5

Germany’s Innovative Chemists And Engineers	6

Understanding The Enemy	6

Are The Germans Ahead In Avgas? - Gasoline Composition 1941	8

Spoils Of War – Captured Oil Supplies And Facilities	24

Find Out About The German Oil Industry ! 	25

Bios Overall Report No. 1 - Report On The Petroleum And Synthetic Oil Industry Of Germany	34

Avgas Composition, Specifications & Test Methods 1941	38

Bios Misc. Report No. 72 Manufacture Of Aviation Gasoline In Germany	47

Gasoline From Coal	50

German Chemical Processes	51

1. Synthesis Of Iso-Paraffins	51

2. Synthesis Of Aromatics And Production Of Base Stocks	53

Conclusions On German Production	57

Oil Supplies, Refineries And Plants	58

Oil Refineries	69

German Industry Goes Underground	76

“Target – Destroy Germany’s Oil Industry”	77

Famous Scientist To U.S.	84

“Airspeed Records  - Germany Temporarily Takes The Record In 1944”	85

Epilogue For The Axis Years – The Have’s And Have Nots	86

Index	88

Research Sources	91

# Summary

Chronology

1930’s	German oil industry develops gasoline production

1934	Formation of Wirtschaftliche Forschungsgesellschaft m.b.H. (WIFO)

Sept 1, 1939	Second World War starts. German Luftwaffe thought to be superior.

1940	Captured avgas stocks add to German supplies

Maximum German avgas production and consumption.

1943	Commercial ‘Alkylation’ plants operate in Germany

German oil industry under constant bombing and goes underground

1943-44	First operational Jet Aircraft comes into service, but too late to save Germany from defeat

Aug.-Sept 1944	Axis supplies from Rumanian refineries and captured territories are lost.

1945	German oil industry collapses

Photo  2. Daimler Benz DB-605B (RAF Museum Hendon 2005) as used in the famous Messerschmidt Bf-109.

# German Aviation Gasolines

A view from the US in 1924 was as follows:

“We are more accustomed to a type of general publicity statement such as appeared in a recent daily paper: Chemists in Germany are said to have discovered a process for producing synthetic gasoline at a considerably smaller cost than regular gasoline. The artificial gasoline is said to be equal in every respect to the natural product in regard to chemical composition and utility. It is produced largely from by-products of oils, coke, etc., which were heretofore wasted.”

# German Aviation Gasolines

Innovation followed by desperation.

After the First World War in the 1920’s and early 1930’s, Germany was in a state of turmoil – frequent changes in government as the nation struggled with democracy, civilian unrest and the rise of Communism in the East. With the rise of the National Socialist Party (the Nazi Party) to power in 1933, some sense of order began to occur, and as the country appeared to become more settled and the demand for petroleum products rose it became part of the world oil marketing area. Petroleum products were supplied from the Balkans, Russia and North Africa. The aviation gasoline specifications were generally uniform around Europe.

Also at this time, industry (including those supplying the military) began to expand and new chemical plants were brought into operation. This was all to change with the rise of Nazism and military expansion and led to the military quest for the oil supplies of the Balkans and the North Africa and the Middle East.

# The Uniqueness of German Aircraft

With great advances in aircraft design and performance in the 1930’s both in Germany and elsewhere in the world, the German aircraft designs - a prelude to military use - were based on essentially on the existing aviation fuels available which were 87 Octane (MON). These could be manufactured from coal to oil conversion processes. In some regards the German aircraft engine manufacturers were in advance of the US, Britain and France with such innovations as fuel injection (which eliminated a significant problem of carburettor icing and momentary cut-out during inversion of the aircraft). The Germans (and also the British) aircraft fuel systems had been developed from the days when the fuels were high in aromatic content, and this was also the case with aviation gasolines produced from coal to oil conversion processes. This also applied to the synthetic rubber industry which was also developing in Germany.

With the expansion of long distance travel in the 1930’s German passenger and cargo aircraft would be refuelled at locations outside of German aviation gasoline production, and that the fuel would be of a quality specified by the airlines and oil companies, usually the aviation services companies such as INTAVA, STANAVO and Shell Aviation Services. The source would be from that obtained by those oil companies for example in the UK it would be from the Middle East and East Indies. Thus, the development of the German aviation gasoline production, composition and specification would be driven by the demands of the Axis Forces and limited by the raw materials available.

# Germany’s innovative chemists and engineers

Germany had always enjoyed a reputation for technical innovation and inventions; indeed, many of the chemical processes were started in this country. The large supplies of coal and practically no crude oil reserves directed the chemists’ interest to coal to oil conversion processes very early on the path to war. Their aviation fuel production was essentially one of a Synthetic Fuel industry.

# Understanding the Enemy

The coordination of production and supply of petroleum products including aviation fuels was done at the most senior level in the US and British governments and involved the Chiefs of Staff of the armed services. At the various meetings, there were usually a number of items which dealt with the quality of the aviation gasoline produced by the Axis. These matters were handled by a Technical Panel. Here the quality of German Avgas was discussed:

“The fuels are all good quality hydrocarbon aviation spirit containing 5.5 mls. Tetra Ethyl Lead per gallon (Imp Gal) blended to 90 to 94 Octane Number (MON).

The 90 Octane fuels which predominate, are a mixture of petroleum spirit with one or more of the following:

30-50% Hydro-petrol (from hydrogenation of coal or creosote)

5-10% Synthetic Octanes

15% Aromatics (from coal tar or synthetics)

The 94 Octane fuels (used in Messerschmitt 110’s) contain

30-40% Aromatics

Approx. 10% Octanes

50-60% Petroleum Spirits

The petroleum spirit may possibly be Roumanian, but may also be drawn from reserves accumulated before the war.

However, the number of samples obtained is too small to allow definite conclusions on composition trends. The essential differences from British fuels are the lead content which is normally 5.5 cc TEL/IG and the absence of 100 O.N. grade.

Vapour pressure which is normally 5-6 lbs (PSI) (i.e. 1½ lbs lower than British maximum specification) though this may be due to weathering during flight, and secondly a tendency in high aromatic fuels to allow reduction in volatility, this being necessary as up to 25% of Xylene or heavier is used as the aromatic addition.”

In another memo the following outline was given

File S 1372/A.1.9. (L) Memo from D.A.C. Dewdren, 27 March 1941

“General Description of Fuels British & German

British – Over the first year of the war there was a drop in Octane from 100 to 90, and appearance of 5% alcohol. The ON drop was due to the convenience of distributors, and 5% alcohol was due to carburettor icing troubles.

Germans – Up to August 1940 the Germans were using for all types of aircraft on 89-91 ON fuel containing 5.5 cc TEL/IG and consisting of 30-60% hydro-petrol ex bituminous coal or tar, blended with 40-70% straight run petrol spirit which has been obtained from either Roumanian or Venezuelan sources.

About August/Sept 1940, the German appeared to experiment with a variety of fuel brews being aromatics and iso-octanes of various types, as well as hydro-petrol as blending constituents.”

Committee on Enemy Oil Position 6th Report May 1941 for UK Cabinet

Page 5 Item 7

“All captured samples (of Aviation Spirit) so far are blends containing up to 50-60% of straight run petroleum spirit possibly of Roumanian origin or from pre-war reserves indicating that the dependence on hydrogenation plants is not yet so great as anticipated. However, these plants are the largest potential source of aviation spirit up to the end of 1940.”

Table 1. List of German Synthetic Oil Refineries.

| Synthetic Refineries | Refineries |
| --- | --- |
| Leuna | Hamburg |
| Gelsenkirchen – Hydrogenation plant | Misburg |
| Pölitz | Bremen |
| Scholven – Hydrogenation plant |  |

Petroleum Products were at a premium

While the focus has been on aviation fuels it should not be forgotten that lubricating oils were also important to ensure that the “War Machine’ for both the Allies and the Axis was ‘lubricated’. Lack of lubricating oils and grease, or unsuitable lubricants could literally stop an army in its tracks, as the German Panzers found out to their peril in the Eastern Front in the fierce Russian winter of 1943, where unsuitable lubricants froze and made their tanks useless. Since generally lubricating oils were derived from the processing of crude oil, any alternate source of ‘lubricant’ was always of interest to both the Allies and the Axis. Lube Oils “Voltol” derived from electrically treated rape oil was successfully used as aircraft engine lubricant.

# Are the Germans ahead in Avgas? - Gasoline Composition 1941

One area of military intelligence dealt with technical expertise, and determining was the ‘enemy’ ahead technically? As with all military intelligence much information can be obtained from the capture of enemy war materiel – this not only meant aircraft, but also extended to fuels and lubricants. In order to determine that status of the German aviation gasoline and lubricant quality, if possible whenever an enemy aircraft crashed, or enemy fuel supply dumps were captured, samples were taken and analysed to answer this question.

The following are some examples of the information obtained by the British from aircraft shot down over England in the 1941. The data source is UK PRO File POWE 33/1470 and a selection of examples is listed covering the two main German aviation grades B4 and C3.

Photo 3. Crashed Heinkel He-111 – a valuable source of military intelligence

Table 2. German Avgas from captured aircraft 1941 (Part 1) Composition Blue B4 Grade

| Sample | Not stated | 85 | 81 |
| --- | --- | --- | --- |
| Date | Not stated | 4 March 1941 | 3 March 1941 |
| Source | Junkers Ju 88 | Junkers Ju 88 | Heinkel He-111 |
| Crash Date | 7 Dec 1940 | 16 Feb 1941 | 16 Feb 1941 |
| Colour | Blue | Blue | Blue |
| Specific Gravity | 0.7395 | 0.7415 | 0.7375 |
| Octane No. (CFR) MON | 89.5 | 91.3 | 90.5 |
| Octane Base (lead free) | 73 | 73.6 | 71.9 |
| Octane base (lead & aromatic free) |  | 69.8 | 69.0 |
| RVP psi | 5.9 | 6.1 | 6.1 |
| cc TEL/IG | 5.6 | 5.44 | 5.51 |
| Freezing point Deg C | <-60C |  |  |
| Sulphur % Free mg/100ml |  |  |  |
| Sulphur % Total | 0.004% | 0.016 | 0.019 |
| Existent Gum mg/100ml | 2 | 2 | 2 |
| Potential Gum mg/100ml |  |  |  |
| Induction Period | 525 mins |  |  |
| Bromine No. | 2 |  |  |
| Initial Boiling Point deg C | 41.5 | 43 | 45 |
| 5% |  | 60 | 62 |
| 10% |  | 65.5 | 67 |
| 20% |  | 73.5 | 74.5 |
| 30% |  | 81 | 82 |
| 40% |  | 88.5 | 89 |
| 50% |  | 95.5 | 95.5 |
| 60% |  | 102.5 | 102 |
| 70% |  | 111 | 109 |
| 80% |  | 121 | 119.5 |
| 90% |  | 134 | 132.5 |
| 95% |  | 147 | 143.5 |
| Final Boiling Point deg C | 157 | 158 | 158.5 |
| % Loss | 1 | 1 | 1 |
| % Distilled 70C |  | 16 | 14 |
| % Distilled 75C | 23.5 | 22 | 20 |
| % Distilled 100C | 59 | 56 | 57 |
| % Distilled 140C |  | 92.5 | 93.5 |
| % Distilled 150C | 91.5 | - | - |
| Paraffins % Volume |  |  |  |
| Iso Butane |  | 0.12 | 1.21 |
| n-Butane |  | 1.41 | 1.32 |
| Iso Pentane |  | 8.57 | 4.56 |
| n-Pentane |  | 4.37 | 6.26 |
| Methyl pentanes |  | 8.33 | 7.7 |
| n-Hexane |  | 4.04 | 5.1 |
| Methyl Hexanes |  | 5.22 | 6.12 |
| n-Heptane (& iso-octane) |  | 1.88 | 1.96 |
| Highly branched Octanes |  | 1.14 | 1.52 |
| Single branched Octanes |  | 2.77 | 3.74 |
| n-Octane |  | 0.5 | 0.94 |
| Double branched nonanes |  | 0.58 | 1.27 |
| Branched nonanes |  | 1.12 | 1.66 |
| Branched decanes |  | 0.28 | 0.7 |
| Naphthenes |  |  |  |
| Cyclopentane |  | 0.77 | 0.88 |
| Methyl Cyclopentane |  | 7.99 | 6.97 |
| Cyclohexane |  | 4.24 | 3.85 |
| Dimethyl cyclopentane |  | 4.52 | 5.46 |
| Methyl cyclohexane (& Cyclo C5) |  | 12.48 | 11.99 |
| Dimethyl Cyclo C6 (& Cyclo C5) 110-125C |  | 5.31 | 5.65 |
| 125-137C |  | 4.75 | 4.34 |
| 137-153C |  | 3.92 | 2.74 |
| >153C |  | 3.46 | 4.15 |
| Aromatics |  |  |  |
| Benzene | 3.5 | 2.04 | 2.54 |
| Toluene | 3.5 | 4.06 | 3.29 |
| Xylenes | 3.5 | 4.53 | 2.66 |
| 145-166C, >160C | 1.5 | 1.6 | 1.42 |
| Aromatics | 11.5 | 12.23 | 9.91 |
| Paraffins | 44 | 40.33 | 44.06 |
| Naphthenes | 43.5 | 47.44 | 46.03 |
| Unsaturates | 1 |  |  |
| Iron Carbonyl | Nil |  |  |
| Phenols | 0.018 |  |  |
| Water Solubility | Nil |  |  |

Photo 4. Junkers Ju-87 Stuka comes to a fiery end in the Western Desert 1941.

Photo 5.  German Luftwaffe Junkers Ju-88 – Inspection of crashed enemy aircraft

Table 3. German Avgas from captured North Africa stocks 1942.

| Sample | Gibraltar R/S 157 | Gibraltar R/S 193 |
| --- | --- | --- |
| Report No. | AIR 164 | AIR 165 |
| Date | Circa 1942 | Circa 1942 |
| Source | Laraiche Morocco | Tetuan Morocco |
|  | ex-Military stocks | ex-Military stocks |
| Colour | Pale Blue | Pale Blue |
| Specific Gravity | 0.737 | 0.7355 |
| Initial Boiling Point Deg C | 44 | 40 |
| %Rec. to 75C | 14% | 15% |
| %Rec. to 100C | 50% | 50% |
| %Rec. to 150C | 95% | 96% |
| Final Boiling Point Deg C | 159 | 168 |
| Total distilled | 98% | 98.5% |
| Loss | 1% | 1% |
| TEL | Yes | Not Confirmed |
| Ethylene Dibromide; Cracked spirit; Cracked spirit. | Yes | Not Confirmed |
| Bromine No. | 11.5 | 9.8 |
| Initial Boiling Point deg C | 44 | 40 |
| Final Boiling Point deg C | 159 | 168 |
| % Loss | 1% | 1% |
| Aromatics | 10% | 10% |
| Paraffins | 54.5% | 55% |
| Naphthenes | 29% | 29.5% |
| Unsaturates | 6.5% | 5.5% |

These fuels from Morocco were markedly different to the typical German B4 fuel and were probably from Italian stocks made from straight run gasoline.

Table 4. German Avgas from captured fighter aircraft 1941 (Part 2) Green C3 Grade

| Report No. | AIR 118 | AIR 141 | AIR 144 |
| --- | --- | --- | --- |
| Source | ME-109 | ME- 109 E7 | ME-109F |
| Crash Date | 8 May 1941 | 9 June 1941 | 10 July 1941 |
| Colour | Green fluorescent | Greenish Yellow Strong green bloom | Light Jade Green |
| Specific Gravity | 0.7905 | 0.785 | 0.7835 |
| Octane No. (CFR) MON | 93 | 93 | 93 |
| Octane Base (lead free) | 81 | 82 | 82 |
| IBP Deg C | 56 | 53 | 55 |
| %Rec. to 75C | 4% | 6.5% | 5.5% |
| %Rec. to 100C | 35% | 35% | 42% |
| %Rec. to 150C | 79% | 90.5% | 89.5% |
| Final Boiling Point Deg C | 193 | 174 | 171 |
| Total distilled | 97% | 98% | 97.5% |
| Loss | 1% | 1% | 1% |
| Residue | 2% |  |  |
| RVP psi | 2.1 | 4.2 | 3.3 |
| cc TEL/IG | 5.7 | 5.2 | 5.53 |
| Freezing point Deg C | <-60C | <-60C |  |
| Sulphur % Free mg/100ml | 4 | <1 | 4.2 |
| Sulphur % Total | 0.01% | 0.01% | 0.025% |
| Existent Gum mg/100ml | 1650 (lube oil, rubbery) | 15 (lead deposit) | 10 (grease trace) |
| Induction Period | 22 hrs | 18 hrs |  |
| Bromine No. | 0.9 | 2.2 | 2 |
| Benzene | 7% | 9% | 7.5% |
| Toluene | 9.5% | 13% | 17.5% |
| Xylenes | 7.5% | 12% | 11% |
| 145-166C, >160C | 16% | 8% | 6% |
| Aromatics | 40% | 42% | 41.5% |
| Paraffins | 32.5% | 33% | 31.5% |
| Naphthenes | 27.5% | 24% | 26% |
| Unsaturates |  | 1% | 1% |
| Iron Carbonyl | Nil | Nil | Nil |
| Phenols | 0.0002% | 0.0008% | 0.00006% |
| Water Solubility | Nil | Nil | Nil |
| Nitrogen Bases (as Aniline) | 0.007% | 0.032% |  |
| Ethylene Dibromide |  | 1.05 T |  |

The ‘Green fluorescent’ and ‘Strong green bloom’ are most likely due to the high aromatic hydrocarbon content.

The distillation curves of the German grades B4 and C3 reveal the distinctly different sources. The C3 grade (green) with its high aromatic content and higher specific gravity has a much higher distillation curve.

Graph 1. Distillation curves of German Aviation Grades 1941

In answer to the Allies question – Are the Germans ahead in avgas performance? The answer based on the captured samples was – “not quite”. However, the B4 grade was slightly higher in octane (89 MON) than the equivalent British Specification 230 of the time which specified a minimum octane of 87 MON. The C3 grade was only 93 MON, because of the high aromatic content it was giving a performance equivalent to the Allied 100 ON grade. Both B4 and C3 grades were higher in TEL content than the Allied gasolines (German fuels 5.5 cc TEL/IG compared to an Allied specification of 4 maximum, typically 3.8 cc TEL/IG).  This will be discussed further on.

Of note was the absence of ‘100 octane fuels’ and the low content of highly branched octanes, which indicated that the German Axis were not yet using Alkylation processes to produce aviation gasoline. This would be confirmed after the war by various technical reports from the US Armed Force although they were producing iso-octanes. (see below).

Photo 6. Messerschmidt Bf-109 exits from the battle

[One obscure US reference was found which stated in 1943 “examination of fuel from certain captured German planes showed the German aviation gas to be 145 grade (under rich mixture conditions). This caused some persons in the AAF (US Army Air Force) to argue that the performance number for US fuel should be increased from 130 to 140” . Certainly, high aromatic fuels as used by the Germans could account for the higher rich rating, however one suspects that other agendas may have been in play by the US Army Air Force to ensure production of higher grades 145 PN for the new ‘super’ bombers (B-29) being designed and prototyped.]

The composition of the German aviation gasolines can be illustrated by the PONA distribution which is markedly different to the Allies aviation gasoline which were lower in naphthenes and aromatics. The high percentage of naphthenes in B4 grade is characteristic of synthetic gasolines from hydrogenation plants. The high aromatics content of C3 grade >40% of which most is Benzene, Toluene and Xylene, would account for a high rich rating.

Graph 2. German Avgas B4 Grade 1941 (Part 1) - PONA Distribution

Grade B4 would be used in light and medium bomber aircraft such as the Heinkel 111 powered by two Daimler Benz DB-601 engines, the twin engined Junkers Ju-88, and the famous Junkers Ju-87 Stuka dive bomber. Grade B4 would also be used for transport aircraft such as the Junkers Ju-52/3M originally powered by three Pratt & Whitney Hornet radial engines, later production models of the Ju-52 mainly received BMW 132 engines, a refinement of the Pratt & Whitney design.

Photo 7. Junkers Ju-52/3M transport powered by three BMW 132N motors. Lufthansa’s main passenger aircraft.

Graph 3. German Avgas [C3] grade 1941 PONA Distribution

C3 grade would be used in high performance aircraft such as fighters Messerschmidt Bf-109, which was the main German frontline fighter in 1941 powered by a Daimler Benz DB-605B.

Photo 8. Messerschmidt Bf-109F-1 of Jagdgeschwader on a channel coast airfield in the summer of 1941.

Many of the remaining Messerschmidt Bf-109 examples in museums have been refurbished and marked with 87 octane triangle, however since none are in their original condition, and some may have been used after the war by the Allied forces, these markings may not be relied upon. For example:

Photo 9. Messerschmidt Bf-109 with 87 Octane markings near the fuel tank (Hendon 2006)

Photo 10. Messerschmidt Bf-109 with 100 Octane markings near the fuel tank (Duxford 2005).

Photo 11. German airfield refuel tanker drawn by a tractor, circa 1940.

This refuel tanker was similar to those used by the Allies, and would be similar to those used in peacetime before WWII. A Junkers Ju-52 sits behind the tanker.

Photo 12. A flight of Messerschmidt Bf-110’s on patrol circa 1940.

Table 5. German Avgas from captured aircraft 1941 (Part 1) Blue B4 Grade

| Report No. | AIR 126 | AIR 106 | AIR 108 | AIR 122 | AIR 133 | AIR 134 | AIR 127 | 137 | AIR 146 | AIR 162 | AIR 152 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | Bucker | Heinkel He-111 | Fokker G1 | Heinkel He-111 | Heinkel He-111 | Junkers Ju-88 A-1 | Heinkel He-111 H-5 | Heinkel He-111 | Junkers Ju-88 A-6 | Heinkel He-111 | Dornier Do-217E |
| Crash Date | 30 April 1941 | 3 May 1941 | 5 May 1941 | 6 May 1941 | 8 May 1941 | 8 May 1941 | 22 May 1941 | 5 June 1941 | 24 July 1941 | 24 Nov 1941 | 12 Dec 1941 |
| Colour | Blue | Blue | Blue | Blue | Blue | Blue | Blue | Blue, Clear & Bright | Blue | Blue | Blue |
| Specific Gravity | 0.733 | 0.736 | 0.738 | 0.741 | 0.745 | 0.743 | 0.734 | 0.74 | 0.739 | 0.7455 | 0.7325 |
| Octane No. (CFR) MON | 93 | 90 | 89.5 | 89.4 | 89.5 | 89.5 | 91 | 91 | 91 | 90 | 91 |
| Octane Base (lead free) | 76 | 72.5 | 71 | 71.7 | 71 | 70 | 73 | 72 | 73 | 73 | 74 |
| IBP Deg C | 48 | 47 | 46 | 41 | 49 | 48 | 47 | 45 | 46 | 52 | 49 |
| %Rec. to 75C | 17.5% | 18.5% | 19% | 20% | 17.5% | 17.5% | 16% | 23% | 22% |  | 18.5% |
| %Rec. to 100C | 59.5% | 54% | 52% | 54.5% | 52.5% | 56.5% | 53% | 59% | 58.5% | 51% | 55.5% |
| %Rec. to 150C | 97% | 96.5% | 93% | 89% @135C | 96.5% |  | 96% | 97% | 98% | 92% | 96.5% |
| Final Boiling Point Deg C | 153 | 156 | 165 | 160 | 156 | 156 | 162 | 157 | 154 | 168 | 159 |
| Total distilled | 97.5% | 97.5% | 97.5% |  | 98% | 98% | 98% | 98.5% |  |  |  |
| Loss | 1% | 1% | 1% | 1% | 1% | 1% | 1% | 1% | 1% | 1% | 1% |
| RVP psi | 5.3 | 5.5 | 6.7 | 6.3 | 5 | 5.4 | 5.65 | 5.85 |  | 5 | 5.8 |
| cc TEL/IG | 5.44 | 5.2 | 5.54 | 5.52 | 5.71 | 5.6 | 5.4 | 5.4 | 5.45 | 5.6 | 5.59 |
| Freezing point Deg C | <-60C | <-60C | <-60C |  | <-60C | <-70C | <-60C | <-60C | <-60C | <-60C | <-60C |
| Sulphur % Free mg/100ml | 0.3 | 0.1 | 0.1 |  | 0.1 |  | 1 | <1 | <1 | 0.1 | 0.1 |
| Sulphur % Total | 0.021% | 0.022% | 0.019% | 0.005% | 0.018% | 0.015% | 0.01% | 0.01% | 0.01% | 0.02% | 0.015% |
| Existent Gum mg/100ml | 1.5 | 4.5 greasy | 2 | 2 | 4.5 greasy | 30 (Oil) | 6 rubbery | 5 | 2 | 2 | 1 |
| Potential Gum mg/100ml | Trace |  | 0.5 |  | Trace |  |  |  |  | Trace | Nil |
| Induction Period |  |  |  | 560 mins |  | 370 mins | 15.5 Hrs | 12 hrs | 13 hrs |  |  |
| Bromine No. | 1.5 | 1.3 | 2.5 | 3 |  | 1 | 1 | 1.5 | 1.3 | 1.6 | 1.2 |
| Benzene | 2.5% | 1.5% | 2.5% | 2.6% |  | 5.1% | 1.5% | 3.5% | 4.5% | 2 | 1.5% |
| Toluene | 6% | 4.5% | 7.5% | 5.4% |  | 4.9% | 4.5% | 4% | 5.5% | 7 | 5% |
| Xylenes | 4.5% | 4.5% | 7% | 6.3% |  | 3.9% | 7% | 3% | 6.5% | 8 | 5% |
| 145-166C, >160C | - | 1.5% | 1.6% | 2.3% |  | 1.5% | - | - |  | 3.5 | 0.5% |
| Aromatics | 13.5% | 12% | 18% | 16.4% | 11% | 15.3% | 13% | 10.5% | 16.5% | 21.5% | 13% |
| Paraffins | 54.5% | 53% | 49% | 47.2% | 44% | 47.9% | 56% | 46.5% | 52% | 49.5% | 52% |
| Naphthenes | 31% | 34.5% | 31.5% | 34.7% | 44% | 36.2% | 31% | 43% | 31.5% | 29% | 35% |
| Unsaturates | 1% | 0.5% | 1.5% | 1.7% | - | 0.6% |  | - |  |  |  |
| Alcohol |  |  |  |  |  |  |  |  |  | Nil |  |
| Iron Carbonyl | Not detected | Not detected | Not detected | Nil | Nil | Nil | Nil | Nil | Nil | Undetected | Not detected |
| Phenols | 0.00016% | 0.0026% | 0.00005% | 0.012% | 0.0026% | 0.007% | 0.0023% | 0.0053% | 0.0034% | 0.002% | 0.0027% |
| Water Solubility | Nil | Nil | Nil | Nil | Nil | Nil | Nil | Nil | Nil |  | Nil |
| Nitrogen Bases (as Aniline) |  |  |  |  |  |  | 0.0025% | 0.003% | 0.001% | Typical of Blue class |  |
| Ethylene Dibromide |  |  |  |  |  | - |  | 1.0 T |  |  |  |

# Spoils of War – Captured Oil supplies and facilities

With the German Blitzkrieg advances in the west came the capture of enemy (French) aviation gasoline stored supplies and oil plants. In the east came the Balkan oil resources, and since Rumania was an ally of Germany then these oil supplies of product and crude continued.

German refuelling

The German refuelling operations were much the same as the Allies. Gasoline was shipped in rail tank cars and then transferred to usable jerry-cans for transport by truck to the operational areas.

Photo 13. German soldiers fill jerry cans from rail tank car prior to loading trucks.

Photo 14. German soldiers fill jerry cans from rail tank car

# Find Out About The German Oil Industry!

US Strategic Bombing Survey

Toward the end of the war while the Allies were advancing through Europe and to the final defeat of the Germany, the effectiveness of their Strategic Bombing strategy would be examined. The United States Strategic Bombing Survey was established by the US Secretary of War on 3 November 1944. The organisation comprised 300 civilians, 350 officers and 500 enlisted men. The Survey operated from headquarters in London and established forward headquarters and regional headquarters in Germany immediately following the advance of the Allied armies. It made a close examination and inspection of several hundred German plants, cities and areas, amassed volumes of statistical and documentary records; and conducted interviews and interrogations of thousands of Germans, including virtually all of the surviving political and military leaders. Germany was scoured for its war records, which were found sometimes, but rarely, in places where they ought to have been; sometimes in safe deposit vaults, often in private houses, in barns, in caves; on one occasion in a hen house, and on two occasions in coffins. Targets in Russian held territory were not available to the Survey.

Some two hundred detailed reports were made. During the course of its work, the Survey rendered interim reports and submitted studies and also suggestions in connections with the air operations against Japan. While the European War was going on, it was necessary in many cases to follow closely behind the front; otherwise vital records might have been irretrievably lost. Survey personnel suffered several casualties, including four killed.

Thus, a number of interesting reports were written by US technical experts who visited Germany to investigate every phase of the German petroleum and synthetic fuel plant and research activities. Indeed, it was reported that by May 1945 there were more than 17 reporting teams, many of them tracking over the same area. Some of the teams were combined teams with British investigators.

The teams had begun to assemble around October 1944, however the German offensive termed by the Allies as the ‘Battle of the Bulge’ in late 1944, set back the investigation until February 1945 as the Allied armies were crossing the Rhine. In early March, 1945 a Fischer-Tropsch plant at Moers, and a hydrogenation plant at Wesseling were captured and opened for investigation. An I.G.Fabenindustrie plant at Ludwigshafen was captured on March 23, and the following day 65 technical experts – the largest team assembled for a single target, and including experts interested in many items other than oil – swarmed over the huge chemical plant which covered an area of many square kilometres. Seizure of these plants was followed rapidly by the opening of the Ruhr and, finally the withdrawal of the German armies from Leipzig, Leuna, Zeitz, Böhlen and Luetzkendorf. Many plants in the Hamburg area particularly crude oil refineries were also examined. Leuna was the most important oil target in Germany.

The general pattern of investigation rapidly became established. Examination of the badly damaged plant areas was generally found to have minor importance, except where particular items of equipment need to be measured and carefully studied. The major sources of information were the German engineers and scientists, and the plant or research records. Frequently members of the German technical staff were found in the plant area when the investigators arrived. Others could usually be located at their homes if these were in the vicinity of the plant. A few of the top-flight scientists and engineers withdrew with the German army, but as more and more of the country was occupied, these individuals could go no farther and were soon available for questioning.

CIOS/BIOS

The Allies had a number of organisations gathering information about their enemy - BIOS (British Intelligence Objectives Subcommittee) and CIOS (Combined Intelligence Objectives Subcommittee). There was also FIAT (Field Information Agency Technical) and JIOA (Joint Intelligence Objectives Agency). Their role was Allied investigations of a wide variety of military and civil targets (industries, research institutes, military installations, scientific and technical personnel, etc.) in Germany and to a lesser extent Japan with reference to the Second World War, covering amongst other subjects agriculture and food production/processing, armaments and munitions, aircraft development, civil engineering, the chemical industry, domestic appliances, electrical and mechanical engineering, consumer goods, metallurgy, mining, the optical industry, medical research and pharmaceuticals, textiles, and transport.

As part of CIOS there were also technical teams attached to the British Army in particular the 21st Army Group under the command of Field Marshall Montgomery (‘Monty’s Army’), as they headed towards Germany. These were known as CAFT or Consolidated Advanced Field Team whose task it was to assess captured oil and chemical installations, possibly with the view to reconstruction and utilization for the Allied war effort.

At the end of the war BIOS produced a number of reports:

BIOS Overall report No. 1 - Report on the Petroleum and Synthetic Oil Industry of Germany

BIOS Misc. Report No. 72 Manufacture of Aviation Gasoline in Germany prepared by M.E. Spaght – Technician. [Dr. Monroe Edward Spaght was no ordinary ‘technician’. He was born in 1909 in Eureka, California, USA. He received his PhD from Stanford University in 1933 and commenced as a research chemist with Shell Oil Co., California. From 1940 to 1945 he was the Development Manager, Shell Oil Co., California, and in 1945 joined the US Navy Mission to England as a ‘Technician’. After that job he was the Director of the Strategic Bombing Survey, Japan. In 1946 he returned to Shell as Vice-President, Shell Development Co, USA (New York), by 1949 he was President, Shell Development Co., USA, by 1953 a Director of Shell Oil Co. USA, and by 1960 President, Shell Oil Co, USA. In 1965 he became Managing Director and Chairman, Royal Dutch Shell Petroleum Co., and on retirement in 1970 remained a Director of Royal Dutch Shell Petroleum Co. He died in 1993 after a distinguished career and service in the oil industry.]

BIOS Final Report No. 967 Deutsche Vacuum Oel AG – Fuels and Lubricants

BIOS Final Report No. 122 Interrogation of Herr Helmut Plote of the Construction Department - Addendum. July 8th - 11th, 1945. Reported by: - Mr. H.L. West, British Ministry of Fuel & Power.

Organisation of WIFO

Final Report No. 122 Interrogation of Herr Helmut Plote of the Construction Department - Addendum. July 8th - 11th, 1945. Herr Helmut Plote appeared an intelligent man and appeared to have a good, if somewhat limited, knowledge of the (WIFO) organisation and supplied quite a wealth of information on the whole concern. He is aged 37 and trained as a mechanical engineer and worked in the Construction Department. He joined the Nazi Party in 1933 and was a member of the S.S.

WIFO = Wirtschaftliche Forschungsgesellschaft m.b.H. was founded in 1934 with capital of  100,000 (Reich Marks), the capital being jointly subscribed by the I.G.Farben one part and the Government four parts. The original object was to store petroleum products and to manufacture nitric acid.

Later in 1934 the capital was increased to 100,000,000 all held by the German (Nazi) Government, the object then being to store all military and economic reserves, particularly petroleum products, both for the Wehrmacht (German Army) and industry, and to manufacture certain chemicals (apparently these were the main chemicals required for explosives);

The management of the concern was directed by a board of directors and five main departments. The General Director was Franz Wehling, deputy General Director Friedrich Frey with Hans Attig, Albert Henne and Prankel to complete the board. The departments were Administration, Mineral Oil, Transport, Construction, and Factories.

The Mineral Oil Department operated largely as a commercial undertaking, buying raw materials and products from refineries and synthetic manufacturers, and also from foreign countries, reselling finished, blended products to the Air Force, Army, Navy, and to industry.

The sub-divisions of the departments for the Air Force Fuels and Lubricants were:

Quantities and disposition

Frontline supply and recalls

Quality Control

New Construction and Extensions of Plants

The same sub-divisions were established for the Army.

Before the war WIFO was concerned mainly with buying aviation, army and industrial products for reserve storage. For example, in 1935 to 1936 WIFO bought aviation products direct from U.S.A. (from Standard Oil Co. of New Jersey, in which transactions the Deutsch-Amerikanische Petroleum Gesellschaft were intimately connected), and from Mexico. These products were transferred from tankers to barges on the Weser and Elbe rivers and shipped to WIFO depots. Later they bought direct from Roumania via Astra, Romano Americana and Creditul Minier. Subsequently, as other countries were over-run WIFO (in Paris 1940) purchased stocks and products from French Companies, undertook construction repairs to refineries and arranged manufacture: the French WIFO organisation was practically independent of the WIFO Headquarters in Berlin. Much the same type of organisation existed in other occupied countries. Also in the Eastern European countries trusts were formed to purchase Roumanian and Russian products.

Inside Germany WIFO purchased products from Z.B. (Zentral Büro für Mineralöl) comprising all German oil firms (Shell Standard, Olex, etc.) which handled all motor fuels and lubricants for German industry. As the war progressed WIFO tended to by-pass the Z.B. and purchased direct from refineries, producers of synthetic products and coal tar manufacturers.

Plote alleged that all Ethyl Fluid plants were constructed and owned by WIFO.

WIFO Installations – German distribution System

The oil storage and blending installations were divided into four classes, namely Grosslager or Hauptlager - large installations dealing mainly with products for the Luftwaffe and to a smaller extent with Army supplies. Heereslager or Heerestanklager - being as the name suggest depots concerned solely with Army products. Lufttanklager – depots only concerned with air force products (including facilities for T.E.L. blending). Marinelager – depots only concerned with stocks for the German Navy.

Below is a list of WIFO oil storage and blending depots. Before the war most of these depots had camouflage names which were used in an endeavour to retain some secrecy as to their exact location. During the war the use of these names was almost discontinued.

Table 6. Grosslager (large installations)

| Depot | Camouflage name | Total Fuel Capacity in 1,000 cu. metres | Date of Construction |
| --- | --- | --- | --- |
| Derben | Löwenberg | 160 | 1934 |
| Drugehnen | Hegeberg | ? | End of 1934 |
| Farge | Wasserberg | 220 | 1936 |
| Hitzacker | Hellberg | 100 | End of 1935 |
| München | Münchberg | 100 | 1934—1935 |
| Neuberg | Buchenberg | 100 | 1935-1936 |
| Nienberg | Kuhberg | 100 | End of 1934 |
| Raudnitz | - | 60 | 1940 |
| Stassfurt | Bela | 220 | 1934 |
| Vienna | - | 140 | 1940 |

Table 7. Lufttanklager (large installations) (No capacity data was listed)

| Depot | Camouflage name |
| --- | --- |
| Annaburg | Elbtal |
| Bad Berka | Erftal |
| Dülmen | Westtal |
| Ebenhausen | Kirchtal |
| Ehmen | Elendstal |
| Langenselbold | Burgtal |
| Löwenhagen | Osttal |
| Niederullersdorf | Ullertal |
| Weissenhorn | Horntal |

Later the Luftwaffe built Kloster Loccum, Oldendorf and Buchholz

Distribution

WIFO installations had to inform Berlin of their stock position daily and it can be assumed that stocks were kept at a reasonable level. Order would be sent to Headquarters in Berlin for supplies for the Luftwaffe or the army and the depot would prepare the necessary quantity, notifying Berlin when the supplies were ready for dispatch. According to the district in which the depot was situated the central distribution office (Hamburg) – which was usually associated with the Reichsbahn – would call forward a train and/or other transport which would be loaded and dispatched. The only indication of the destination for the product would be the train reference number; the personnel at the installation never knew where the product was going.

Although the Luftwaffe drew the bulk of their supplies of fuels and lubricants through WIFO, the army only obtained about 30 to 40% from this source, the remainder being obtained directly from the distributing companies, e.g. Rhenania, Ossag, Vacuum, D.A.P.G., Olex, etc. As transport difficulties increased both the Luftwaffe and the army tended to draw stocks from the most convenient source, a practice which resulted in an increasing use of the distributing companies.

During the war the Luftwaffe took over the Luftanklager and operated them independently of WIFO and even constructed some additional installations. As the war progressed and other countries were over-run WIFO set up installations to serve the advancing armies, making use of existing storage installations in the occupied countries and where necessary, even building new capacity.

The following lists some of the installations in occupied France These were all called Heereslager and were operated by the French under German supervision.

WIFO in France

Places in which in 1940 Heereslager were set up in France

Paris

Paris – Genevilliers

Paris – St. Ouen

Honfleur

Rouen

Cherbourg

Rennes

Nantes-Donges

La Rochelle

Raum Bordeaux

Sens

St. Florentin

In the beginning of 1944 new erections were set in the following places for safety from allied bomb attacks

Sens

Bazacourt bie Reims

Lyon

Le Mans

Also, in Oslo Norway.

Pipe Lines

It was known that pipes lines were used by the Germans for transportation of their petroleum supplies. The pipelines constructed did not necessarily relate entirely to petroleum as some were laid to feed chemical works with natural gas for such purposes as synthetic rubber manufacture. The following petroleum pipelines were known to have been laid or to have been under construction or consideration.

Oil Pipelines

Ploesti to Guirgui

Raudnitz (Roudnici) to Vienna (300 mm diam. 1940)

Neusiedl to Vienna

Stassfurt to Güssen

Farge to Osterholz

Five pipe lines across the Rhine. (It is known that three of these pipelines across the Rhine were completed).

Bomb damage to Oil Installations

From documentary evidence it was known that several of the Grosslager and Heereslager were damaged by allied bombing. For example, the depots at Drugehnen and Vordeheide were out of use on the 1st February 1945. Similarly, it is known that on the 18th December 1944, repair work was in hand at the Vienna and Nienburg depots, to make good the damage done due to air attack. Heiligenstadt depot had four tanks damaged on Food Friday 1945 whilst at Hitzacker the extension to the site was damaged, one set of five tanks receiving a direct hit and the railway siding damaged. Most of the bombs however fell well outside the target area.

WIFO also had technical papers relating to the USSR Petroleum Industry; it would appear that this subject was being carefully studied so as to make the best use of the facilities when, and I, they should be available.

The WIFO organisation appeared to have been one of the most important of the State organisations connected with the prosecution of the war. Not only were its connections with the petroleum producing companies and distribution concerns of importance, but its obvious connections with the chemical industry were of interest to the Allied interrogators. The fact that it would appear that only trusted Nazi party members were allowed to hold executive office indicates that its activities may not be above suspicion

These reports can be located at the Imperial War Museum, and excerpts are included in this chapter.

As the Allied armies crossed Europe these technical teams were close to the frontline and at work

CIOS/CAFT Reports 1945

The following are examples of these reports of captured German oil installations.  UK PRO Ref: FO 1039/556 Reports and Information - Refineries

They are CIOS/CAFT reports by Allied scientific personnel on the status of German oil facilities - Interrogations and assessments in 1945 for the 21st Army Group under the command of Field Marshall Montgomery.

CIOS CAFT Assessment Report Oil Target Hanover

Major V Ignatieff 	CAFT Leader, Group III

G(T) and CW Branch 	21 Army Gr Main HQ

1.	Target No.	GSI 87

2.	Title		Deutsche Erdöl AG – Wilhelmsburg, Hamburg

3.	Location		Wilhelmsburg, Hamburg

4.	Condition	Practically all destroyed, Could not operate in any way in less than 6 months work.

5.	Description	This refinery was equipped to distill 125,000 tons crude/year [approx. 2,500 Bbls/day], equipped with both atmospheric and vacuum stills, dewaxing and refining of lub(e) oils. The only solvent extraction was a small 3 ton/day. Phenol extraction unit, on which pilot work was done, but permission to build a larger unit was never obtained. Laboratory was routine and all experimental and research work was done in Mariendorf Berlin, where the work was associated with Edeleanu Labs. About 40% of crude was converted to lub(e) oils

Products

Benzine (assume gasoline) 		Rhenania

Kerosine				Sold in pool

Gas Oil				ditto

Lub Oils		chiefly to A.S.V. (Govt.) some for own use.

Asphalt		Sold in drums and tank cars.

(Deutsche Mineralöl Verkauftsverein)

Employees	500-600 Much plant equipment sent to Wietze

Other factories of this concern in

Nova – Vienna	Oilfield Neusiedol

Heide – Holstein

Bohren Betrieb – Wietze

Pechebrome Refinery and Vinegar Works

Crude oil for this works was from Wietze, Heide, Reitbrook, Alten gamma Austria

Work was destroyed Jun, Aug, Oct, Dec 1944, March 1945.

No steam or power plant now workable, lab entirely destroyed.

Date of Assessment: 6 May 1945, Assessor’s Name: Lt. Col. C.R. Middleton, 21 Army Gr 12 May 1945(32065/7)

To CAFT Group I

21 Army Gr Main HQ

1.	Target	None (Opportunity Target)

2.	Title		Deutsche Gasoline AG – Fabrik Betrieb Dollbergen

3.	Location	Dollbergen, 20 km east of Hanover on R.R. to Berlin

4.	Condition	Considerable bomb damage. Complete destruction of dewaxing plant, acid recovery plant and 85% of tankage. Many vital sections of refining still intact, including small works lab.

5.	Description	This is a fairly complete but not up to date refinery. Owned jointly by Shell, Standard and I.G.Farben, the latter controlling 51% interest. Refinery Superintendent is Otto Kurz who still lives in Dollbergen. Equipment still existing is as follows:

1)	Crude Distillation Unit, 2-stage atm/vac type. Has run 150 tons/day of Nienhagen or Wietze crudes, cutting gasoline, diesel, light and heavy lube distillates and 35% asphalt. Plant undamaged

2)	Asphalt Oxidation Plant. Produced up to 400 T/month of blown asphalt (balance of asphalt burned or sold for fluxing, etc.) Plant undamaged.

3)	Acid Treating Plant and Contact Plant. Can treat and contact neutralize up to 40 tons lube stock per day. Use one bergedoff centrifuge for treating. Entire plant undamaged. Acid sludge burned in rotary burner, which is intact.

4)	Powerhouses. Three all in good condition

5)	Stocks on hand. About 1,000 tons of crude oil, 300 tons of various lube stocks and a large stock of brown asphalt in drums.

6)	Items guarded – None

7)	Priority Assessment. Target is of no intelligence interest, Its only immediate value is in ability to produce asphalt for military. (presumably for reconstruction of war damaged roads).

8)	Other Remarks: Otto Kurz stated that the dewaxing plant was designed on the basis of some development which he and Dr. Pfaff (now in Pirna) had conducted around 1935-36, and it is the only one of its kind ever built. It was actually a combined acid treating dewaxing process which involved diluting the raw waxy oil with equal volumes of naphtha, chilling and filtering at 0 deg C, chilling filtrate to -25 deg C, mixing with 6-10% Sulfuric acid, based on oil, and co-settling of sludge and wax. The clear solution was caustic washed, distilled and the oil clay contacted. Final pour point -10 deg C. Interesting but not competitive with modern solvent refining methods.

9)	Date of Assessment	17 April 1945

10)	Assessor’s Name: Louis P. Evans, Civ (CAFT Group 3), Army Gp 21

Item Gp 30 (Target of Opportunity)

This plant is a duplicate of the plant Deutsche Gasoline AG at Emmerisch 985604

CIOS CAFT Assessment Report

1.	Target No.	3.10 and 3.11 (no CIOS priority)

2.	Title Deurag/Nerag (Joint ownership Gewerkschaft, Deutsche Erdöl Raffinerie and Neue Erdöl Raffinerie.

3.	Location	Main factory located at Misburg, 5km east of Hanover. Auxiliary small plants dispersed in vicinity of Hanover as described under remarks.

4.	Condition of Target: Complete destruction of crude oil tank farm, main office building, original laboratory, original dewaxing plant, doctor-treating plant, and one power house installation. Extensive damage to buildings in general, but relatively slight damage to vital refinery equipment in these buildings. Severest air raid was on June 20, 1944 when over 50% of lube oil capacity permanently crippled. Partial operation until 8 April 1945. Plant capable of being restored to substantial production.

5.	Description: This is claimed to be the largest oil refinery in Germany with a crude oil charging capacity of 1,300 tons/daily. [10,000 Bbls/day]  Normally employs 1,200 people. Has operated chiefly in Nienhagen for lube oils and motor gasoline, made no aviation gasoline. Also processed topped crude received from auxiliary topping plants described below. The Deurag and Nerag section of the refinery comprise the following facilities:

1)	Atmospheric Tube Still. Crude topping capacity 700 t/day taking 405 OH {overheads}. Slightly damaged.

2)	Dubbs Coking Unit. Cap. 33 t/d charging mixture of excess reduced crude, furfural extract, wax and asphalt. Slightly damaged.

3)	Pressure Distillate Treating Unit. Cap. 180 t/d includes caustic washing of Dubbs P.D. continuous sulphuric acid treating, caustic neutralizing and water washing. Slight damage.

4)	Distillate Rerun Unit. Cap. 180 t/d. Slight damage.

5)	U.O.P. Poly Plant. Designed to process 40 t/day of unsat. Refinery C3 and C4 cut. Includes absorption plant for preparing the charge. Plant was installed in 1939 but never operated because unable to get UOP catalyst in time. Efforts to develop substitute catalyst through cooperative work with Ruhr Chemie proved unsuccessful, as described under remarks. Catalytic Unit slightly damaged. Stab OK [Stabiliser OK]

Nerag

1)	Comb, Atm/vac Distillation Unit. Cap. 600 t/day. Crude Atm. stage 15% gasoline and 25% Diesel Oil, vacuum stage gas oil, spindle, neutral, light cylinder distillates, and 30% bottoms on crude. Unit slightly damaged

2)	Propane Deasphalting Unit. Kellogg 2-stage type i.e. resin and asphalt, although one stage taking wide cut is normally used with equal success. Have processed 180 t/d of 30% residuum, rejecting 55% of 50-60 deg C m.p. (melting point) Asphalt. Slight damage.

3)	Furfural Unit, Twin unit, conventional design, one side for treating deasphalted cyl. stock and smaller side for treating neutral. Both stocks treated to 74 VI and ultimately blended for motor oils of various grades/ Also made 95 VI aero oil until June 44, but discontinued due to transfer of Coubro Unit, which prepared the lube stock, to underground plant at Porta. Furfural plant has not operated since June 44 due to bomb damage, and shortage of Furfural. Extensive repair required.

4)	Acid Treating Plant. Installed early in 1945 for lube oil treating after shutdown of Furfural unit

5)	Contact Plant. For contact treatment of furfural refined cylinder and neutral oils. Continuous pipe still with Sweetland Process.

6)	B/A Dewaxing plant. Rebuilt in 1941 after destruction of original plant, designed by TEXACO. Present plant similar to first.

Products

Straight Run Gasoline 150 deg C EP (end point)		3,000 T/month

Cracked Gasoline 210 deg EP 68 ON			4,200 T/month

Kerosine	or total as Diesel fuel				7,000 T/month

Gas Oil							3,000 T/month

Coke	Sold to Siemens or IG Farben for electrode manufacture	2,800 T/month

R.R. Axle Grease- made by mixing equal parts dewaxed spindle oil and furfural extract from cylinder stock 1,500T/month

Waxes

[The report then continues for 4 more pages with description of comments from Refinery Superintendents and Lab Chemists on catalyst]

To 	Major V Ignatieff	CAFT Group 3

CIOS CAFT Assessment Report [CAFT = Consolidated Advanced Field Team]

1.	Target No.	3.08

2.	Title		Deutsche Vacuum Oel AG Refinery

3.	Location		Oslebshausen, 2km north of Bremen

4.	Description	This refinery is a modern lube refinery built just prior to the war. Had crude capacity 400 t/day or 10,000 t/month. Ran mixture of 35% Reitbrooh crude and 65% Nienhagen crude. Chief product during the war was 95 VI Red Band (Rotring) Aero Oil of which 900 tons/month were supplied to Luftwaffe through INTAVA. Employed 370 people very little slave labour. Refinery operated up to March 21, 1945 when heavy air raid forced shutdown.

INTAVA may have done test work on aero fuels and lubes in labs originally located in Schulan and recently transferred nearby.

Assessor Louis P Evans, Date 29 April 1945

[Comment: It is interesting to note that even in war, the Standard Oil companies aviation marketing activities were still apparently a going concern.]

1.	Target No.	GSI 83

2.	Title		Ebano Asphaltewerke AG

3.	Location		Hamburg

4.	Condition	Almost completely destroyed

5.	Manager 	Joseph Schmidt

May 5, 1945

[Comment: Joseph Schmidt later worked for Emoleum in Australia in the 1970-80’s as their Bitumen Technical Manager. Emoleum was a subsidiary of Mobil Oil Australia.]

To 	Major V Ignatieff	CAFT Leader Group 3

CIOS CAFT Assessment Report No. 126

1.	Target No.	Opportunity target

2.	Title		Pernis Refinery Bataafsche Petroleum Maatschappy

3.	Location		On South bank of Nieme Mass River directly opposite Vlaardingen (near Rotterdam). Map Ref. 660705 on sheet 4 (Rotterdam….)

5.	Description	This refinery which was in operation before the war was equipped to carry out the following work.

(i)	Topping of asphaltic crude oil

(ii)	Distillation of asphaltic residue, under vacuum to asphalt

(iii)	Cracking of asphaltic residue to gasoline, coke and gases

(iv)	Separating cracked gases into various fractions, some of which were polymerized

(v)	Hydrogenation of Octylenes

(vi)	Rerunning of gasolines

(vii)	Treating of gasolines

Apart from the ‘rerunning’ of a small quantity of oil during the first few months of the German occupation, the refinery has not operated since May 1940

[Comment: It is ironic that the high-octane aviation fuels produced by hydrogenation of octylenes used by the Allies, was in the control of the German occupation in 1940 and not utilized. This was the Shell Pernis Refinery in the Netherlands]

# BIOS Overall report No. 1 - Report on the Petroleum and Synthetic Oil Industry of Germany

This report was published as a book and some relevant excerpts (Section H) are listed here:

Testing and Evaluation of Products

1. INTRODUCTION

The appraisal of the work done in the development of new fuels and lubricants was made the subject of a separate study, but since a valid appraisal is impossible until all the available documents have been examined, this report must be confined to a record of the major problems studied by the Germans which have so far come to light, without passing judgement on the work done.

2. FUELS

(i) AVIATION PETROL

The German supply of aviation fuels was based on the hydrogenations of coal, and in fact caused them to follow a different path from that of the Allies to high engine performance. At the start of the war they had standardised on an 87 ON grade (known as B4) of leaded hydropetrol from brown coal, and which contained 5-15% aromatics. In 1940 an improved fuel (C2) of 95 ON (CFR Motor Method) was introduced having a higher aromatic content of 35-38%, and giving a performance equivalent to the Allied 100 ON grade of that time. At the end of 1942 a different kind of (C3) began to come into service with a still higher performance, and was equivalent to the Allied 150 grade. This fuel contained some 5% more iso-octane than the previous version, but the aromatic content was maintained at the previous level by dehydrogenation of the higher boiling point naphthenes on the fuel. The policy background to these changes in fuel type is not yet fully known, but the new fuel was an outstanding development and was a result of the introduction of the DHD process which is dealt with under hydrogenation. No conclusive evidence was obtained to show that piston-ring sticking was increased as a result of the use of these aromatic fuels.

The standard test for knock-rating of aviation fuels was the BMW 132 supercharged method, in which a full mixture–response curve of the test was compared with the appropriate reference curve, the test curve being required to be above the reference curve over the significant mixture range. This requirement was slightly relaxed at the end of the war owing to supply difficulties.  Extensive correlation test programmes were undertaken between the various test stations. Several small-scale test units were also developed, as exemplified by the supercharged IG engine, the IG ‘K’ unit and the supercharged NSU (motorcycle) engine, these being devised with the object of reproducing the BMW 132 ratings and achieving adequate correlation for the purpose of investigating new fuel components. The CFR and its counterpart the I.G. Prüfmotor, using both the Motor and Research Methods, were also used for control purposes.

No improved fuel was actually in sight at the end of the war, and although the permissible TEL concentration was raised from 0.12% to 0.18%, this was to assist in the maintenance of supplies; the extent to which advantage was taken of this relaxation is not yet known.

The use of 2% aniline was also proposed as a blending agent and a C3 type was produced from B4 (the 87 ON aviation fuel) by blending with aviation benzole and aniline, and adding TEL to 0.16%.

The use of GM-1 (Nitrous oxide) for emergency power boosting at altitude was developed for operational use in 1943. Approximately 3 hp per gm/sec GM-1 was obtained, irrespective of altitude, and rates of flow up to 120 gm/sec were employed. The material was carried in Dewar type containers at -80 deg C and was metered into the engine before the blowers. Difficulties were experienced with explosions occurring in the induction system, and engine temperatures were generally higher when using GM-1.

Little information has yet been discovered on the use of internal coolants but reference is made to the use of methanol for this purpose, in conjunction with GM-1 operation. Pre-ignition was experienced in this connection, although normally it was not a service problem probably on account of the use of direct fuel injection. The tendency of methanol to pre-ignite was the subject of a good deal of research and various additives were tried experimentally to reduce pre-ignition in this fuel, but no success was achieved.

Large numbers of pure aromatics and other components were tested for anti-knock quality. Of a wide range of alkyl benzenes, di-ethyl benzene was particularly satisfactory and was used extensively as a blending component under the name of ‘Kybol’. Pinacoline – tertiary butyl methyl ketone – was found to be exceptionally good experimentally, being about equal to triptane, illustrating the use of the tertiary carbon atom structure to obtain high anti-knock effects; it was not, however, put into operational use, although production could have been arranged.

A good deal of fundamental research on combustion problems was in progress. This included laboratory tests concerned with flame propagation in relation to detonation and also in connection with gas turbine development. Detonation was also studied from the points of view of adiabatic self-ignition, and slow oxidation of hydrocarbons. Engine work in the same sphere involved the use of ionized gap and photo-cell methods for studying combustion.

The effects of engine design, operating temperature and chemical composition on the knock-rating of a fuel were studied extensively in an attempt to formulate a theory of knock mechanism. The effect of large valve overlap in fuel injection engines, which results in the highest knock-free BMEP point with paraffinic fuels occurring at weak mixtures, and which was observed on trials of captured German engines in this country some time ago, was one of the principal starting points for the experimental investigation, and considerable progress had been made.

Both Daimler Benz and BMW were developing engines with variable valve timing for automatic reduction of valve overlap with increase in altitude. This development work was abruptly stopped when the German Air Ministry rules that everything else must be dropped in favour of jet turbines.

The problem of lead attack had been largely solved by stelliting and chromium plating exhaust valves and, in addition, an unleaded running-out fuel was introduced on which the engine was shut down, to avoid cold corrosion. To what extent this fuel was used in service is not yet known.

The only inhibitor found in service was known as ZV1 and consisted of 0.01% mixed cresols. Cadmium-plated filters had to be adopted to avoid plugging which arose from a reaction between the ethylene bromide in the TEL and the zinc in the galvanized filter that was previously in use.

The Deustche Versuchsanstalt für Luftfahrt (DVL) carried out some studies on lead deposition in storage which became serious with the introduction of high performance fuels of relatively high aromatic content and , as a result, an autocatalytic mechanism has been suggested as the explanation. According to this theory, initial oxidation of the hydrocarbons causes the decomposition of the TEL, and the decomposition products then accelerate the further oxidation of the hydrocarbons, resulting in the deposition of lead sludge. By the use of substances which make innocuous the primary decomposition products of TEL (fuel soluble tri-and di-ethyl lead compounds with one or two free valencies have been identified), or which suitably combine with the primary products of the hydrocarbon oxidation, the stability of the leaded aromatic fuels could be appreciably increased. In practice, the use of small quantities (about 0.01%) of phenolic extract from the products of bituminous coal hydrogenation was specified.

In connection with this work, a method was devised for estimating the bromine from the ethylene bromide in the fuel.

Several test methods were developed for the determination of TEL in petrols, including one which is reported to employ X-rays, but, in general, the methods at present known do not seem to be of general application and are suitable only when the source and composition of the aviation petrol are known

Operations on the Eastern Front led to cold starting difficulties, but as far as can be seen these were less severe with aircraft than with vehicles. Foe aero-engines induction priming with a light petrol of 30-100 deg C boiling range, containing 4% lubricating oil, was satisfactory down to -30 deg C, below which acetylene was used on certain engine types. Various ethers and pure hydrocarbons were investigated with a view to developing a liquid fuel for -60 deg C, and the behaviour of current aviation fuels with respect to viscosity and water separation was studied down to -70 deg C., in view of possible fuel distribution difficulties in low temperature.

On account of the shortage of aviation fuel components, the A3 grade used for training aircraft was later blended with 30% ethanol and troubles were consequently experienced with separation, corrosion, etc., as well as with cold starting.

Work was begun some years before the war on the development of a true safety fuel, which had not only a higher flash pointy than conventional fuel, but which, unlike the usual safety fuels, also had a low velocity of flame spreading over  the liquid surface. An isobutylene polymer of approximately 3,000 molecular weight was applied for this purpose in engines operated by the “Ring process”, which employed a system of compression ignition. This involved the separate injection of the di-ethyl-ether of di-ethylene-glycol (listed as R-300) into the fuel charge towards the end of the compression stroke. This ignition fuel could be used with diesel fuel. Flight testing was carried out, but the system had not been fully proved and difficulties were expected at high altitudes with low temperatures. The system was claimed to give lower cruising consumption.

(ii) AVIATION DIESEL FUEL

Only a very small amount of work on aviation diesel fuel has so far come to light, but this is in accordance with the limited use if diesel powered aircraft in the German Air Force. The Junkers engine was the only diesel aero engine in service, and for it a minimum cetane number of 50 was specified, and a pour point of -45 deg C. The fuel contained no ignition accelerator additive, but a special starting fuel was supplied containing 25% ether.

The development of electrostatic charges in the fuel tanks with these engines caused trouble when ether was present, however this was overcome experimentally by adding halogen or sulphur compounds to the fuel.

One aircraft that was equipped with Junkers diesel engines was the Ju 86. Diesel engines operated well under constant power loads, however they proved unreliable when tasked with the challenge of frequent changes in power demand required by combat aircraft.

Photo 15. Junkers Ju-86 (Swedish markings). It is on permanent static display at the Flygvapenmuseum in Sweden

Photo 16. German Junkers Ju-86

(iii) GAS TURBINE FUEL

No fundamental work on the constituents of gas turbine fuels has been discovered, although studies were made of the effect of different burners on the flames obtained with different fuels. It is reported that the initial specification requirements were quite strict owing to early difficulties with corrosion, coking of burner jets, and the effect of phenols and aromatics on packing materials and liners, but the latest gas turbine fuel specification provided for a maximum of 1 ½% sulphur, 1% phenols and 1 ½% Conrad son carbon, which may be considered quite lenient.

The standard fuel for jet engines was J2, a carefully refined distillate fuel, of which important properties were a setting point below -25 deg C, and adequate filterability at the same temperature. A running-in fuel, J2-einlauf, for which satisfactory low temperature behaviour only at 0 deg C was specified, was also supplied, presumably as a fuel economy measure.

Towards the end of the war, an unrefined distillate of higher boiling range was used in jet planes, the oil being heated before filling the tanks. No trouble was experienced as a result of this expedient, beyond the sooting up of ignition plugs.

As special fuels for restarting at altitude following flame extinction, the nitro-alkyls were found to be satisfactory and a starting device with a glow plug was developed for starting these fuels.

Photo 17. German Messerschmidt Me-262 – the next generation of aircraft with gas turbine engines

# Avgas Composition, Specifications & Test Methods 1941

German Aviation Gasoline Specification

German military aviation had three grades of aviation gasoline: “A-3” for training aircraft and for operations “B-4” Blue and “C-3” Green. Both operational grades were leaded to 4.35 cc TEL/IG. The source of these grades was by various coal to gasoline conversion processes, supplemented by synthetic chemical processes.

“A-3” contained about 30% ethanol, aromatics 35% and TEL content from 0.045% up to 0.12% vol.

“B-4” was simply a fraction of the gasoline product from coal and coal tar hydrogenation. It contained 10-15% aromatics, 45% naphthenes, and the remainder paraffins.

“C-3” was a mixture of 10-15% synthetic isoparaffins (alkylate and iso-octanes) and 85% aromatized base stocks produced by hydroforming types of operation on coal and coal tar hydrogenation gasolines. The C3 grade was permitted to contain not more than 45% aromatics. (This was due to the absence of inhibitors and concerns about lead instability with high aromatic content). This aromatic limitation sometimes required that the base stock component include some diluent other than the aromatic fraction, which could be then balanced if necessary by the inclusion of slightly more iso-paraffin.

Table 8. Reich’s Luftfahrtminsiterium (RLM) Aviation Gasoline Specification circa 1945

| Grade | B4 | C-3 |
| --- | --- | --- |
| Colour | Blue | Green |
| Density @ 15 deg C | 0.71-0.760 | 0.76-0.795 |
| Distillation Initial Boiling Point (min deg C) | 40 | 40 |
| 10% max deg C | 70 | 80 |
| 50% max deg C | 105 | 110 |
| 90% max deg C | 160 | 160 |
| End Point (max deg C) | 170 | 180 |
| Recovery Vol. min | 98% | 98% |
| Reid Vapour Pressure max. psi | 7.0 | 6.3 |
| Aromatics | 25% max | 45% max |
| Tetra Ethyl Lead Content % Vol. | 0.115-0.120 | 0.115-0.120 |
| Ethylene Dibromide Content % Vol. | 0.05-0.053 | 0.050-0.053 |
| Melting Point deg. C | -60 | -60 |
| CFR Motor Method Octane (leaded) Min. | 89 | 95 |

The Tetraethyl Lead limit of 4.35 cc/USG is equivalent to 5.22 cc TEL/Imp Gallon or 0.120%. (The Allied limit was 4 cc TEL/IG). The distillation data has been converted to centigrade. This distillation data was listed as maximum temperature for a specified volume, as used in Allied specifications.

Deutsche Vacuum Oel AG Specification

Table 9. Deutsche Vacuum Oel AG Gasoline Specification circa 1939-1945

The data was in metric and the distillation data was listed as minimum volume up to a specified maximum temperature, as used in Axis specifications.

| Grade | A3 | B4 until 1944 | B4 from Nov 1944 | C3 until 1944 | C3 from Nov 1944 |
| --- | --- | --- | --- | --- | --- |
| Colour | - | Blue | Blue | Green | Green |
| Appearance | - | Clear, free from dissolved water and acid. Shall contain no solid impurities. | Clear, free from dissolved water and acid. Shall contain no solid impurities. | Clear, free from dissolved water and acid. Shall contain no solid impurities. | Clear, free from dissolved water and acid. Shall contain no solid impurities |
| Density @ 15 deg C | - | 0.710-0.755 | 0.700-0.760 | 0.760-0.795 | Not stated |
| Initial Boiling Point | Not below 40C | - | Not below 40C | ca. 45C | Not below 40C |
| 10% vol. (min) | - | Up to 70C | Up to 75C |  | Up to 80C |
| 35% vol. (min) |  | - | Up to 110C | - | - |
| 40-50% by vol. |  | - | - | Up to 100C | - |
| 50% vol. (min) | Up to 100C | Up to 105C | - | - | Up to 120C |
| 90% vol. (min) | - | Up to 160C | - | - | Up to 180C |
| 95% vol. (min) | - | - | Up to 180C | - | - |
| Final Boiling Point | Below 190C | Below 170C | Below 190C | Below 180C | Below 190C |
| Distillation Loss Vol. | 2% max. | 2% max. | Not stated | 2% max. | Not stated |
| Reid Vapour Pressure max. kg/cm2 | 0.5 | 0.5 | 0.5 | 0.45 | 0.45 |
| Tetra Ethyl Lead Content % Vol. | 0.045-0.120 | 0.115-0.120 | 0.115-0.120 | (not stated) | 0.115-0.120 |
| Ethylene Dibromide Content % Vol. | - | - | 0.050-0.053 |  | 0.050-0.053 |
| CFR Motor Method Octane (leaded % vol.) | 80 | 89 min. with 0.115% TEL | 89 min. with 0.115% TEL | ca. 95 | About 95 with 0.115% TEL |
| Supercharging Performance | - | - | Mixture response curve in the range λ = 0.8 to 1.15 shall lie below that of the reference fuel |  | Mixture response curve in the range λ = 0.8 to 1.15 shall lie below that of the reference fuel |

| Aromatics %Vol. max | 35% |  | 35% |  | 45% |
| --- | --- | --- | --- | --- | --- |
| Ethanol content (+/- 3%) Vol. | 30% | - | - | - | - |
| Iodine value (gm/100gm) | - | 5 max. | - | 4 max. |  |
| Melting Point deg. C | Melting point of the fuel after cooling until crystallised shall not be above -50C | -60 Melting point of the fuel after cooling until crystallised shall not be above -60C | Not above -50C | Melting point of the fuel after cooling until crystallised shall not be above -60C | Not above -60C |
| Corrosion Test (BVM) | No grey or black spots or pitting in the copper strip test | No grey or black spots or pitting in the copper strip test | No grey or black spots or pitting in the copper strip test | No grey or black spots or pitting in the copper strip test | No grey or black spots or pitting in the copper strip test |
| Residue on evaporation (mg/100ml) | 10 max. | 8 max. | 10 max. | Not stated | 10 max. |
| Reaction on residue | Weakly acid | Neutral | - | Neutral | - |
| Inhibitor content | - |  |  |  | 0.01% by wt. |
| Storage Stability | - | After 6 months, test every 3 months | After 6 months, test every 3 months | Not stated | After 6 months, test every 3 months |

Note: there were some omissions and errors in the source data; the obvious errors have been corrected, and ‘not stated’ indicates where no data was listed.

The above Deutsche Vacuum Oel AG specifications for German aviation fuels revealed some interesting points. It must first be appreciated that the source of these fuels were different to the Allies– in that grades B4 and C3 were from coal conversion, therefore the quality requirements were different – for example to ensure that no coal particulates or other impurities were carried through the manufacturing process there was the requirement for ‘Appearance’., ‘Reaction on residue’ which were different to the Allied specifications for petroleum hydrocarbon based fuels. A3 was an ethanol mix for use in training aircraft and therefore the specifications are different again.

As noted previously specifications for aviation products are based on defining properties which can be translated into good engine performance. The German engines were different to the Allied engines. The German approach was different to the Allies particularly in the manner in which the distillation was expressed and the values, the supercharger method, the lower melting point (Allies freezing point).

Engine Testing:

The anti-knock performance of aircraft fuels was evaluated in two different manners: by octane number, using a test very similar to the CFR Motor Method, and by a mixture response curve. The Octane number was measured on the one cylinder “I.G. Prüfmotor” (technical details of motor are provided in reference source). The values obtained on the IG Motor closely agreed with those from a CFR Engine.  The mixture response curve was measured on a BMW single cylinder engine. (Supercharge engine).

Table 10. Performance of Aromatic Fuels due to supercharging

| Mixture of 50 vol.% benzene (gasoline) from hydrogenation ex hard coal ON 73 plus 0.12 vol. % TEL with | % performance at air excess. Index of |  | Octane Number |  |
| --- | --- | --- | --- | --- |
|  | λ=0.9 | λ =1.1 | MON | RON |
| Ethyl Propyl Benzene | 100 | 71 | 99 | 110 |
| Diethyl Benzene | 95 | 67 | 97 | 107 |
| Aviation fuel | 81 | 60 | 94 | 104 |
| Aromatics for dehydrogenation 150-175 deg. C | 80 | 59 | 93 | 103 |
| Aromatics for dehydrogenation 175-200 deg. C | 77 | 55 | 92 | 101 |
| Iso-octane | 60 | 52 | 100 | 101 |
| For comparison, known high performance fuels |  |  |  |  |
| Aviation benzine (gasoline) ex hard coal (0.09 vol.% TEL) | 44 | 33 | 88 | 91 |
| Motor fuel ON 100 containing about 50 vol.% isoparaffins (0.09 vol.% TEL) | 56 | 49 | 101 | 104 |
| Aromatized benzene (gasoline) ex hard coal (300 atmospheres pressure) (0.12 vol.% TEL) | 64 | 50 | 90 | 100 |
| Aromatized benzene (gasoline) ex pitch (700 atmospheres pressure) (0.12 vol.% TEL) | 80 | 59 | 93 | 103 |
| Pure iso-octane (0.09 vol.% TEL ) | 72 | 67 | 114 | 115 |

Graph 4. Supercharging Curves of High Performance Fuels

The CFR method was used in aviation fuel testing merely to establish the identity of a fuel. For evaluation as regard knock behaviour only the DVL supercharging test was regarded as valid. According to the Germans, the CFR method had shown itself inadequate for this purpose.

The Germans also found (as the Allies did) that supercharging performance was quite different when measured in air cooled radial engines (BMW 132N) and liquid cooled in-line engines (DB 601)

Reference Fuels

Iso-octane and normal heptane fuels were used for reference purposes in carrying out the CFR (ASTM) Motor Method test in CFR and IG engines. An auxiliary reference fuel, the IG Z benzene (gasoline) was employed. Values over 100 were not tested by the CFR Method. The reference fuels for the DVL supercharging test were “Eich B4” for examining B-4, and “Eich C3” for C-3. The “Eich B4” consisted of a Leuna hydrogenation benzene (gasoline) with 0.12% vol. TEL. “Eich C3” contained 80% DHD benzene (gasoline) from IG Ludwigshafen and 20% Iso-octane and had a lead content of 0.12% vol. TEL.

Photo 18. BMW 132N 9-cylinder supercharged fuel injected air-cooled radial engine (displacement 27.7 litres) as used in Ju-52.

Some other notable points are:

Aromatic content –The high aromatic content of C3 fuels was a consequence of the production planning. It had been decided to make aromatic rich fuels in the manufacture of C3 because of the nature of the raw materials available. The high performance of C3 in rich mixture range followed naturally from its composition. The mixture response curve of aromatic rich fuels is very steep, while that of paraffinic fuels is flatter.

C3 was used for fighter engines only, e.g. Daimler Benz DB6 603 and DB6 605 used in Messerschmidt Bf-109 (refer Photo 2), and BMW 801 used in the Focke-Wulf FW-190

Photo 19. BMW 801 14-cylinder supercharged two-row air-cooled radial engine (displacement 41.8 litres)

When an excess of C3 was available, then it was used in engines designed for B4 fuel. Excess C3 was also used as a blending component in making B-4.

Gum Stability – The tendency to gum formation was noted especially in the C3 fuel and 0.01% inhibitor was added. The inhibitor was made by I.G. Farben, its composition was unknown.

Photo 20. Focke Wulf FW-190A8 circa 1944 powered by BMW801 with a MW50 supercharger with methanol/water mix injection

Photo 21. Focke Wulf FW-200 ‘Condor’ refuelling circa 1943.

The Condors were a threat to Allied convoys; they also provided air reconnaissance for the German U-boats Wolf packs in their onslaught against Allied shipping in the Atlantic.

# BIOS Misc. Report No. 72 Manufacture of Aviation Gasoline in Germany

There was great interest by the Allies in the German oil industry and their processes for the manufacture of aviation gasoline. As detailed above there were many technical missions to Germany in 1945, immediately following the advancing Allied armies, with the express objective of technical investigation of captured facilities and interrogation of the German technical expertise. The following is the report of one such mission.

Summary of US Naval Technical Mission in Europe July 1945

This report BIOS Misc. Report No. 72 records information obtained by technical investigators on the quantity, quality, composition, and manufacture of German aviation gasoline during the past war years (WW II) (1939-1945). It was prepared by M.E. Spaght (Technician) – refer to earlier notes on Dr. Monroe E. Spaght.

Figures for the quantities of components and finished gasoline produced are presented and analysed. The qualities and compositions of the different grades are shown and discussed.

The methods and plants used in Germany for synthesizing isoparaffins, for manufacturing base stocks, and synthesizing aromatics are described. Process and operating data are given for these operations, particularly where the practice is new or different from that used in the United States.

The synthesis of nitration grade toluene was described in an appendix. (Toluene was required for the manufacture of Tri Nitro Toluene better known as ‘TNT’).

Production

Overall German supply of aviation gasoline was from processes producing isoparaffins, base stocks and aromatics.

The German aviation gasoline supply came largely from synthetic oil plants that hydrogenated coals and coal tars. A very small volume came from petroleum, while essentially none came from Fischer-Tropsch plants. Some components in small volume came from various chemical plants.

Parallel to the situation in the United States, great efforts were put forth continually in Germany to increase the supply of aviation gasoline. Much of the new construction was never completed due firstly to the Allied bombing and then to termination of the war.

Highest German production was in 1943 at 56,100 Barrels per day (8.9 M.Litres/day) (German Motor gasoline was 55,000 Barrels/day (8.8 M.Litres/day) at that time). Average Avgas for that month (1943) was 52, 200 Barrels/day (8.3 M.Litres/day).

[By contrast, in the United States in 1943 Avgas production was 269,000 Barrels per day (42.8 M.Litres/day), and by 1944 it was over 500,000 Bbls/day (>80 M.Litres/day) and rising.]

Table 8. gives a partial breakdown of the sources and volumes of German aviation gasolines and their components.

Table 11. Sources and Supply of German Aviation Gasoline (All figures are Barrels per day)

| Company & Location | Total Aviation Components | Base Stocks & Aromatics | Synthetic Isoparaffins |
| --- | --- | --- | --- |
| I.G. - Leuna | 6,900 | 5,500 | 1,400 |
| Brebag - Böhlen | 4,100 | 4,100 | - |
| Brabag - Magdeburg | 2,750 | 2,750 | - |
| Hibernia – Scholven | 5,800 | 4,400 | 1,400 |
| Gelsenberg - Gelsenkirchen | 8,000 | 8,000 | - |
| Pölitz A.G. - Pölitz | 13,900 | 12,400 | 1,500 |
| Rheinbraun - Wesseling | 2,750 | 2,750 | - |
| Ruhröl - Welheim | 1,100 | 1,100 | - |
| Sudetendeutsche - Brüx | 5,500 | 5,500 | - |
| I.G. – Oppau | 1,200 | 1,100 | 100 |
| I.G. – Heydebrek | 600 | 300 | 300 |
| I.G. – Moosbierbaum | 2,000 | 2,000 | - |
| I.G. – Hüls | 200 | 200 | - |
| I.G. – Schopau | 200 | 200 |  |
| Total | 55,000 | 50,300 | 4,700 |
| Aromatic Oils from Coal Tars | 1,100 | 1,100 | - |
| Grand Total | 56,100 | 51,400 | 4,700 |

# Gasoline from Coal

There were a number of processes for manufacture of aviation gasoline from coal and to some extent these were determined by the type and quality and quantity of coal. Unleaded Coal hydrogenation gasoline had an octane of MON 73 (Clear), however with TEL this could be increased significantly.

Table 12. Details of various Octanes of fuel blends

| Fuel (Note 1.) | Lead% vol. Pb | MON | RON |
| --- | --- | --- | --- |
| Aviation ‘benzine’ ex hard coal | 0.09 | 88 | 91 |
| Motor fuel ON 100 containing about 50% isoparaffins | 0.09 | 101 | 104 |
| Aromatized ‘benzine’ ex hard coal (300 atmos. pressure) | 0.12 | 90 | 100 |
| Aromatized ‘benzine’ ex pitch (700 atmos. pressure) | 0.12 | 93 | 103 |
| Iso-Octane (Pure) | 0.09 | 114 | 115 |

Note 1. ‘benzine’ in this case refers to gasoline.

The manufacturing process is illustrated in the following figure.

Figure 1. German Production of Aviation Gasoline by Hydrogenation of Coal

# German Chemical Processes

German chemist had always enjoyed a worldwide reputation for the development and operation of innovative chemical processes. Their technical expertise was directed to the Axis war effort. Many of the processes were similar to those in use in the United States and elsewhere (UK and Middle East). It should be recognised that the German oil industry before the war was already established and that trading with other American, British and Dutch oil companies was a fact of German pre-war economy. The US Naval Technical Mission in their investigation found that the German processes included the following:

# 1. Synthesis of Iso-paraffins

Isoparaffins were synthesized commercially in Germany by two processes; Isobutylene polymerisation followed by hydrogenation of the polymer, and by alkylation of butylenes and isobutane. Of the two processes, alkylation was much the more important from the standpoint of volume produced.

(a)	Isobutylene Polymerisation and polymer hydrogenation

Manufactured at Leuna, Ludwigshafen-Oppau, and Heydebrek.

Isobutyl alcohol was synthesized from Carbon monoxide and Hydrogen. It was then dehydrated to isobutylene. The Isobutylene was then polymerized to the dimer and trimer, and distilled off. The dimer-trimer mixture was then hydrogenated to form the isoparaffins. (Details of process are listed in the reference source).

This was known as “ET 110” or “Di 1000” and had the following properties.

Table 13. Properties of Isoparaffins from Isobutylene Polymerisation & Hydrogenation

| Property | Value |
| --- | --- |
| Density @ 15C | 0.71 |
| Distillation - Initial Boiling Point deg C | 80 |
| 10% deg. C | 75 |
| 50% deg C | 101 |
| 90% deg C | 110 |
| End Point (Final Boiling Point) deg C | 196 |
| Octane Number (Motor Method) Unleaded | 98 |
| Octane Number (Motor Method) 4.35 cc TEL/US Gal | 115 |

Before the advent of the alkylation process, isobutylene was produced by isobutane dehydrogenation at Leuna, Pölitz, and Scholven. Polymerisation and polymer hydrogenation systems were used to convert this isobutylene to “T-52”, a product nearly identical to ET 110.

(b)	Alkylation

Although research and development work on alkylation was started in Germany prior to 1940, the commercial production of alkylate did not begin until 1943. Prior to the time Leuna, Pölitz, and Scholven had been producing isobutylene by isobutane dehydrogenation, and those dehydrogenation were then shifted to normal butane feed.

In early 1944, these three plants were still the only operating alkylation units, but plants were being constructed in Wesseling, Brüx, Böhlen, and Blechhammer. These plants were not completed.

Normal butane dehydrogenation and isomerisation processes were both in use in Germany.

By the application of the processes of dehydrogenation, isomerisation, and alkylation, C4 components from the large coal and coal tar hydrogenation plants could be totally converted to butylene alkylate. Only Butylene alkylation was practiced in Germany, no propylene or amylene alkylation was carried out commercially. (more explanation is detailed in the Source)

Butylene Alkylation Plant (more detail on operating parameters are listed in the reference source).

Reactor Feed

Isobutane	54.8%

n-Butane	34.0%

n-Butylene	4.3% (of which a Butylene 43% and b Butylene is 57%)

Propane	6.9%

(Ratio Isobutane to olefin in feed 13)

Output

Aviation alkylate (% Vol. of total Debutanised alkylate)	93.5%

MON of Aviation Alkylate (unleaded)				 94

MON of Aviation Alkylate (4.35 cc TEL/USG or 0.115 %)	110

Table 14. Composition of German Alkylate

| Composition | % Vol. |
| --- | --- |
| 2,3 Dimethyl Butane | 6 |
| 2,4 Dimethyl Pentane | 6 |
| 2,2,4 Trimethyl Pentane (‘Iso-octane’) | 21 |
| 2,3,4 Trimethyl Pentane | 29 |
| 2,3,3 Trimethyl Pentane | 28 |
| Nonanes | 10 |

(c)	The Peroptan Synthesis

The premium value of Triptane (2,2,3 Trimethyl Butane) was recognized in Germany and much effort was spent to develop a method for its synthesis. In 1943 a program of study was undertaken this lead to a process and product known as “Peroptan”, but the construction of a plant never progressed beyond the design stage. (More detail on the chemical process is detailed in the reference source).

(d)	Isomerisation of Normal Paraffins

Normal butane isomerisation plants producing isobutane for alkylation had been built at Leuna, Böhlen, Blechhammer and Scholven. (Process described in reference source).

There was no commercial isomerisation of pentane in Germany, but the process was extensively studied in the laboratory.

Some research was conducted on isomerisation of C6 paraffins, however some cracking occurred.

Isomerisation of normal heptane was studied but found impossible to avoid cracking of heptanes. (More detail on the chemical reactions is found in the reference source).

# 2. Synthesis of Aromatics and Production of Base Stocks

The base stocks comprised 85% of the total aviation gasoline volume.

Most of the base stocks originated in coal and coal tar hydrogenation plants. Only a very small volume of carefully selected petroleum fractions was blended directly into aviation gasolines. The large high pressure hydrogenation plants at Leuna, Scholven, Pölitz, Brüx, Galsenkirchen, Böhlen, Magdeburg, Blechhammer and Wesseling all produced aviation base stocks. (Process described in reference source).

The B4 gasoline was hydrogenated gasoline stabilized to a specific vapour pressure (7.0 psi). The quality varied somewhat, depending upon the raw material to hydrogenation, and the individual gasolines needed some quality correction either with small amounts of isoparaffins or outside base stocks.

Table 15.. Properties of B4 Base Stocks from High Pressure Hydrogenation Plants.

| Feed to Hydrogenation | Brown Coal | Stein Coal | Brown Coal Tar | Stein Coal Tar |
| --- | --- | --- | --- | --- |
| Density @ 15 deg .C | 0.723 | 0.73 | 0.725 | 0.725 |
| Distillation % distilled @ 100 deg. C | 65 | 57 | 58 | 65 |
| End Point (Final Boiling Point) deg C | 132 | 153 | 150 | 160 |
| Paraffins Content % Vol. | 53 | 40 | 60 | 37 |
| Naphthene Content % Vol. | 42 | 52 | 30 | 55 |
| Aromatic & Olefin Content % Vol. | 5 | 8 | 10 | 8 |
| Octane Number (Motor Method) Unleaded | 71 | 73 | 69 | 76 |
| Octane Number (Motor Method) @ 4.35 cc TEL/US Gal | 90 | 91 | 89 | 94 |

The supply of high quality 85% base stock component to C3 grade aviation gasoline involved additional processing of the hydrogenated gasolines, aromatizing processes were invoked. (Several processes are discussed in the reference source).

(a)	The DHD Process

The DHD process (developed at Ludwigshafen) was a catalytic process for increasing the aromatic content of a gasoline through naphthene dehydrogenation and paraffin cyclization

At the end of the war four (4) DHD plants were in operation; Ludwigshafen, Leuna, Scholven, and Pölitz.

Because of their high naphthene content, gasolines from Stein coal and coal tars were preferred feeds to DHD units. By altering the operating conditions to encourage paraffin cyclization as well as naphthene dehydrogenation, gasolines from brown coal and brown coal tars were also greatly increased in aromatic content by this operation.

(More detail on the process is listed in the reference source).

Table 16. Properties of Final Product from DHD Process

| Density @ 15 deg. C | 0.780 |
| --- | --- |
| Distillation % distilled @ 100 deg. C | 48 |
| End Point (Final Boiling Point) deg. C | 171? |
| Paraffins Content % Vol. | 30 |
| Naphthene Content % Vol. | 20 |
| Aromatic Content % Vol. | 50 |
| Olefin Content % Vol. | 0.5 |
| Octane Number (Motor Method) Unleaded | 80 |
| Octane Number (Motor Method) 4.35 cc TEL/US Gal | 92 |
| MON Unleaded of Residual Oil after aromatic extraction | 70 |

For the highest quality aviation gasoline, the Luftwaffe insisted on a high aromatic content.  The 700 atmosphere hydrogenation method met this requirement, but only as far as very high aromatic middle oil feed stocks were concerned.  Another method was therefore adopted for the main production of first quality aviation gasoline which consisted of the dehydrogenation of the 80 – 185 deg C cut of gasolines obtained from petroleum distillation, or by 300 atmosphere splitting hydrogenation of coal and tar products.  In particular, this process was applied to the comparatively low quality gasoline distilled from the crude saturation stage product.  Usually, the naphtha cut was 80-85% by weight of the original petrol.  The process is discontinuous, reaction periods of rather more than 100-150 hours for bituminous coal naphthas and about 60-80 hours for brown coal and petroleum feeds alternating with 20-24 hour catalyst reactivation periods.

The hydrocarbon gas made in the reaction is roughly made up of equal proportions of C1, C2, C3 and C4 hydrocarbons.  Heavy polymer is formed during the reaction and re-distillation of the crude product to an end point of 180º C left a residue of approximately 4% by weight.

The final DHD naphtha contains 65-68% aromatics, and when blended back with the corresponding light gasoline fraction gives a finished DHD gasoline with an aromatic content of 40-55% and a minimum octane number of 78 unleaded and 87 with 0.09 vol. % of tetraethyl lead (or 3.41 cc TEL/USG.  The octane number of DHD gasoline made from coal hydrogenation and bituminous coal tar products was at least 2 units higher.

It is also interesting to note that the yield of finished DHD gasoline from brown coal tar or petroleum middle oil was only 71-72%.  This compares with an 89-90% yield of ordinary 6434 hydro-gasoline, showing that the Luftwaffe were prepared to sacrifice up to 20% output in order to obtain a highly aromatic fuel.  In assessing this cost of the improvement in aromatic content, it must, of course, be borne in mind that the hydrocarbon gases, particularly the butane produced in the DHD process; could be used for increasing the output of other aviation components, such as iso-octane or alkylate.

The DHD process was first tested out on a large scale on a modified vapour phase hydrogenation stall at Pölitz.  This plant also had the first properly designed commercial unit, which started up at the beginning of 1942.  DHD plants had also been in operation at Scholven and Leuna and were in course of erection at Blechhammer, Böhlen, Brux and Ludwigshafen.

b)	Hydroforming

There were two (2) hydroforming plants in operation in German territory. Both were located in the Moosbierbaum refinery near Vienna. A straight run petroleum gasoline boiling from 75 to 165 Deg. C was hydroformed in conventional discontinuous (batch) units. Both Roumanian and Austrian crudes were processed at this refinery.

Table 17.  Properties of Feed and Product - Hydroforming process

| Hydroforming Process | Feed | Product |
| --- | --- | --- |
| Density @ 15 deg. C | 0.750 | 0.776 |
| Distillation - Initial Boiling Point deg C | 62 | 44 |
| % Distilled @ 100 deg. C | 18 | 36 |
| % Distilled @ 160 deg. C | 95 | 94 |
| End Point (Final Boiling Point) deg C | 166 | 166 |
| Olefin Content % Vol. | 0.5 | 1.5 |
| Aromatic Content % Vol. | 14 | 54 |
| Naphthene Content % Vol. | 44 | 8 |
| C1-C3 % wt. | - | 11.0 |
| Isobutane %wt. | - | 1.3 |
| Normal Butane %wt. | - | 2.7 |
| RVP psi | 5.3 | 5.1 |
| Octane Number (Motor Method) Unleaded | 58 | 80 |
| Octane Number (Motor Method) @ 4.35 cc TEL/US Gal | 79 | 91 |

(c )	Synthetic Alkyl Aromatics

The only important commercial synthesis of alkyl aromatics was of Diethyl Benzene. These plants were at Huls and Schopau and produced together about 300 Bbls/day (47 K.Litre/day) of this material named “Kybol” as a by-product of styrene. Benzene was alkylated with ethylene and the product contained some diethyl benzene. No Cumene was being made.

(Details are listed in the reference source on other research developments)

(d)	Catalytic Cracking

There were no commercial scale catalytic cracking units in operation in German areas. The pilot plants and designs were similar to TCC process in the USA. The product from these pilot plants were:

36% yield of gasoline of 166 deg C End Point.

20% wt. Aromatics, 4% wt. Olefins.

Octane quality was MON = 75 (Unleaded), and MON =94 @ 4.35 cc TEL/USG).

By-products included Butanes 6.7% wt. of which 3.1% was isobutane.

# Conclusions on German Production

The following conclusions were made by the Technical Mission to Germany in July 1945.

Maximum rate of production of total aviation gasolines achieved by Germany during the war was roughly 50,000 Bbls/day (8 M.Litres/day) of which essentially the entire volume came from coal and coal tar hydrogenation plants. Of this total volume, 10% synthetic isoparaffins, 40% high aromatic base stocks produced by processing of hydrogenation plant gasolines, and the remaining 50% was almost entirely hydrogenation plant gasolines of aviation endpoint and volatility.

Two grades of aviation gasoline were produced,

B4 (blue) MON 91 (leaded 0.115% or 4.35 cc TEL/USG), 10% aromatics, 50% distillation point 105 deg C. Produced directly by the addition of TEL to the entire liquid product from the large coal and coal tar hydrogenation plants. The volatility adjusted to 7 psi RVP.

C3 (green) MON 95 (leaded 0.115% or 4.35 cc TEL/USG), 40% aromatics, 50% distillation point 110 deg C. The C3 grade was a leaded blend of 15% Vol. synthetic isoparaffins and 85% Vol. of a base stock containing 45-50% vol. aromatics, produced by further processing of a hydrogenated gasoline (DHD Process) almost identical to B-4. The C3 represented 2/3 of the combined volume of the two grades.

(Author’s Note: There were in fact three grades of aviation gasoline.

A3 fuel of 80 C.F.R. motor octane number and having an ethyl alcohol content of ca. 30% volume was used for training aircraft.

B4 fuel of 89 octane number was the normal grade for operational aircraft.

C3 fuel of 95 octane number was used in aircraft powered with high duty engines.

In addition, an easy starting fuel, comprising a blend of light gasoline and 5% lubricating oil and an unleaded "Running Out" fuel were also used.)

Synthetic isoparaffins were manufactured primarily by the alkylation of butylenes and isobutane. Some isobutylene polymerisation and polymer hydrogenation was carried out. No propylene or amylene alkylation was being done. No triptane synthesis was developed.

# Oil Supplies, Refineries and Plants

At the start of the war 1939 Germany had only 0.3% of the world’s crude oil supplies, its allies Rumania had 2%, while the USA had 60%, Venezuela 11%, Russia 10%. This lack of crude oils was the drive to (a) gain access to crude supplies by conquest and (b) utilize coal as a fuel source for gasoline.   The marked difference between the Axis production and the Allies was the high dependence on coal as a raw material and on processes other than the ‘typical oil refinery’; for example Hydrogenation, Fischer-Tropsch and Coal Tar distillation.

The following figures briefly outline some of these processes.

Figure 2. Typical German Oil Refinery

World War II Starts - German Aviation Stock Position

The German aviation gasoline position at the start of World War II on the 1st. August 1939 was such that they could support a war-time demand just under five months, this was probably better than the Allies who were working on less than 3 months. The critical product was not aviation gasoline in fact, but anti-knock additive Tetra Ethyl Lead for aviation purposes which was more critical. The situation was as follows:

Table 18. German Aviation Stock position 1st. August 1939

| Product | War-time needs Tons/month | Production in Germany | German Stocks at 1st. Aug. 1939 including reserves Tons | Duration (1) Months |
| --- | --- | --- | --- | --- |
| Aviation gasoline | 152,500 | 46,200 | 480,000 | 4.8 |
| TEL | 430 | 158 | 860 | 2.6 |
| Aviation Diesel fuel | 3,300 |  | 48,000 | 14.5 |
| Aviation Motor Oil | 9,500 | 2,400 | 50,000 | 7.0 |

Note 1. Probable duration (in months) of full covering of the war-time demand by domestic production and available stocks.

It would appear that the Germans were not expecting a long war of attrition of resources, or perhaps it hoped to acquire the stocks of other nations such as those in the Middle East, specifically the rich oil fields of Persia and Balkans.

German Petroleum Production

In 1940 the demand for petroleum products for motor gasoline was 24% of all production, with aviation gasoline around 14%; like the Allies by 1943 aviation gasoline was in high demand and German motor gasoline had reduced to 15% while aviation gasoline had increased to 24% of all production.  The following table shows the distribution of all gasoline petroleum production in Greater Germany in 1940 by process for a total production of 4,564,000 metric tons.

Table 19. German 1940 Gasoline production by process type.

| 1940 Process Thousands metric tons | Aviation Gasoline |  | Motor Gasoline |  |
| --- | --- | --- | --- | --- |
| Crude Refining | 18 | 2.8% | 135 | 12.0% |
| Hydrogenation | 612 | 95.0% | 299 | 26.6% |
| Fischer-Tropsch |  | 0.0% | 223 | 19.8% |
| Coal Tar Distillation |  | 0.0% | 24 | 2.1% |
| Alcohol |  | 0.0% | 80 | 7.1% |
| Benzol | 14 | 2.2% | 364 | 32.4% |
| Total | 644 | 13.8% | 1,125 | 24.2% |

The process types (and percentages) for the manufacture of aviation gasoline were to remain the same for the next four years.

In the same way as the Allies demanded more avgas, the demand for more German aviation gasoline increased as can be seen in the following graph, where the motor gasoline from 1940 to 1944 remained essentially the same, while aviation gasoline increased nearly threefold. Of particular note was the use of captured aviation gasoline stocks, and imports of shipments outside of Germany directly to the German forces outside Germany. Captured stocks were a feature of the German advances in 1940 into France.

Graph 5. German Aviation Gasoline Supply 1940-1944 (Production, Captured Stock/Imports)

The stock position was a different story.

German Stock Position 1940-44

The annual stock position of Aviation gasoline of interest was over the main war years 1940 to 1944, (1939 was known as the “Phoney War” and by 1945 Germany was in disarray). The following graph shows the changing situation. The aviation gasoline production includes imports and loot.  As the war progressed favourably for the Axis in 1940/41 with victories in France, and conquests in Eastern Europe, the stock position increased, as did consumption. However, as the war progressed in Africa in 1942 and the Eastern Front, the stock position decreased significantly while production and consumption increased. By 1943 production was at a maximum which helped ease the stock position even though consumption had increased again. By 1944 the bombing raids, defeats on the Eastern Front, the onslaught of the Allies Normandy invasion from the west (June 1944), the Allied advances through Italy were all taking their toll on both aviation gasoline production (and with Axis aircraft losses and decreased production – hence less consumption). By early 1945, the Allies had control of the air, despite the new breed of German aircraft – the jet fighter.

Graph 6. German Aviation gasoline position 1940-1945

The impact of the bombing raids in Germany and Roumania can be seen in the decreases in 1944.

Photo 22. USAAF B-24 Liberator bombers attack Ploesti refineries 1943

Refineries under Axis Control 1943

The Allies had identified ‘Oil’ as a priority target from the start of the war, and intelligence efforts were directed to obtaining information on the Axis oil position. There were many reports by the Allied Joint Intelligence Committee which received reports from Enemy Oil Committee on the on the status of crude, availability, production and consumption. With the rapid advance of the German forces into Europe many refineries came under Axis control. However, by 1943, the Allies had targeted these refineries and some were severely damaged, if not destroyed, others were captured.

The following list of refineries shows the extent of the Axis control and damage to the German war effort.. One of these reports illustrates the views in November 1943

Present Position of the Refining Industry in German-Held Europe (22 November 1943)

Report by the Enemy Oil Committee

10.	Whereas German-held Europe had, at the time of our last report (10 June 1943) a comfortable and fairly well distributed excess of refining capacity, it is now clear that the excess had been reduced to the danger point. The loss of certain Italian refineries, the destruction of certain Italian, German and Rumanian refineries, added to the impotence of the French and Lowlands refineries, has completely changed the outlook. For the first time it is evident that an immediate effective blow at the European refining industry would cause crude oil to be shut back in the wells.

11.	The Allied capture of the Naples and Beri refineries represents a permanent loss to the German economy of 450,000 (a)* tons per year of crude handling capacity. The approach of Allied armies and the destruction of crude supply lines by guerrillas is understood now to have immobilized the remaining Italian capacity to the further extent of 1,600,000 (b) tons per year. The damage to Rumanian and German refineries represents a loss, at least for several months, estimated at 2,795,000 (c) tons and 540,000 (d) tons respectively. The destruction of tankage and dismantling of equipment in Northern France and the Lowlands is calculated to represent a loss of 5,030,000 (e) tons. The total actual loss therefore is estimated to have been 10,415,000 (f) tons. In addition, it must be noted that, excepting the small plants for local oil, the remaining capacity in France and the Lowlands, 4,085,000 (g) tons, is unavailable due to its remoteness from the centres of production and consumption. Furthermore, of the so-called “capacity” in Germany 720,000 (h) tons represents simply lube or gasoline finishing capacity which could not be adapted to handling crude. This leaves only 10,895,000 (i) tons, of the original 26,115,000 (j) tons of European refining capacity, now available to the Germans for crude handling.

12.	If subjected to no further molestation it is believed that a large part if not all of the damaged refining equipment in Rumania and Germany could be replaced by the Spring of 1944. Equipment had been removed from French refineries which, with attendant facilities, would be capable of handling several million barrels per year of crude oil. Some of this equipment may be destined for the Lobau refinery, now under construction, or for the rumoured new refineries to be built at Komaron and Smederevo. It is even possible that “unknown” plants have been erected with these transplanted facilities, although no intelligence whatsoever has been received to such effect. On the other hand, a large part of it may have been consumed in the construction of the new synthetic plants. In any case, this or newly fabricated equipment could be installed in the presently damaged refineries and a safe margin of excess capacity re-established by next Spring. It is obvious that the recovery of such a margin, before further raids, would be a matter of first urgency for the Germans.

13.	Ignoring the possibility if “unknown” plants it is deemed most probable that for the treatment of the 8,810,000 (k) tons of crude petroleum currently in production, there is refinery capacity in German-held Europe, accessible and operable, totalling 10,895,000 (i) tons per year. This leaves a margin of only 2,085,000 (l) tons excess capacity which is hardly adequate since it is only 24% of the crude produced. This available excess capacity and the deficit capacity, in thousands of metric tons yearly, is distributed approximately as follows:

* These letters refer to the items in the Annex

Table 20. Country and excess crude capacity

| Country | Excess Crude Capacity | Country | Deficit Crude Capacity |
| --- | --- | --- | --- |
| Germany | 252 | Austria | (468) |
| Czechoslovakia | 668 | Hungary | (400) |
| Italy | 40 |  |  |
| Yugoslavia | 130 |  |  |
| Rumania | 1,358 |  |  |
| Poland | 400 |  |  |
| France, Estonia | 45 |  |  |
| Total Excess | 2,953 | Total Deficit | (868) |
| Net Excess |  | 2,085 |  |

14.	From the above considerations it appears that a successful new attack on the European refining industry would have an excellent chance of valorizing the Ploesti raid. It also appears that the sooner such an attack is made the greater would be the return.

15.	 Notes on Report of Refining Industry in German-held Europe Nov 1943

(Figures in thousands of metric tons per year, are those of the report of 10 June 1943 except as regards present crude productions, item k).

Italy

(a) Italian Loss

Naples	200

Beri	250

Total	450

(b) Italian Immobilization

Aquila-Trieste		300

SIAP-Trieste		150

ROMSA-Fiumo 	150

AGIP-Venice		350

NAFTA-Spezia		400

ANIC-Leghorn		250

Total			1,600

Rumania

(c) Rumanian loss

Colombia (100%)	535

Concordia (100%)	225

Credit Minier (100%)	535

Stoaua (100%)		1,500

Total			2,795

Germany

(d) German loss (assumed)

Rhenania-Harburg (40%)	220

Ebano-Harburg (40%)		160

Eurotank-Pet’haven (40%)	160

Total				540

France

(e) French loss

Dunkerque		450

Courchelettes		300

Gonfreville		1,600

Port Jerome		1,250

Gravenchon		300

Petit Couronne		750

Consomm.-Donges	130

Pechel-Donges		250

Total			5,030

All (f) Total actual loss

Italian loss		450

Italian shutdown	1,600

Rumania damage	2,795

German damage	540

French demolition	5,030

Total			10,415

N.W. Europe immobilization (g)

Pauillac			500

Bec D’Ambes		350

Frontignan	300

Martigues	650

Berre		550

L’Avera		400

Belgium, Netherlands, Denmark, Norway	1,335

Total		4,085

German non-crude capacity (h)

Vacuum-Schulau		15

Albercht-Grasbrook		60

Scliemann-Grasbrook		65

Rhenania Ossag-Grasbrook	130

DPAG Neuhof			65

Schindler- Neuhof		40

Deutsche Gasoline-Dollbergen	40

Vacuum- Bremen		80

Schindler-Peine			20

Neidersachen-Dollbergen	10

Deutsche Gasoline-Emmerich	60

Ersag-Wintershall-Salzbergen	30

Schmitz-Dormund		20

Rhenania-Ossag-Menheim	115

Total				720

By 1943 although the Allies had made a concerted attack on the refineries, there were still many in operation. The Enemy Oil Committee would continue their reviews right through to May 1945 providing intelligence to the Joint Intelligence Committee for the purpose of directing the air war against Axis Oil.

Axis Oil Plants 1944

Additional information from the U.S. Strategic Bombing Survey provides an enlightening view of Axis Oil Plant operations and the impact of the bombing raids particularly in mid-1944. This particular table compares all production by various processes and sources from normal output with that obtained at the peak of the bombing raids (Sept ’44).  No data was available for normal output by individual countries. At this time the Allied forces were liberating Italy in the south, and France, Holland and Belgium in the west, while the Russians were attacking in the East through Rumania.

Table 21. Estimated Output of Axis Oil Plants 1944

| All Products (Thousands metric tons/month) | Nominal Capacity | Normal output | July '44 Estimated output | Sept '44 Estimated output |
| --- | --- | --- | --- | --- |
| Synthetic Plants |  |  |  |  |
| Bergius Hydrogenation |  | 368 | 98 | 45 |
| Fischer-Tropsch |  | 112.5 | 56 | 22 |
| Total from Synthetic Plants |  | 480.5 | 154 | 67 |
| Percent of pre-raid total |  | 100% | 32% | 14% |
| Crude Oil Refineries |  |  |  |  |
| (Thousands metric tons/month) | Nominal Capacity | Normal output | Estimated output | Estimated output |
| Germany | 149 | n/a | 31 | 27 |
| France | 260 | n/a | 6 | 0 |
| Belgium | 42 | n/a | 0 | 0 |
| Holland | 52 | n/a | 0 | 0 |
| Czechoslovakia | 48 | n/a | 31 | 8 |
| Poland | 78 | n/a | 43 | 0 |
| Austria | 54 | n/a | 7 | 18 |
| Hungary | 60 | n/a | 14 | 9 |
| Italy | 80 | n/a | 3 | 0 |
| Yugoslavia | 21 | n/a | 3 | 0 |
| Albania | 2 | n/a | 2 | 0 |
| Roumania | 740 | n/a | 185 | 0 |
| Total from Crude Oil | 1,586 | 697.5 | 325 | 62 |
| Percent of pre-raid total |  | 100% | 47% | 9% |
| Total Other sources (benzol, alcohol, tar oils) |  | 166 | 192 | 175 |
| Grand Totals (‘000 Metric tons) |  | 1,344 | 671 | 304 |
| Percent of pre-raid total |  | 100% | 50% | 23% |

This information shows that by September 1944,

Axis Synthetic Oil Plants were operating at only 14% of their normal output.

Crude Oil Refineries were down to 9% of their normal output.

The vast oil crude oil and refineries of Rumania (in particular Ploesti) accounted for over 50% of all crude refinery output in July 1944, and their loss to the Russians dramatically cut supply of valuable petroleum products. The Russians captured Ploesti on the 22nd. August 1944.

Only 43% of the normal crude oil refinery capacity (of 1,586,000 metric tons/month) could be achieved prior to July 1944. This was due to lack of crude, bombing raids, sabotage and belligerence of the occupied workforce. The monthly capacity of France, Belgium and Holland were 260,000 tons, 42,000 tons, 52,000 tons respectively, however the estimated output in July 1944 was only 6,000 tons from France, and none from Belgium and Holland. By August 1994 most of France had been liberated and output to the Axis cause had ceased. (By contrast, the American refinery processes were operating at capacities exceeding design)

By September 1944, with the advances of the Allied forces in the south and Russian forces in the east, Italy, Poland, Yugoslavia and Albania had all ceased to be a source of petroleum products for the Axis cause.

Total Estimated output for September 1944 was only 23% of the pre-bombing raid total highlighting the declining fortunes of the Axis powers.

German Refining Capacity

Although the majority of German aviation gasoline was made from Synthetic fuel, there was substantial oil refining capacity in Greater Germany and Protectorate. The total crude distillation capacity including re-run plants was 3,000,000 tons per year. (excluding the Poland-Galician area).

The following table shows the company, location and capacity, also included is reference to the map of their location. All of these would be limited by the supply of crude oil, coal and other raw materials. It is interesting to note the facilities of German affiliates of the British/Dutch Shell Company and the American Vacuum Oil Company were still in use. This is not surprising to find that the plants established by these companies prior to the war continued in operation.

# Oil Refineries

Table 22. German Oil Refining Capacity (Old Germany)

| No. | Company | Location | Capacity Tons/month |
| --- | --- | --- | --- |
| 26 | Rhenania (Shell) | Harburg (Hamburg) | 60,000 |
| 27 | Rhenania (Shell) | Grasbrook (Hamburg) | 17,000 |
| 28 | Rhenania (Shell) | Wilhelmsburg (Hamburg) | 8,350 |
| 29 | Rhenania (Shell) | Monheim | 16,000 |
| 30 | Deutsche Erdoel A.G. | Wilhelmsburg | 10,000 |
| 31 | Deutsche Erdoel A.G. | Heide | 16,000 |
| 32/33 | Deurag-Nerag | Misburg | 27,000 |
| 34 | Wintershall | Lutzkendorf | 10,000 |
| 35 | Wintershall | Salzbergen | 5,250 |
| 36 | Deutsche Gasolin | Emmerich | 6,000 |
| 37 | Deutsche Gasolin | Dollbergen | 4,200 |
|  | Pheonix-Sengawald | Hannover | 1,700 |
| 38 | Mineraloel and Asphalt A.G. | Ostermoor | 10,000 |
| 39 | Oelwerke J. Schindler | Hamburg | 2,500 |
| 40 | Oelwerke J. Schindler | Peine | 1,500 |
| 41 | Ebano | Harburg (Hamburg) | 45,000 |
| 42 | Eurotank | Hamburg | 35,000 |
|  | I.G. | Oppau | 1,250 |
| 43 | Vacuum Oel A.G. | Oslebshaussen | 10,000 |
| 44 | Vacuum Oel A.G. | Wedel | 5,000 |
| 45 | Pechelbronn | Pechelbronn | 10,000 |
| 46 | Ernst Schliemann’s Oelwerke | Hamburg | 5,000 |
| 47 | Albrecht & Co. Mineraloelwerke | Hamburg | 1,250 |
|  | Adler | Hannover | 600 |
|  | Korff | Bremen | 500 |
|  | Freital | Leipzig | 250 |
| 48 | Ludwigshafen | Mannheim | 1,250 |

| No. | Company | Location | Capacity Tons/month |
| --- | --- | --- | --- |
| 49 | Shell Floridsdorf | Wein-Floridsdorf | 13,000 |
| 50 | Vacuum Oel A.G. | Kagran (Wein) | 9,000 |
| 51 | Dea-Noya | Schwechat (Wein) | 15,000 |
| 52 | Benzol-Verband | Vosendorf | 7,00 |
| 53 | Deutsche Gasolin | Korneuburg | 7,000 |
| 54 | I.G. | Moosbierbaum | 12,500 |
| 55 | Ostmarkische Mineraloelwerke | Lobau (Wein) | 25,000 |

Table 23. German Oil Refining Capacity (Protectorate Czech Area)

| No. | Company | Location | Capacity Tons/month |
| --- | --- | --- | --- |
| 56 | I.G. | Pressburg | 16,700 |
| 57 | Oderfurter Mineraloelwerke | Oderfurt | 6,000 |
| 58 | Vacuum Oel A.G. | Kolin | 12,500 |
| 59 | Fanto | Pardubitz | 12,500 |
| 60 | Oderberger | Oderburg | 1,000 |
| 61 | Kralupy | Kralupy (Prague) | 1,500 |
| 62 | Privos (Moravska Ostrava) |  |  |
| 63 | Kontinentale Oel | Trzebinia | 22,500 |
| 64 | Vacuum Oel A.G. | Kagran (Wein) | 8,500 |
| Five Refineries in these areas |  | Drohobycz | 28,300 |
|  |  | Jazzlo | 5,800 |

Synthetic Fuel Plants

The Synthetic Fuel plants types were Hydrogenation and Fisher-Tropsch plants and were distributed throughout Greater Germany. The following tables list the installed capacity Tons per month for these plants.

Table 24. Synthetic Fuel Production Plants (Hydrogenation type)

| No. | Company | Installed Capacity Tons/month |
| --- | --- | --- |
| 1 | Leuna | 50,000 |
| 2 | Boehlen | 25,000 |
| 3 | Magdeburg | 20,000 |
| 4 | Zeitz | 25,000 |
| 5 | Scholven | 20,000 |
| 6 | Gelsenburg | 35,000 |
| 7 | Welheim | 12,500 |
| 8 | Poelitz | 50,000 |
| 9 | Luetzkendorf | 4,000 |
| 10 | Wesseling | 17,000 |
| 11 | Ludwigshafen – Oppau | 4,000 |
| 12 | Moosbierbaum | 7,500 |
| 13 | Bruex | 50,000 |
| 14 | Blechhammer | 13,750 |
| 15 | Heydebreck | 3,330 |
| 16 | Auschwitz | 2,500 |

Table 25. Synthetic Fuel Production Plants (Fischer-Tropsch)

| No. | Company | Installed Capacity Tons/month |
| --- | --- | --- |
| 17 | Ruhrchemie | 5,000 |
| 18 | Victor | 3,330 |
| 19 | Rheinpreussen | 5,500 |
| 20 | Krupp | 5,500 |
| 21 | Essener Steinkohle | 7,080 |
| 22 | Hoesch | 4,000 |
| 23 | Schwarzheide | 14,170 |
| 24 | Schaffgotsch | 3,333 |
| 25 | Luetzkendorf | 1,000 |

Figure 3. Location of Hydrogenation, Fischer-Tropsch Plants & Refineries in Greater Germany

German TEL Plants

The Ethyl organisation, either directly through the Ethyl American office or through the Ethyl Export Corp in London, licensed plants abroad for the manufacture of anti-knock fluid in the period 1936-1940. Under shared ownership arrangements the construction of factories for the manufacture of anti-knock fluid took place in Germany, France and England.

The German plant was built in 1936.

The relations between Ethyl Corporation and Ethyl Export Corporation on the one hand, and the Germans on the other during the 1930’s were in part conditioned by the always complex and sometimes debatable agreements between Standard Oil of New Jersey (later Esso) and the German chemical trust I G Farben.

In 1929 there was an understanding between I G Farben and Standard Oil of New Jersey for exchange of patent information. Standard Oil of New Jersey had long been interested in German experiments in the hydrogenation of coal, and then especially in the development of synthetic rubber.

With the assistance of F D Banks of Ethyl Export Corporation, in 1931 Lufthansa began a series of tests using the Ethyl additive. In their flights to China in 1933 the German pilots carried litre cans of TEL that had been imported through Ethyl Export of London. The German flight crews blended the TEL with fuel at the Gobi Desert stop. The German nation imported only 12 tons of TEL in 1934, 104 tons in 1935. In 1936, only 4.65 tons of TEL because in that year the German TEL plant began operating.

German TEL Plant

Negotiations for this plant had begun in the winter of 1934-35. At the request of the German Ministry of Aviation, I G Farben communicated with the Ethyl Gasoline Corporation about the possibility of constructing such a plant.

Ethyl’s London office tended to oppose the building of the German plant. Banks noted “When the Germans indicated that they wished to build a lead manufacturing plant in their own country, this was at first resisted by us as uneconomic. Eventually, we had to agree to a joint manufacturing company formed between ourselves and I G Farben.” The company Ethyl G.m.b.H. was owned 50% by I G Farben, 25% by subsidiaries of General Motors and 25% by Standard Oil of New Jersey. The stocks of these subsidiaries (GM and Standard Oil NJ) were soon transferred to Ethyl Export Corporation. After agreement had been reached between Ethyl and I G Farben, the German chemists went to DuPont and Ethyl for training. The plant was built at Gatel, about 50 miles west of Berlin. By April 1936 the Gatel plant was supplying TEL for use in aviation gasoline.

Second Secret German TEL Plant

The second plant in Germany was built in 1939 at Frose, this theoretically without Ethyl’s knowledge or permission. The London office had heard rumours of an unauthorised plant and through a mailing mix-up obtained first-hand information on the plant. By mistake the Germans had put an envelope directed to the Frose plant into a packet of mail addressed to Ethyl Export. Banks promptly steamed open the envelope, read all about this unauthorised plant, resealed the envelope and returned it to the Germans, who were telephoning frantically for the return of this particular piece of mail. Banks notified the British authorities when then knew exactly where the second plant was located.  These German TEL plants were to become targets for Allied bombing raids during World War II.

# German Industry goes underground

The intensity of the Allied bombing was such that to minimise this threat, the Germans attempted to develop and oil industry underground. Units were to be comparatively small size, and dispersed throughout the country. Obviously a very large number would be needed to meet the German war requirements, and this job would take a long time – slave labour was often used for the heavy underground construction work.

A system of names was developed to cover each type of underground oil plant, as follows:

Table 26. German Code Names for Underground Oil Plants 1945

| German Name | English equivalent | Operation |
| --- | --- | --- |
| Schwalbe | Swallow | Hydrogenation plant |
| Kukuck | Cuckoo | Iso-octane and Polymerisation plant |
| Dachs | Badger | Lubricating Oil plant |
| Taube | Pigeon | Cracking Plant |
| Ofen | Oven | Dispersed distillation plant |
| Rest | Grate | Batch distillation plant |
| Jakob | Jacob | Primitive cracking plant with an ‘ofen’ |
| Molch | Salamander | Plants for removing phenol & cresol from middle oil to make diesel fuel oil |
| Karpfen | Carp | Small Fischer-Tropsch plants |
| Rabe | Raven | Tetra Ethyl Lead plant |
| Iltis | Polecat | Super fractionation plant for manufacture of high octane aviation gasoline stocks |
| Biber | Beaver | Furfural extraction plant |
| Wuste | Desert | Shale carbonization plant |
| Krebs | Crab | DHD plant (dehydrogenation) |
| Dack | Lacquer | Plant for manufacture of nitrogen compounds |

These names were used with the intention of keeping secret the nature of the plant as well as its location. One plant in Austria was reported to have been finished and in operation. One or two others were approaching completion at the end of the war, but most although started were never completed.

Control of German Oil Industry

The German oil industry was completely controlled from Berlin. Operations were directed from a central bureau, and all plants closely coordinated. Very little finished product blending was carried out at the point of manufacture. Normally, the blending stocks, carefully disguised under code names, were shipped to WIFO (Wirtschaft Forschung), underground, camouflaged blending plants. A number of these plants throughout Germany were examined, and many were of tremendous size. At Hitzsacker and Aken the plants were in pine forests, and everything was underground including tanks, pump stations, and all delivery equipment. Blending formulas, amount of tetra ethyl lead to be used in aviation gasoline, or additives for lubricating oil were all specified by the central office in Berlin. In this way, the components of the finished product were known only to a limited number of persons high in the Nazi inner circle.

# “Target – Destroy Germany’s Oil Industry”

As the war in the world progressed, it was apparent that this could be more properly termed “Air War I”. In Europe, after the fall of France and the evacuation of the British from Dunkirk, it was essentially ‘War in the Air’ on the Western front.  In the Pacific, great naval battles would be fought using aircraft flown from aircraft carriers where the opposing ships never sighted each other. In North Africa, armies would advance, stall, or retreat depending on fuel supplies. Therefore all those involved knew the importance of maintaining and securing supplies of petroleum products. For the Allies, a major part of their strategy was to destroy the Axis oil industry, and this would be demonstrated by the strategic targeting by the bomber forces of both U.S. Army Air Forces and British RAF Bomber Command. Their targets were the refineries, chemical plants, oil depots and supply routes.

The strategic bombing efforts are shown in the following figure; it is interesting to note the inaccuracy of the bombing and the impact on production in early 1944.

Photo 23. USAAF B-24 Liberator bomber attacks oil facilities at Ploesti circa 1943

Figure 4. Impact of Allied Strategic Bombing on German Aviation Gasoline Production.

# Famous Scientist to U.S.

Some of the more famous were to be brought to the United States by the US Army (and US Navy) to work on U.S. programs, indeed it was reported in the New York Times (1945), that some 1,500 ‘Enemy Alien’s’ had been brought to the United States by the U.S. Army (which were fighting the ground war in Europe). Others would be captured by the Russians to do similar work. Perhaps the most noted to go to the U.S.A. were:

Dr. Werhner Von Braun - the German rocket scientist responsible for the Peenemünde V-2 Rocket project. He would later be the head of U.S. Army Missile Development, then Director of U.S. NASA Space Booster Program.

Hans Joachim-Pabst Van Ohain, Co-inventor of the Jet Engine.

Photo 24. Hans Von Ohain. - Lab Chief Scientist USAF Research Laboratory in the 70's.

Hans Joachim-Pabst Van Ohain, in partnership with the Heinkel Aircraft Company developed his engine for an operational model which resulted in the world’s first true jet powered flight on 27 August 1939 in the Heinkel He-178 aircraft. Emigrating to the United States in 1945, Von Ohain became a leading jet engineer for the U.S. Air Force and subsequently director of the USAF Research Laboratory for Propulsion and Energy Conservation at Dayton Ohio.

Photo 25. Heinkel He-178 first jet aircraft flight August 1939

# “Airspeed records  - Germany temporarily takes the record in 1944”

The war effort was directed to faster and better military aircraft, however some propaganda value could still be obtained for the country with the fastest plane. In 1944, this air speed record was achieved by the unusual German Dornier Do 335, which was powered by two Daimler-Benz DB 603G 12-Cylinder inverted-vee, liquid cooled engines in a push/pull configuration. Horsepower: 1,900 hp.

Photo 26. German Dornier Do-335

Photo 27. German Dornier Do-335

This record would only last until Aug 1944 when an American XP-47J ‘Thunderbolt’ in the U.S.A. would fly at over 800 Km/hr. (497 mph)

In this period, the (land-based) aircraft achieved the following:

| Date | Location | Pilot | Aircraft | Achieved Km/hr |
| --- | --- | --- | --- | --- |
| 1944 | Germany - Oberpfaffen | unknown | Dornier Do 335 VI (M1) (Ameisenbär) (CP+UA) [push/pull design] | 752 (467 mph) |

# Epilogue for the Axis years – The Have’s and Have Nots

The war years showed that the U.S. led the world in aviation gasoline production and supplied the Allies with the “Gasoline to Win a War”. The Axis powers lost because they did not have the quantity, nor the quality of aviation gasoline to sustain their war machine - even their territorial ambitions were driven by the conquest for oil. They failed, however they were to change aviation forever and demand for aviation gasoline would dramatically decline to be replaced by a new fuel – JET FUEL.

The Messerschmitt Me-262 was the first operational jet propelled aircraft. It was powered by two Jumo 004B turbine engines using a kerosene type fuel.  It was a formidable aircraft, some 100 mph faster than the best Allied fighters fuelled by avgas.

Photo 28. Messerschmidt Me-262 – the first operational jet aircraft in the world.

Photo 29. Jumo 004B with two-cylinder 10 HP Reidel engine at the right

Ironically, the each of two jet engines required a small starter petrol engine to start them. Yes - these starter motors operated on 87 Avgas. (B4). ‘B4 mit 3% Schmierstoff’. (Avgas 87 with 3% lubricant). The ‘A3’ triangle is for gasoline (training grade); the ‘B4’ triangle identification marking can be observed on the Me-262 engine nacelles of the aircraft located at the National Aerospace Museum in Washington DC (USA), and also at San Diego Aerospace Museum.

The engines were two cylinder 10 HP Reidel engines, it took 1 minute to bring the jet engines to 2000 rpm.  This small engine can be seen on the lower right side of the jet engine.

Photo 30. Jumo 004B with two-cylinder 10 HP Reidel engine (side view) – on display at San Diego Aerospace Museum 2003.

# Index

A

Acid Treating	27, 28

AGIP-Venice	56

Albercht-Grasbrook	58

alcohol	5, 19, 45, 52, 59

Allied 100 ON grade	12

Allied Joint Intelligence Committee	55

ANIC-Leghorn	56

aniline	31

Aquila-Trieste	56

Aromatics	5, 8, 10, 11, 19, 35, 37, 38, 43, 47, 49

AVIATION DIESEL FUEL	32

B

Bec D’Ambes	57

Benzene	8, 11, 13, 19, 49

Beri	55, 56

Berre	58

BIOS	21, 22, 30, 42, 75

BMW 132	14

BMW 132N	14, 39, 40

BMW 801	40

Böhlen	21, 43

British Specification 230	12

Bromine No.	7, 10, 11, 19

Brüx	43

C

CAFT	22, 26, 27, 28, 29

CFR method	39

CIOS	21, 22, 26, 28, 29

Colombia	57

Concordia	57

Consomm.-Donges	57

Contact Plant	27, 28

Courchelettes	57

Cracked Gasoline	29

Credit Minier	57

D

Daimler Benz 601	14

Daimler Benz 605B	3

Daimler Benz DB-605B	15

Deustche Versuchsanstalt für Luftfahrt	31

Deutsche Gasoline-Dollbergen	58

Deutsche Gasoline-Emmerich	58

Deutsche Vacuum Oel AG	22, 29, 36, 38

Dewaxing	28

Dewdren	5, 75

DHD process	30, 47

Diethyl Benzene	38

Distillate Rerun Unit	28

DPAG Neuhof	58

Dubbs	28

Dunkerque	57

DVL	31, 39

E

Ebano-Harburg	57

Eich B4	39

Eich C3	39

Enemy Oil Committee	55, 58

Ersag-Wintershall-Salzbergen	58

Ethanol	37

ethanol mix	38

Ethyl Propyl Benzene	38

ethylene bromide	31, 32

Eurotank-Pet’haven	57

Evans	28, 29

Existent Gum	7, 11, 18

F

FIAT	22

Fischer-Tropsch	21, 42, 50, 52, 59, 63, 64, 66

Focke Wulf FW-190A8	41

Focke Wulf FW-200 ‘Condor’	41

France	57

Frontignan	58

Furfural Unit	28

G

GAS TURBINE FUEL	33

Gelsenkirchen	6, 43

GM-1	31

Gonfreville	57

Gravenchon	57

H

hydrogenation	5, 6, 13, 21, 30, 32, 34, 38, 39, 44, 45, 47, 50, 64

Hydrogenation	6, 29, 44, 45, 47, 50, 52, 59, 62, 63, 64, 66

hydropetrol	5, 30

I

I.G. Farben	41

I.G. Fabenindustrie	21

I.G. Prüfmotor	30, 38

Ignatieff	26, 29

Induction Period	7, 11, 19

INTAVA	4, 29

Iron Carbonyl	9, 11, 19

Iso-octane	38, 39, 46, 66

Italy	56

J

JIOA	22

K

Komaron	55

Kybol	31

L

L’Avera	58

Leuna	6, 21, 39, 43, 45, 46, 47, 63

Ludwigshafen	21, 39, 45, 47, 61, 63

Luetzkendorf	21, 63

M

Magdeburg	43

Martigues	58

Middleton	27

Montgomery	22, 26

N

NAFTA-Spezia	56

Naphthenes	8, 9, 10, 11, 13, 19

Naples	55, 56

Neidersachen-Dollbergen	58

nitro-alkyls	33

Nitrous oxide	31

P

Paraffins	8, 10, 11, 19, 46, 47, 48

Pauillac	57

Pechel-Donges	57

Petit Couronne	57

Phenols	9, 11, 19

Pinacoline	31

Ploesti	56

Pölitz	6, 43

PONA Distribution	14, 15

Port Jerome	57

Potential Gum	7, 19

Pressure Distillate Treating Unit	28

Propane Deasphalting Unit	28

R

Red Band (Rotring) Aero Oil	29

Reichs Luftfahrtminsiterium	35

Rhenania Ossag-Grasbrook	58

Rhenania-Harburg	57

Rhenania-Ossag-Menheim	58

ROMSA-Fiumo	56

Roumanian	5, 6, 49

Royal Dutch Shell Petroleum Co.	22

Rumania	20, 50, 57, 58, 59

S

safety fuel	32

Schindler-Neuhof	58

Schindler-Peine	58

Schmitz-Dormund	58

Scholven	6

Scliemann-Grasbrook	58

Shell Aviation Services	4

Shell Development Co.	22

Shell Oil Co.	22

Shell Pernis Refinery	30

SIAP-Trieste	56

Smederevo	55

Spaght	22, 75

STANAVO	4

Stoaua	57

straight run	5, 6, 49

Straight Run Gasoline	28

Strategic Bombing Survey	21

Sweetland Process	28

Synthetic Octanes	5

Synthetic Refineries	6

T

tertiary butyl methyl ketone	31

TEXACO	28

Toluene	8, 11, 13, 19, 42

triptane	31

U

U.O.P. Poly Plant	28

U.S. Strategic Bombing Survey	58

Unsaturates	9, 10, 11, 19

V

Vacuum- Bremen	58

Vacuum-Schulau	58

Venezuelan	5

Voltol	6

W

Welheim	43

Wesseling	21, 43, 45, 47, 63

X

Xylene	5, 13

Xylenes	8, 11, 19

Z

Zeitz	21, 63

ZV1	31

# Research Sources

UK Public Records Office File S 1372/A.1.9.(L) Memo from D A C Dewdren, 27 March 1941

UK Public Records Office UK PRO File POWE 33/1470 Axis fuel supplies: German and Italian aviation spirits sample analysis

Aviation Gasoline Production and Control USAF Historical Studies: No. 65 September 1947 MA/AH Publishing Sunflower University Press Kansas USA ISBN No. 0-89126-146-X

American Petroleum Institute 25th Annual Meeting & Victory Jubilee - Chicago Illinois November 12-15, 1945 Division of Refining “Investigation by the U.S. Government Technical Oil Mission” by W.C. Schroeder

“The United States Strategic Bombing Survey” -Oil Division Report. 2nd. Edition January 1947. Source: American Petroleum Institute Library, Washington DC.

Website http://www.open.ac.uk/ou5/Arts/chemists/person.cfm?SearchID=5485 Accessed Nov 7, 2006

UK Public Records Office FO 1039/556 Reports and Information - Refineries

. BIOS Overall report No. 1 - Report on the Petroleum and Synthetic Oil Industry of Germany, Pages 119-121. Source: Imperial War Museum London UK.

BIOS Final Report No. 967 Deutsche Vacuum Oil AG Fuels and Lubricants, Pages 13-15. Source: Imperial War Museum London UK

US National Aero Space Museum Library – Washington D.C. Ref NASM B4-311028-01. “The Manufacture of Aviation Gasoline in Germany Report” prepared by US Naval Technical Mission in Europe - July 1945. B.I.O.S. Miscellaneous Report No. 72 prepared by M E Spaght (Technician)

US National Aero Space Museum Library – Washington D.C. Ref NASM B4-311028-01. “The Manufacture of Aviation Gasoline in Germany Report” prepared by US Naval Technical Mission in Europe - July 1945. B.I.O.S. Miscellaneous Report No. 72 prepared by M E Spaght (Technician)

US Navy Technical Report No. 145-45 MANUFACTURE OF AVIATION GASOLINE IN GERMANY Website http://www.fischer-tropsch.org/primary_documents/gvt_reports/USNAVY/tech_rpt_145_45/rpt_145_45_toc.htm

“Report On Investigations by Fuels and Lubricants Teams At The I.G. Farbenindustrie, A. G., Works, Ludwigshafen and Oppau” Information Circular I. C. 7375 August 1946, United States Department of the Interior, Bureau of Mines, R. R. Sayers, Director

BIOS Final Report 119 “Interrogation of evacuated personnel at Strass near Neurenberg (Nürenberg)” August 4th and 8th 1945.

“Oil as a Factor in the German War Effort” 1939-1945, Report by the C.O.S. Technical Sub-Committee on Axis Oil, 8th March 1946.

UK Public Records Office CAB 122/1481 Present Position of the Refining Industry in German-Held Europe- December 1943.

Meeting Of Members Of The Technical Oil Mission Washington, D.C., December 13 and 14, 1945. Title "HAMBURG AREA" by Paul K. Kuhne. http://www.fischer-tropsch.org/Bureau_of_Mines/tom_meeting/agenda.htm

Wright Patterson Air Force Base Website. http://www.pr.afrl.af.mil/history_wpafb.html Accessed Jan 11, 2004

“World Speed Record Aircraft” by Ferdinand Kasman F.C.W. -Cologne - June 1990. Leicestershire Library (UK) Call No. 639.1334.