Chapter

19

German Aviation Gasoline 1939-1945

Part 2 Composition

Photo 1. German Messerschmitt Bf-109-G refuelling in Russia 1942.

Table of Contents

Introduction	3

War Cabinet Technical Committee On Axis Oil – Enemy Oils And Fuels Committee	4

German C3 (Green) Type Aviation Gasoline	5

Speculation On German Aviation Fuel Position	6

Appendix I	13

Appendix II	25

Appendix III	41

Appendix IV	50

Appendix V	65

References	91

Appendix – Additional Information	94

Index	96

Research Sources	101

Photo 2. Messerschmitt Me-110 with wing tanks. (RAF Hendon 2005) Petroch Services ©

# Introduction

In June 1944 a report was prepared by the British War Cabinet Technical Committee on Axis Oil – Enemy Oils and Fuels Committee

This report was the work of many dedicated chemists who toiled away at the Billingham Division of Imperial Chemical Industries with the best available apparatus of their day to determine the composition of the German aviation gasoline Grade C3, known as “German C3 (Green) Type Aviation Gasoline”.

It is a remarkable analysis of the various samples captured by the Allies.

Their report is as hopefully reproduced as follows:

Note: There have been some minor changes to the terminology of the report to be consistent with other chapters and current terminology, for example their report refers to “95 weak, 110 rich” which in fact is a reference to the octane ratings and has been replaced by “95 Lean rating and 110 Rich rating” which refers to the C.F.R. Motor Aviation rating test and 3-C Rich Mixture Supercharge rating test methods, respectively. T.E.L. refers to Tetra Ethyl Lead.

Other changes have been obvious typographical errors or spelling. These are listed in italics. Additional pictures had also been added to illustrate various points. The table and appendices numbers have been retained from the original report and checked for total calculations; however, some do not add up correctly in the original reference document, this may be due to additional values not reported or calculation errors. In all cases the original data has been reported uncorrected. Additional table numbers have been included for indexing purpose in this chapter.

Photo 3. German Junker JU-87 Stuka bomber refuelling.

# War Cabinet Technical Committee on Axis Oil – Enemy Oils and Fuels Committee

June 1944

Enemy Oils and Fuels Committee

Lt.-Col. S.J.M. Auld	Petroleum Board (Chairman)

Mr. H.C. Tett          )	Anglo-American Oil Co., Ltd.

Mr. E.W. Hardiman)

Dr. D.A. Howes 	Anglo-American Oil Co., Ltd.

Mr. J.A. Oriel       )	Asiatic Petroleum Co., Ltd.

Mr. C.H. Barton   )

Dr. R. Holroyd    )	Imperial Chemical Industries, Ltd.

Mr. D.H. Peel     )

Professor F.H. Garner	University of Birmingham

Squadron Leader D.A.C. Dewdney	Air Ministry

Flight Lt. R.S. Randolph	Air Ministry (A.I.2.g)

Dr. K.E.W. Ridler      )	Ministry of Aircraft Production (AD/RDE/F)

Mr. E.L. Bass             )

Mr. O.F. Thompson	Foreign Office & Ministry of Economic Warfare

Dr. F.B. Thole	Ministry of Fuel and Power

Mr. A.T. Wilford	Ministry of Supply (D.T.D.)

Mr. D.L. Wright	U.S.A. Embassy (Petroleum Attaché’s Office

Secretaries

Mr. C. Chilvers	Petroleum Board

Mr. E. Herbert	Foreign Office & Ministry of Economic Warfare

Index

Contents										Page

Summary and Conclusions								1

Speculation on German Aviation Fuel Position					1 - 2

Discussion										2 - 7

Appendix I General Inspection and Comments

Appendix II Composition of Aromatic Hydrocarbons in C3 type Aviation Gasoline	i-iv

Appendix III Composition of Synthetic Octanes in C3 type Aviation Gasoline	i-v

Appendix IV Composition of Complete Fuel - Possible Make-Up. Source of Base Fuel	i-xiii

Appendix V Notes on Methods of Analysis - accuracy and reproducibility		i- vii

The work recorded in this report was carried out by Imperial Chemical Industries Ltd., (Billingham Division)

# German C3 (Green) Type Aviation Gasoline

Properties, Composition and Possible Methods of Manufacture of Samples Obtained during the Period summer 1940 to autumn 1943.

Summary and Conclusion

Spread over a period of 3½ years, since the introduction during the Battle of Britain in the late summer of 1940, some 22 authentic samples of the green C3 type of aviation gasoline, had been examined, 14 of these by detailed fractionation and analysis.

It was concluded that these gasolines consisted of a blend of at least three components: -

25-30% of light-cut gasoline (ca. 85 deg. C. end. point)

60-50% of highly aromatic naphtha of low volatility

15-20 % of synthetic octane·

It is further concluded that all three components were produced in conjunction with hydrogenation plants. The aromatic naphtha was highly characteristic and was considered to be the product of high temperature ‘aromatising” destructive hydrogenation of a bituminous material. The octane was considered to be the product of a low temperature butylene polymerisation process which gave large amounts of 2,2,4 trimethyl pentane (it was not alkylate) which was probably operated in conjunction with the hydrogenation plants.

Composition and specification have shown only minor changes from mid-1940 up to early 1943, when the engine performance was increased from 95 Lean rating to 97 Lean rating, 110 Rich rating to 125-130 Rich rating by adding 5% more octane, 5-10% more light petrol and by removing 8-10% of higher boiling naphthenes by some further dehydrogenation treatment.

Some speculation as to the possible German fuel position were followed by a discussion of the main of the main points of interesting arising out of the analytical work, while the actual analytical methods are described and analytical results tabled in Appendices I to V.·

# Speculation On German Aviation Fuel Position

When the whole position of aviation gasoline production in Germany is considered, it is seen that throughout the war there has been a marked preference for gasoline rich in aromatic hydrocarbons. This preference for aromatics is shown firstly by the green C3 type of aviation fuel which contains 35-40% aromatics, and is considered to be based largely on a highly aromatic hydrogenation product and secondly by the blue B4 aviation fuel which for three years contained 10-15% aromatics and is now up to 20% aromatics and is now considered to contain of the same aromatic naphtha made by hydrogenation as is present in the green fuel. Motor gasolines also often contain large amounts of added benzole (up to 25% benzene added to Fischer-Tropsch material) and it is probable that some benzole is also being added to the aviation fuels (possibly in a hydro-refining process).

The high aromatic contents are well known to be particularly useful for rich mixture performance and the green C3 fuel has been found to have a very high rich mixture rating when examined under British standard test conditions. However, such trials as have been carried out with captured German engines on German green fuel, or a match for it, have indicated that German engines are not taking full advantage of the possibilities of green fuel at the rich end. At the weak mixture end, on the other hand, there is a deficiency in fuel performance judged by the U. K. methods of test for 100 Octane Number fuels.

In view of the large potential production capacity of naphthene fuels and synthetic octanes in hydrogenation plants which the Germans have, it is surprising to find an apparently unbalanced fuel of this nature, but the explanation seems to be that official policy at, or before the outbreak of war, was to concentrate on aromatics as high octane blending agents rather than synthetic octanes. The hydrogenation plants are, of course, as capable of making an aromatic blending agent as paraffinic.

It would appear that practically the only hydrogenation products which are going into aviation or motor gasoline are those made by such ‘aromatising’ processes which consume relatively little hydrogen. It is fairly certain in view of the marked shortage of iso-pentane that no plants are being operated in Germany using sensitive-catalyst vapour phase hydrogenation process such as that used in the U.K., which produces a volatile aviation gasoline and much iso-pentane, but which consumes more hydrogen.

The synthetic octane found in German green fuel is characterised by a high proportion of 2,2,4 trimethyl pentane (also known as Iso-octane). It may be that this particular iso-octane is valued for its high weak (lean) mixture rating. It is considered that the octane is being made from butane produced hydrogenation processes, using a low temperature high throughput phosphoric acid polymerisation process, probably operated with partial polymerisation so as to increase the yield of 2,2,4 trimethyl pentane.

At the end of 1942 there was an appreciable change in the composition of the green fuel, the octane content rising by 5% and the high boiling naphthenes content falling by 8 to 10% due to further hydrogenation treatment. These changes had the effect of raising the C.F.R. motor method octane rating by 2 points, and the 3-C rich mixture rating by 15 points. The change at both the weak (lean) and the rich end has been largely due to the removal of the high boiling naphthenes. Both main engine and single cylinder aero engine tests carried out on the green fuel have indicated very clearly that, at least by our (British) standards, the German main engines are not taking advantage of this increased rich mixture performance. The solution to the puzzle of why the Germans have an aviation fuel specification apparently far better than their engines would seem to need, can only come from further work with German engines.

It is difficult to find a reason why, when at the end of 1942, the Germans decided to improve the quality of this type of gasoline that they should introduce an additional aromatising or hydroforming type of hydrogenation process which had the effect of increasing still further the (apparently surplus) rich rating and still further unbalancing the fuel at the weak (lean) end rather than to expand on octane production and increase the (apparently deficient) weak (lean) rating unless there are some unknown factors limiting octane production or which make this course preferable from the German point of view. Meanwhile the following are possible explanations:

- That the policy of using aromatics as the main high performance additive inevitably leads to a surplus of quality at the rich end.

- That our test works on German fuels is not rating them as they are rated by the Germans. In this connection it is understood that the Germans plot B.M.E.P. (Brake Mean Effective Pressure), not against specific consumption, but against a per cent of theoretical air consumption based on the hydrogen carbon ratio of the fuel. One set of data has been plotted in this way and shows that the German green fuel is displaced in a manner relative to British 100 octane grade so that the weak (lean) mixture performance of the German fuel is rated as better that the British.

- That this improved green gasoline was introduced at the beginning of 1943 in anticipation of a new service engine which would make full use of it, but that this engine has been delayed by development troubles.

Discussion

1).	Samples Obtained.

Green (C3) aviation gasoline first appeared in aircraft destroyed during the Battle of Britain in the late summer of 1940. The total number of authentic sampled to date is small, so that it is unwise to draw conclusions as to the extent of use of this type of fuel, although from total actual samples received the indication is that less than a fifth of the total operational planes use this type of material.

Some twenty-one samples of green C3 aviation gasoline have been recorded (Appendix I) and to date fourteen have been analysed in detail by fractionation (Appendix II-IV).

There was a period toward the end of 1942 when so few samples of this type of aviation gasoline were encountered that the suggestion arose that the use of this type of gasoline was being discontinued; a number of recent samples have shown, however that this fuel is still being used.

2).	Use of C3 Aviation Gasoline

Captured documents infer that this type of aviation fuel must be used in at least two types of motors; Daimler Benz DB 601 N, and the BMW 801 C & D. There are cases of its use in Junkers machines, although practically all recent samples are from (Focke-Wolf) FW 190 fighters.

Photo 4. Daimler Benz DB-605B as used in the famous Messerschmitt Bf-109. (RAF Museum Hendon 2005)

Photo 5. Focke Wolf FW 190 refuelling from a fuel truck.

The indication was that this fuel (initially of 94-95 C.F.R. motor rating) was a special fighter grade fuel as distinct from the bomber and general purpose fuel – the B4 blue-coloured aviation gasoline of 90 C.F.R. motor octane number. Some fairly recent Me 109-F fighters with DB 601 E engines have been operating on the B4 gasoline and the contention that present aero engines used by the G.A.F. (German Air Force) are not being operated to take advantage of the rich mixture potentialities of the C3 type of gasoline render the reasons for the introduction and use of this fuel somewhat puzzling.

A number of samples of this fuel have been obtained from the Middle East and some from the Russian front (Table 1 Appendix I).

3). General description of Green C3 Type Aviation Gasoline – Specification (see also Appendix I)

The lead content of the green C3 gasoline is the same as that of the lower grade B4 blue gasoline

The average properties have been:

Table 1. Properties of German grades C3 and B4.

| Property | C3 |  | B4 |
| --- | --- | --- | --- |
| Colour | Fluorescent | Yellow-green | Blue |
| Octane Rating | (a) | (b) |  |
| C.F.R. Motor | 94-95 | 96-97 | 90-91 |
| 3-C Rich Mixture rating | 110 | 125-130 | - |
| Base Spirit, lead-free C.F.R. Motor | 83 | 83 | 72-74 |
| Base Spirit, lead & aromatic free C.F.R. Motor | 77 | 78 | 67-70 |
| Tetra Ethyl Lead mls./U.S. Gal | 4.6 | 4.6 | 4.6 |
| Density 15/15C | 0.782 | 0.770 | 0.773-0.748 |
| Volatility % to 75 deg. C | 7 | 15 | 20 |
| Volatility % to 100 deg. C | 40 | 50 | 55-60 |
| Volatility % to 140 deg. C | 77 | 85 | 95 |
| End point deg. C | 185-175 | 175 | 160 |
| End point deg. F | 365-345 | 345 | 320 |
| Butanes + pentanes % vol. | 6 | 10 | 8-13 |
| Reid Vapour Pressure lbs./sq.in. | 3-4 | 5 | 5-6 |
| Aromatics % vol. | 37 | 37 | 9-20 |

The C3 type of gasoline is readily recognised by such tests.

Early in 1943 changes occurred in the C3 gasoline towards a more volatile fuel of increased performance as shown at (a) and (b) in the above table. This change is discussed more fully below.

4).	Hydrocarbon composition – Interpretation

The C3 type of gasoline is of low volatility for an aviation fuel and contains nearly 40% of aromatic hydrocarbons. It has also been shown to contain 15-20% of synthetic octanes. About 14 samples have been examined in detail by super-fractionation and also in several cases by spectroscopic analysis.

- The composition and distribution of the aromatic hydrocarbons are highly characteristic, as also are the amount and distribution of naphthene hydrocarbons.

- The interpretation of such analysis involves comparison with as wide a field as possible products from known sources. It has always been emphasised that conclusions based on such comparisons are much stronger when based on wide dissimilarities in composition. Thus, it is possible to conclude positively that an unknown hydrocarbon mixture has not been obtained from a certain source, but where composition is similar to that of a known product it can only be concluded that the source “could be” the one in question.

3.	The composition of C3 type fuels obtained by the above methods have been compared with a fairly wide field of products of known origin, including:

I	Alkylates, hydrocodimers (hot and cold acid, of various types)

II	Aromatics from coal carbonisation; hydro-reformed petroleum and creosote-hydrogenation naphthas; products from selective dehydrogenation (hydrofining) of petroleum and various hydrogenation naphthas from coal or petroleum; highly aromatic naphthas made by high temperature “aromatisation”; destructive hydrogenation of bituminous materials.

III	About 20 virgin petroleum gasolines and naphthas from a variety of sources; a variety of high pressure sensitive-catalyst “vapour phase” hydrogenation gasolines from bituminous coal creosote and also from petroleum gas oils; certain Fischer-Tropsch gasolines; a single sample of catalytically cracked gasoline.

By comparison of the highly characteristic hydrocarbon composition of the C3 type gasoline with such a necessarily limited field of products of known origin it is concluded that the C3 type of aviation gasoline is a blend of at least three components.

A possible composition which would agree with the analytical evidence is:

25-30% of light cut gasoline (providing pentanes, hexanes and C6 naphthenes) and probably of boiling range up to about 85 deg. C.

60-50% of a highly aromatic naphtha of low volatility and probably boiling over the range of 85-175 deg. C. The aromatic content of this naphtha may be as high as 70%.

15-20% of synthetic octane of which the major component is 2,2,4 trimethyl pentane with 2,3,4 trimethyl pentane as the second main component.

Recently, certain changes have occurred in composition which are discussed below, but it is still considered that the C3 gasoline is made up from the above three components with the possible addition of hydro-refined benzene (or coke-oven aromatics mainly benzene), and possibly some added pentane.

5)	Source and Method of Manufacture of Above Components

I.	Light Gasoline cut.	It is considered that this could be produced in the hydrogenation plants (e.g. by high temperature operation or with less active catalysts than those which we associate with “vapour phase” destructive hydrogenation), although the composition is nearer to that usually associated with straight petroleum products. The conclusion that a light gasoline fraction has been added as a separated component of the final blend is based on the amounts of naphthenes boiling in the range 85 deg. C, which are very much less than in normal hydrogenation products, whereas the C3 gasoline contains the normal amount of cyclohexane found in virgin petroleum gasolines. The comparative absence of any material distilling at 90 deg. C suggests that the end point of the light cut is below 90 deg. C.

II	Highly Aromatic Naphtha	In view of the highly characteristic composition (ethyl benzene, n-propyl benzene, and n-butyl benzene) which is discussed in detail in Appendix II, it is considered that this material is derived by high temperature hydrogenation treatment of bituminous raw material containing condensed aromatics such as naphthalene.

It is fairly certain that this material is produced in those high temperature high pressure hydrogenation plants such as Welheim.

Recent C3 gasoline samples of improved rating have shown a change in aromatic composition in conjunction with a marked decrease in the amount of higher boiling naphthenes. It is concluded that the highly aromatic naphtha made by the above process is now being given a further treatment whereby naphthenes are converted into aromatic (possibly hydroforming or hydrofining). This further treatment is considered to be largely responsible for the rise in engine performance. It may be that hydroforming the aromatic naphtha would produce the extra low boiling paraffins found in such recent samples of C3 gasolines.

An alternative possibility is that the process for production of the previous type of highly aromatic naphtha (hydrogenation a high temperature and possibly high pressure in possibly relatively few plants) is now not adequate to meet present fighter requirements of aromatics (possibly not now available), and an alternative means of providing a highly aromatic naphtha (to maintain this type of composition) is to hydroform a suitable naphtha (possibly a hydro-naphtha of another type).

It is known that hydroformed products from treatment of petroleum naphthas will yield products containing up to 70% aromatic with less than 10% naphthenes. A more naphthenic or less paraffinic type of material would be available for treatment from the hydrogenation processes.  It is more in keeping with the known recent increase in engine performance to conclude that the previous type of aromatic naphtha is now being given a further dehydrogenation treatment rather than to conclude that the change in aromatic composition is due to addition of another aromatic naphtha made by dehydrogenation of a hydro-naphtha. It is possible that coal tar aromatics are now being added but there is no evidence of increase in sulphur content.

III 	Synthetic Octane (see also Appendix III)

The main components are 2,2,4 trimethyl pentane and 2,3,4 trimethyl pentane in the ratio of about 3:1. The absence of more than traces of dimethyl hexanes and of 2,3,3 trimethyl pentane and the absence of 2,2,5 trimethyl pentane are considered sufficient evidence to exclude C4 alkylates of the types examined. The presence of 2,3,4 trimethyl pentane and such tracer compounds as 2,3 dimethyl pentane is considered evidence for the presence of hydrogenated co-polymer.

The relatively high proportion of 2,2,4 trimethyl pentane differentiates this octane from that normally produced by the hot phosphoric acid polymerisation of isobutylene and is closer to octane produced by the cold sulphuric acid process.

It is probable that the synthetic octane is produced by dehydrogenation of C4 gases produced in the hydrogenation plants with a subsequent polymerisation process operated at a low temperature.

It has been suggested that the composition could be met by postulating a mixture of cold acid octane with some other type of octane. It is still considered that the evidence excludes alkylates and the remarkably constant composition maintained over three years production suggests that only one type of process is involved.

Speculation as to the method of production of the octane is bound up with the suggested scheme for production of the rest of the blend, and the suggested methods of operation of the hydrogenation plants and a general survey of the availability of C4 and C5 hydrocarbon for aviation fuel purposes. High temperature ‘aromatising’ hydrogenation processes would tend to make much hydrocarbon gas and yet viewing the whole German aviation gasoline position (blue and green types) there appears to be a marked shortage of iso-pentane. This is in agreement with the view that relatively little of the total aviation gasoline is produced by the medium temperature sensitive-catalyst vapour phase process as used in the U.K. which produces large amounts of iso-pentane.

The earlier samples of green gasoline were estimated to contain 15% of synthetic octane. Early in 1943 coincident with the rise in engine rating there was an increase in octane content to about 20%. Composition of the octane added has remained unchanged.

6) 	Changes in Specification and Composition

Definite changed in composition have occurred as follows: (see also Table 2. below)

During 1940 	increase in aromatic content from 33% to 37% vol.

End of 1940 	aromatic naphtha has cut back to a lower end point but aromatic content maintained, octane content raised from 13% to 15%

End of 1942 or early 1943	increase in octane content up to 18% also increase in content of light petrol and pentane, general rise in volatility and fall in aromatic content (by dilution). Marked increase in C.F.R. Motor Rating to 97 and in 3-C Rich Rating to 120.

Summer 1943	still further increase in rating with octane content up to 20% and aromatics increased back to 37, and decrease of 9% in content of higher boiling naphthenes.

The average analysis of green C3 fuel can be taken as some indication of specification.

Table 2. Changes in Specification and Composition

| Property | 1940 | 1941 | 1942 | 1943 |
| --- | --- | --- | --- | --- |
| Density 15C/15C | 0.782 | 0.782 | 0.782 | 0.772 |
| Final Boiling Point deg. C | 185 | 175 | 175 | 175 |
| Volatility at 75 deg. C Vol. %. | 7 | 7 | 7 | 15 |
| Volatility at 100 deg. C Vol. %. | 41 | 41 | 41 | 51 |
| Volatility at 140 deg. C Vol. %. | 77 | 85 | 85 | 86 |
| Vapour Pressure lbs. | 2-4 | 2-4 | 2-4 | 5 |
| Wt.% C4+C5 | 5½ | 5½ | 5½ | 8½ |
| Vol.% Aromatics | 34-35 | 37 | 37 | 34-> 37 |
| % Aromatics >166 deg. C | 30 | 10 | 10 | 10 |
| Vol.% Octanes | 13 | 15-16 | 15-16 | 18-> 20 |
| Vol.% Naphthenes boiling above 95 deg. C | 16 | 16 | 16 | 13-> 7 |
| T.E.L. content mls/Imp Gal. | 5.5 | 5.5 | 5.5 | 5.5 |
| T.E.L. content mls/U.S. Gal | 4.6 | 4.6 | 4.6 | 4.6 |
| T.E.L. content mls/litre | 1.2 | 1.2 | 1.2 | 1.2 |
| Knock Rating |  |  |  |  |
| Weak (Lean) C.F.R. Motor | 94 | 94 | 95 | 97 |
| Rich 3-C % Grade 130 |  | 110 |  | 120-> 125 |

The significance of this recent change is great. It may mean that the Germans have introduced an additional process in order to reduce the amount of high boiling naphthenes in order to gain considerable improvement in rich performance, which as yet they have shown no evidence of being able to use in the existing aero engines. Alternatively, the improved rating may be incidental and the change in aromatic composition may mean that the Germans have been forced to change to another process (i.e. by bombing of plants working the previous process). The latter explanation would seem unlikely, although it may be that hydroforming (or other dehydrogenation process) applied to a naphtha already essentially boiling within the range of the finished product (and hence not requiring ‘splitting’) coupled with a normal hydrogenation-splitting process to produce the naphtha, would be a more flexible tie-up applicable to more plants than the alternative process of high temperature high pressure ‘aromatising-splitting” postulated for the earlier types of aromatic naphtha production. In the latter the ‘aromatising’ and ‘splitting’ are done together in one stage and require high temperatures to give an aromatic product and high pressures to maintain catalyst life, while in the other method two separate stages only require medium or comparatively low pressures.

An alternative view is that this recent change in composition is evidence of an effort to overcome a limitation in weak mixture performance.

The remarkably close similarity in hydrocarbon composition of samples of C3 type of aviation gasoline obtained from such widely scattered sources as Britain (shot down), Russian front and Middle East (Tripoli and Tunis) suggest that this fuel is a synthetic blend made in comparatively few plants, all working under very similar condition, and all probably using the same type of raw materials.

Other Anti-knock Blending Additives

Lead content had remained constant at 5.5 cc/Imp. Gal. = 4.6 cc/U.S. Gal. = 1.2 cc/Litre. There has been no evidence in any sample for the presence of Cumene or tertiary Butyl Benzene, Aromatic amines, Aniline, Methyl Aniline or Xylidene, Iron Carbonyl, Alcohols, ethers or any water soluble component.

# Appendix I

GENERAL INSPECTION

- Colour	The characteristic fluorescent yellow green colour has been maintained throughout (probably fluorescent dye)

- Density	The average density used to be 0.782 up to about the summer of 1942 (samples of higher density were probably weathered e.g. AIR118) [refers to British sample designation]. [Note ‘weathered’ refers to the loss of lighter hydrocarbon components through exposure to air and evaporation]. All samples obtained during 1943 are of lower density averaging 0.772 (except weathered samples AIR289, and 312 jettisoned from fighters). It is possible that this change occurred as early as the autumn of 1942 (AIR195, 240).

- I.P.T. Boiling Range	A change occurred at the end of 1940 when the F.B.P. (Final Boiling Point) fell from 185 deg. C and 95% from 178 deg. C to 165 deg. C. This change at the end of 1940 had little effect on the volatility to 75 deg. C or 100 deg. C or on the vapour pressure. It is reflected markedly in the volatility to 140 deg. C and in the percentage of material distilling above 165 deg. C in the T.B.P. column (line 5). [True Boiling Point].

- Volatility two changes occurred:

- At the end of 1940 the end point was reduced and volatility at 140 deg. C raised, but volatilities at 75 deg. C and 100 deg. C remained at about 7% and 41% respectively.

- At the end of 1942 the ‘front end’ volatility was raised (as described below by adding more octane and more light petrol), pentane content was increased as was Reid Vapour Pressure, and volatilities to 75 deg. C and 100 deg. C were raised to 15% and 50-52% respectively. This caused little or no change in the volatility to 140 deg. C to the T.B.P. % above 165 deg. C.

- T.B.P. Vol. % above 165 deg. C	Up to the end of 1940 this was 17-19% in keeping with the high F.B.P. of 185 deg. C. Reduction of the F.B.P. to 175 deg. C at the end of 1940 brings the T.B.P.% distilling above 165 deg. C down to 7-8%. The latter increase in ‘front end’ volatility at the end of 1942 has not affected this appreciably.

- Butane content.	All the green fuels examined have 1-2% wt. of butane, in all cases this has been practically entirely normal butane.

- Pentane content.	All fuels up to the end of 1942 contained only 3½ -4% wt. of pentane. In 1943 this had been increased to about 6% wt. while such recent samples as AIR317, 318 and 328 contain 8% of pentane and practically no butane. It should be noted that although more iso-pentane than normal pentane is present these fuels all contain much less iso-pentane and a much lower iso/N-pentane ratio than would be obtained by normal vapour phase hydrogenation treatment. Green fuel AIR317 is of interest since more iso-pentane is present, but this higher ration is not maintained in the most recent samples.

Table 3. Pentanes in German Avgas

| Sample | Iso-pentane (wt.%) | N-pentane (wt.%) | % iso-pentane in total pentane |
| --- | --- | --- | --- |
| Green fuel 1940 | 2.5 | 1.8 | 58 |
| Green fuel 1941 | 2.3 | 1.7 | 58 |
| Green fuel 1942 | 2.4 | 2.2 | 50 |
| Green fuel 1943 | 3.7 | 2.5 | 60 |
| Green fuel AIR317 | 5.6 | 2.9 | 65 |
| Green fuel AIR318-330 (avg.) | 4.5 | 3.1 | 60 |
| V.P. Hydrogenation fuel (creosote) | 17 | 3 | 85 |

(8)	Vapour Pressure	All samples have had a low vapour pressure of 2½ -5 lbs. the extra pentane added has only brought the vapour pressure up to 5 lbs.

(9)	Bromine Number	With the exception of samples obtained during 1941 (AIR141, 144 and 176) the bromine number has been less than 1, showing that the unsaturated (hydrocarbons) are practically absent. (Vapour Phase Hydro petrols have bromine numbers less than 0.2) [Unsaturated hydrocarbons can readily polymerise and form gum deposits which affect storage stability and therefore undesirable].

(10)	Aniline Point	Initial aniline point (I) is largely indicative of the aromatic content. The 1940 fuel had an aniline point of about 19 deg. C, which fell to 16 deg. C. In 1941 when the aromatic content was increased from 34-35% to 37-38% Vol. The aniline point has risen again in 1942 fuels (AIR285, 305 and 311) to 23-34 deg. C due to the addition of more octane and light petrol and the falling aromatic content. The latest sample AIR317 etc. have a lower aniline point of about 20 deg. C., since the aromatic content has again been increased.

(11)	Aromatic-free Aniline Point.	This figure in the green fuels is a guide to octane content. The aromatic-free aniline point has remained fairly constant at about 57.5 deg. C up to the end of 1942. Figures of over 60 deg. C (62 and 63 deg. C) now recorded are due to increased octane content and increase in pentane and light petrol fractions.

(12) & (25)	Aromatic Content	As determined directly on the uncut fuel (12) and by summation of analyses of fractions (25) shown that for 1941 there was an increase in aromatic content from 34% to 37% vol. which was maintained through 1941 and 1942 until it was reduced to 33% vol. in 1943 due to the dilution effect of adding more octane and light petrol. It would now appear (AIR317) that the aromatic content had again been restored to 37% while keeping the higher content of octane and light petrol, largely at the expense of the higher boiling naphthenes.

(13) & (14)	Naphthenes & Paraffins.	These figures determined without fractionation are not very reliable. The serve to show that right up to the end of 1942 the non-aromatic portion of the fuel remained fairly constant with about 30% of naphthenes and 30% of paraffins and with an aniline point of 57.5 deg. C (11) and density 0.732 (15), but in 1943 the extra octane and light paraffinic petrol cut added have caused a marked change to 15-20% naphthenes and over 40% paraffins with the aniline point up to 62-63 deg. C and density down to 0.725 (AIR285, 305) and later to 0.715 (AIR311, 317, etc.).

(17)	Lead Content	Apart from fuels which have weathered and thereby increased on lead (AIR289, 312) the lead content has remained constant at 5.5 mls/Imp. Gal. (=4.6 mls/U.S. Gal., or probably 1.2 mls T.E.L./Litre of fuel. Even in the recent fuel of much increased ratings there has been no increase in lead content.

(16) (18) (19) 	Knock Ratings.	The C.F.R. motor octane number up to the end of 1941 remained at 94. Samples obtained in 1942 may have been higher at 95-96, while 1943 samples show figures up to 97. The C.F.R. motor rating of the fuel freed from lead shows no similar changes. An average figure of 82-83 is due to the high aromatic content and the presence of octanes. The C.F.R. motor ratings of the fuel freed from both lead and aromatics are again variable. Up to the end of 1942 the ‘base fuel’ was probably 76.5-77, whereas 1943 fuel is 78 or over. In both cases this octane number is high due to the presence of octane. Assuming 16% of octane in the earlier fuels and 20% in 1943 fuels the lead, aromatics and octane –free base fuel probably has an octane number of about 70-71. Although this residual paraffin/naphthene mixture is fairly rich in normal paraffins it would be expected to have such a rating since the greater part consists of low-boiling components, (more than 75% boiling below 100 deg. C).

Some 3-C rich mixture ratings are given below. Recent 1943 type green fuels have risen to over 125% grade 130. This increase could be accounted for in terms of the changes in composition found, since the appreciable reduction in the amount of higher boiling naphthenes which has occurred in the most recent samples would be expected to cause an appreciable increase in the rich mixture rating while this change and the addition of more octane and light petrol are considered to account for the increase of two units in the weak mixture rating.

Table 4. Results of samples from captured aircraft

| Property | 1940-1942 Green Fuel | AIR285 | AIR305 | AIR311 | AIR317 | AIR318-327 | AIR328-330 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Knock Rating C.F.R. Motor as received | 94 | 96 | 96.5 | 97 | 97 | 97 | 97 |
| T.E.L. mls/Imp. Gal. | 5.5 | 5.6 | 5.5 | 5.5 | 5.6 | 5.65 | 5.65 |
| Research O.N. mls TEL/US Gal. on octane | - | 0.34 | 0.35 | 0.32 | 0.51 | 0.48 | 0.56 |
| 3-C Rich Mixture rating % grade 130 | 110 | 118 | - | 124 | 125 | >125 | >125 |
| Composition (Billingham Analysis) |  |  |  |  |  |  |  |
| Aromatics % by volume | 37 | 33 | 32.5 | 34 | 36.5 | 37.5 | 37.5 |
| Butanes + Pentanes % vol. | 7 | 9.5 | 10 | 10 | 10.5 | 10.5 | 12 |
| 2,2,4 Trimethyl Pentane % | 11 | 13 | 13 | 13 | 13.5 | 14 | 13.5 |
| 2,3,4 Trimethyl Pentane % | 5 | 5 | 7 | 7 | 7 | 6.5 | 7 |
| Naphthenes boiling above 95 deg. C % vol. | 16 | 13 | 11 | 7.5 | 6.5 | 6.5 | 6.5 |

(20)- (25) 	Distribution of Aromatics (Billingham fractionation)

The aromatic distribution is determined quite reliably by the fractionation method. This distribution expressed as a volume percentage of the total fuel is detailed in the large table while below figures are given expressed as a percentage of the total aromatics and as a percentage of the C6+C7+C8 aromatic boiling below 145 deg. C.

It will be seen from both sets of figures that changes have been as follows:

1940 fuel (GF 28 & 40/41)	34% aromatics	30% of which boil above 166 deg. C

1940 fuel (GF 31 & GF 32)	37% aromatics	30% of which boil above 166 deg. C

1941 fuel	37% aromatics	only 10% of which boil above 166 deg. C

1942 fuel	37% aromatics	only 10% of which boil above 166 deg. C

1943 fuel	33.5% aromatics	only 10% of which boil above 166 deg. C

Later 1943	37% aromatics	only 10% of which boil above 166 deg. C

One major change occurred at the end of 1940 when two thirds of the high boiling (>166 deg. C) aromatics were removed by cutting the fuel at a lower end-point. At the same time the aromatic content was maintained. Considering only the distribution of C6, C7 & C8 aromatics the table below shows that this has remained surprisingly constant up to sample AIR305, but from sample AIR311 onwards the samples have contained proportionately less benzene and more toluene and C8 aromatics. Even in these fuels, however, the ratio of C7/C8 aromatics is much the same as previous fuels.

Table 5. Distribution of Aromatic Hydrocarbons

| Fuel No. | Total Vol.% | % of total aromatics |  |  |  |  |  | % of aromatics up to 145OC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0-90OC | 90-118 OC | 118-145 OC | 145-166 OC | >166 OC | 0-90OC |  | 90-118 OC | 118-145 OC |
| GF 28 | 33.35 | 18.5 | 23.5 | 20 | 9.5 | 28.5 | 30 |  | 38 | 32 |
| GF 40/41 | 32.6 | 19.5 | 23 | 19.5 | 8 | 30 | 31.5 |  | 37.5 | 31 |
| GF 31 | 37.35 | 17 | 23 | 20 | 11.5 | 28.5 | 28.5 |  | 39 | 33.5 |
| GF 32 | 35.35 | 18.5 | 22 | 19.5 | 10 | 30 | 30.5 |  | 37 | 32.5 |
| AIR141 | 37.7 | 21.5 | 32 | 24.5 | 11 | 11 | 27.5 |  | 41 | 31.5 |
| AIR144 | 37.5 | 26.5 | 31 | 23 | 8.5 | 11 | 33.5 |  | 38 | 28.5 |
| AIR176 | 37.4 | 23 | 33 | 26 | 11 | 7 | 28 |  | 40 | 32 |
| AIR189 | 38.6 | 23 | 32 | 24 | 11 | 10 | 29 |  | 41 | 30 |
| AIR285 | 32.9 | 23 | 30.5 | 26 | 8 | 12.5 | 28.5 |  | 38.5 | 33 |
| AIR305 | 32.45 | 22.5 | 32 | 24.5 | 12 | 9 | 28.5 |  | 40.5 | 31 |
| AIR311 | 33.95 | 17 | 32.5 | 27 | 11.5 | 11 | 22 |  | 42 | 36 |
| AIR317 | 36.25 | 18 | 34.5 | 27.5 | 9.5 | 10.5 | 22.5 |  | 43 | 34.5 |
| AIR318 | 36.55 | 19.5 | 33.5 | 28.5 | 10 | 8.5 | 24 |  | 41 | 35 |
| AIR328 | 35.75 | 18.5 | 33 | 29.5 | 9 | 10 | 23 |  | 40.5 | 36.5 |

Further information obtained by ultra violet spectroscopic analysis shows that recent changes in distribution are accompanied by marked changes in composition of individual aromatics (see Appendix II).

It should be noted that these fuels average 8% and, in some cases, up to 9.7% wt. of benzene (AIR189) and still shows no signs of freezing out at -60OC. (Benzene has a freezing point of +5 OC and high levels of Benzene can result in the fuel freezing at temperatures above -60OC, the specification minimum of -60OC. Hence the Allies had a specification of max. 5% benzene).

(31) & (32) 	Aromatic Composition (Spectroscopic Results)

The important figures here are the high content (in relation to Xylenes) of ethyl benzene which averages 6% for samples prior to 1943 (and thus represents about 66% of the total C8 aromatics), and hydrindene (C9H10 CAS No. 496-11-7 Benzocyclopentane) which generally amounts to 1.5-2% of the fuel.

There are indications that the more recent fuels of 1943 contain less ethyl benzene and more ortho xylene, as well as less propyl and butyl benzene and more ethyl toluenes (Appendix II).

(26) & (27) 	Methyl Cyclopentane & Cyclohexane

The average contents compared with other fuels are:

Table 6. Composition of other fuels

| Component | Green Fuel | Straight Run Petroleum (Roumanian etc.) | Vapour Phase Creosote Hydrogenation |
| --- | --- | --- | --- |
| Methyl cyclopentane | 5.5% | 5% | 12% |
| Cyclohexane | 4.5% | 4.5% | 2% |

There is no indication of any marked change in the more recent fuels, although the ratio is slightly more in the direction of hydro-petrol.

(28)-(30) & (33)-(35) 	Octanes

The Billingham fractionation method shows the presence of octanes, but only gives an approximate measure of the ratio of 2,2,4 higher boiling trimethyl pentanes and the amounts present. These rough estimates show that the 1940 fuel contained about 13% octanes which increased to 16% for 1941-1942 and has later been increased again to 20% in 1943.

The boiling range of the paraffins has indicated 73-75% 2,2,4 trimethyl pentane in most cases, (the sole exception has been fuel AIR189 in which 2,2,4 TMP was down to 50%). The recent 1943 fuel has shown a somewhat higher proportion of the higher boiling trimethyl pentanes with about 65% 2,2,4 trimethyl pentane. Spectroscopic examination has generally confirmed the figure of 2,2,4 TMP and has shown that 2,3,4 TMP is the main component of the higher boiling trimethyl pentanes. The spectroscopic analyses in uncut or roughly cut samples are somewhat uncertain. Detailed analyses of more finely cut material are discussed in Appendix III.

Presence of other anti-knock additives

No evidence had been found for the presence of any of the following anti-knock agents in any sample of the green aviation gasoline examined:

Cumene or tertiary butyl benzene

Aromatic amines or xylidenes

Iron Carbonyl

Alcohols, ethers or any water-soluble components, other than small traces of phenols.

Photo 6. Junkers Ju-88 crash under guard in Kent during the Battle of Britain September 1940.

Appendix I 	Table 1. (Part 1 1940-1942)

Summary of Relevant Analyses of all Authentic Samples of German Green Aviation Fuel

Table 7. Analysis of German C3 Aviation Gasoline (1940-1942)

| Ref. | Date of crash or capture of sample | 1940 |  |  |  | 1941 |  |  | 1942 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Summer | 27th Sept. | Autumn |  | 8th May | 9th June | 10th July | 17th April | 23rd June | Spring | Winter |
|  | Sample No. | GF 20 | 46/41 | GF 31 | GF 32 | AIR118 | AIR141 | AIR144 | AIR176 | AIR189 | AIR195 | AIR240 |
|  | Type of plane | JU-88 | Me-110 | Me-110 | Me-110 | Me-109 Weathered | Me-109E7 | Me-109F | Middle East | FW-190 | C3 ex Russia via MEW | Egypt TD 533 |
|  | Engine |  |  |  |  |  | DB 601 | DB 601 |  | BMW 801D |  |  |
| 1 | Colour |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Density 15/15 | 0.782 | 0.779 | 0.785 | 0.781 | 0.750 | 0.785 | 0.784 | 0.788 | 0.784 | 0.778 | 0.777 |
| 3 | IPT Distillation 95% deg. C Final Boiling Point (FBP) deg. C | 178 183 | - | 177 184 | 177 182 | 183 193 | 165 176 | 163 173 | 168 181 | 155 170 | - 175 | - |
| 4 | IPT % distilled +loss at 70 deg. C IPT % distilled +loss at 75 deg. C IPT % distilled +loss at 100 deg. C IPT % distilled +loss at 140 deg. C | 5 8 38 77 | - | 4 7.5 40.5 76 | 7.5 11 42 77 | - 4 35 74 | 6 5 36 82 | 3 6.5 43 85 | 3 4 38 81 | 5.5 8.5 44 87.5 | 33 - | 44 - |
| 5 | TBP Vol% boiling above 165 deg. C | 19 | 16 | 17.5 | 17 | - | 8.5 | 7.5 | 8 | 7 | - | - |
| 6 | Wt%. Isobutane Wt%. n- Butane | 0 0.85 | 0 1.6 | 0 0.9 | 0 1.4 |  | 0 1.75 | 0 1.65 | 0.2 0.55 | 0 1.2 |  |  |
| 7 | Wt%. Iso-Pentane Wt%. n-Pentane Wt%. Total C4+C5 | 2.4 1.85 5.1 | 4.0 2.3 7.9 | 2.2 1.25 4.35 | 2.85 1.85 6.1 |  | 2.05 1.95 5.7 | 2.4 1.4 5.45 | 1.85 2.6 5.2 | 2.85 1.9 5.95 |  |  |
| 8 | Reid Vapour Pressure lbs. sq.ins. |  |  |  |  | 2.1 |  |  |  | 4.7 |  | 4.5 |
| 9 | Bromine number | 0.9 |  | 0.8 | 0.8 |  | 3.5 | 2 | 1.7 | 1.2 | 1.7 | 1.0 |
| 10 | Aniline point I deg. C | 20.3 |  | 17.7 | 19.5 |  | 16.7 | 15.4 | 16.4 | 16.2 |  |  |
| 11 | Aniline point IV deg. C | 57.3 |  | 57.7 | 57.8 | Wt.(S) | 57.8 | 57.4 | 58.1 | 57.0 | Wt.(V) | Wt.(V) |
| 12a | Aromatics %wt. | (37.3) |  | (40.2) | (38.8) | 40 | 41.3 | 42.4 | 41.6 | 41.6 | 42.5 | 35.5 |
| 12b | Aromatics %Vol. (I.C.I.). | 33.6 |  | 36.3 | 34.9 |  | 37.3 | 38.2 | 37.7 | 37.5 |  |  |
| 13 | Naphthenes %Vol (I.C.I.). | 35.5 |  | 32.6 | 32.8 | 32.5 | 32.3 | 31.2 | 30.9 | 33.5 |  |  |
| 14 | Paraffins %Vol (I.C.I.). | 30.1 |  | 31.1 | 32.3 | 27.5 | 30.4 | 30.6 | 31.4 | 29.2 |  |  |
| 15 | Aromatic free density 15/15 |  |  | 0.732 | 0.731 |  | 0.732 | 0.732 | 0.737 | 0.732 |  |  |
|  | Knock Rating |  |  |  |  |  |  |  |  |  |  |  |
| 16 | C.F.R. Motor (as received) | 94.0 |  | 94.2 | 94.2 | 95 | 93.0 | 94.4 | 95 | 95.0 | 96 | 89 |
| 17 | T.E.L. Content mls/Imp. Gal | 5.7 |  | 5.45 | 5.4 | 5.7 | 5.5 | 5.3 | 5.3 | 5.1 | 5.55 | 4.6 |
| 18 | C.F.R. Motor, lead free | 81.8 |  | 83.6 | 83.6 | 81 | 82 | 85.1 |  |  |  |  |
| 19 | C.F.R. Motor, lead free & aromatic free |  |  | 76.7 | 77.2 | 74 | 73 | 77.7 | 74.5 | 76.2 |  |  |
|  | I.C.I. Distillation (Billingham) |  |  |  |  | Wt.%(S) |  |  |  |  |  |  |
| 20 | Benzene % Vol. (0-90OC) | 6.2 | 6.35 | 6.35 | 6.55 | 7 | 8.5 | 10.1 | 8.5 | 8.8 |  |  |
| 21 | Toluene % Vol. (0-118OC) | 7.8 | 7.55 | 8.6 | 7.9 | 9.5 | 12.2 | 11.5 | 12.5 | 12.3 | Not available | Not available |
| 22 | C8 Aromatics % Vol. (118-145OC) | 6.65 | 6.3 | 7.55 | 6.95 | 7.5 | 9.25 | 8.55 | 9.8 | 9.2 |  |  |
| 23 | C9 Aromatics % Vol. (145-166OC) | 3.2 | 2.3 | 4.35 | 3.5 |  | 4.15 | 3.2 | 4.05 | 4.4 |  |  |
| 24 | C10+ Aromatics % Vol. (>166OC) | 9.55 | 9.9 | 10.5 | 10.6 |  | 4.05 | 4.2 | 2.6 | 3.9 |  |  |
| 25 | Total Aromatics | 33.35 | 32.6 | 37.35 | 35.22 |  | 37.7 | 37.5 | 37.4 | 38.6 |  |  |
| 26 | Methyl cyclopentane | 5.1 | 5.95 | 5.0 | 5.5 |  | 3.5 | 5.45 | 3.9 | 4.1 |  |  |
| 27 | Cyclohexane | 5.3 | 4.05 | 4.0 | 4.5 |  | 5.35 | 4.55 | 5.3 | 5.35 |  |  |
| 28 | 2,2,4 Trimethyl Pentane (est.) | 9 | 9 | 10.5 | 10 |  | 12.5 | 12 | 11 | 7 |  |  |
| 29 | Higher Trimethyl Pentane (est.) | 3 | 3 | 3.5 | 3.5 |  | 4.5 | 5 | 4 | 8 |  |  |
| 30 | Total Trimethyl Pentane | 12 | 12 | 14 | 13.5 |  | 17 | 17 | 15 | 15 |  |  |
|  | Spectro-on fractions (Oxford & Cam.) |  |  |  |  |  |  |  |  |  |  |  |
| 31 | Ethyl Benzene |  |  |  |  |  | 6.1 | 5.6 | 6.0 | 5.6 |  |  |
| 32 | Hydrindenes |  |  | 1.1 |  |  | 1.4 | 2.1 | 1.4 | 1.44 |  |  |
| 33 | 2,2,4 Trimethyl Pentane (+2,2 Dimethyl Hexane) |  |  |  |  |  | 11 | 12 | 11-12 | 8 |  |  |
| 34 | Higher Trimethyl Pentane (2,3,4 TMP etc.) |  |  |  |  |  | 5 | 5 | 3.7 | 8 |  |  |
| 35 | Total Trimethyl Pentane |  |  |  |  |  | 16 | 17 | 15 | 16 |  |  |
|  | I.C.I. Report Number | G40/1 | G40/2 | G40/2 | G40/2 | - | G41/25 | G41/29 | G41/57 | G42/56 |  |  |

S= Analysis by Shell, V= Analysis by Vauxhall Lab. Other samples generally by I.C.I.

Appendix I 	Table 1. (Part 2 -1943)

Summary of Relevant Analyses of all Authentic Samples of German Green Aviation Fuel

Table 8. Analysis of German C3 Aviation Gasoline (1943)

| Ref. | Date of crash or capture of sample | 1943 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 9th May |  |  |  | 4th June | 21st June | 15th June |  |  |  |  |
|  | Sample No. | AIR285 | AIR287B | AIR289 | AIR291 | AIR305 | AIR311 | AIR312 | AIR317 | AIR318-321 | AIR328-330 | AIR331 |
|  | Type of plane | JU-88 |  | FW- 190 tank jettisoned (weathered) |  | FW-190 | FW-190 | FW-190 tank jettisoned (weathered) | Barrel Middle East | Barrel ex Middle East | Barrel ex Middle East | Middle East S298 TD 739 Suez Report |
|  | Engine |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Colour | Fluorescent Green |  |  |  |  |  |  |  |  |  |  |
| 2 | Density 15/15 | 0.773 |  | 0.783 |  | 0.772 | 0.768 | 0.787 | 0.771 | 0.773 | 0.773 | 0.772 |
| 3 | IPT Distillation 95% deg. C Final Boiling Point (FBP) deg. C | 163.5 175 |  | 178 |  | 167 175 | 165.5 175 | 179 | 160.5 170.5 | 160.5 173 | 162.5 173 | 171 |
| 4 | IPT % distilled +loss at 70 deg. C IPT % distilled +loss at 75 deg. C IPT % distilled +loss at 100 deg. C IPT % distilled +loss at 140 deg. C | 8 12 48 84 |  | - 5 39 - |  | 10.5 15 49.5 86 | 11 51 86 | 1 32 | 12 17.5 53.5 86.5 | 8.5 13.5 47 85 | 8.5 13 47 87 | 47 |
| 5 | TBP Vol% boiling above 165 deg. C | 9 |  |  |  | 7 | 8.5 |  | 6 | 7 | 8 |  |
| 6 | Wt%. Isobutane Wt%. n- Butane | 0 1.3 |  |  |  | 0.05 2.0 | 0.9 2.7 |  | 0 0 | 0 0.7 | 0 0.35 |  |
| 7 | Wt%. Iso-Pentane Wt%. n-Pentane Wt%. Total C4+C5 | 3.5 2.85 7.65 |  |  |  | 3.9 2.25 8.2 | 2.45 2.0 8.05 |  | 5.6 2.95 8.55 | 4.25 3.3 8.25 | 4.8 2.9 8.05 |  |
| 8 | Reid Vapour Pressure lbs. sq.ins. | 4.3 |  | 2.9 |  | 5.0 | 5.0 | 2.3 | 4.9 | 4.5 | 4.1 | 4.0 |
| 9 | Bromine number | 1.1 |  | 1.4 |  | 0.5 | 0.7 | 1.7 | 0.8 | 0.6 | 0.7 | 2 |
| 10 | Aniline point I deg. C | 22.8 |  |  |  | 23.7 | 24.4 |  | 20.6 | 19.6 | 20.1 |  |
| 11 | Aniline point IV deg. C | 58.8 |  | Wt.%(V) |  | 60.3 | 62.4 | Wt.%(V) | 63.2 | 62.5 | 62.3 | Wt. |
| 12a | Aromatics %wt. | 36.8 |  | 41.0 |  | 37.3 | 38.5 | 45 | 43.0 | 42.3 | 42.2 | 41 |
| 12b | Aromatics %Vol. (I.C.I.) | 32.7 |  |  |  | 33.2 | 34.0 |  | 38.2 | 37.6 | 37.5 |  |
| 13 | Naphthenes %Vol (I.C.I.) | 29.3 |  | 16.5 |  | 24.9 | 19.3 | 18 | 15.9 | 18.4 | 19.1 | 17 |
| 14 | Paraffins %Vol (I.C.I.) | 38.0 |  | 42.5 |  | 41.9 | 46.7 | 37 | 45.9 | 44.0 | 43.4 | 41 |
| 15 | Aromatic free density 15/15 | 0.727 |  |  |  | 0.725 | 0.715 |  | 0.712 | 0.713 | 0.714 |  |
|  | Knock Rating |  |  |  |  |  |  |  |  |  |  |  |
| 16 | C.F.R. Motor (as received) | 97.5 | Camb. Rough cut | 95 | Camb. Rough cut | 96.4 | 97.0 | 94.5 | 97.3 | 97 | 97 | 96 |
| 17 | T.E.L. Content mls/Imp. Gal | 5.6 |  | 6.3 |  | 5.5 | 5.5 | 6.75 | 5.6 | 5.7 | 5.75 | 5.6 |
| 18 | C.F.R. Motor, lead free | 81 |  | 82 |  | 82 | 82 | Camb. Rough cut | 83 | 83 | 83.5 | Camb. Rough cut |
| 19 | C.F.R. Motor, lead free & aromatic free | 77.7 |  |  |  | 77.1 | 78 |  | 78.1 | 79 | 79 |  |
|  | I.C.I. Distillation (Billingham) |  |  | Wt.% (V) |  |  |  |  |  |  |  |  |
| 20 | Benzene % Vol. (0-90OC) | 7.5 | 5.2 | 6.5 | 7.50 | 7.35 | 5.75 | 6.3 | 6.5 | 7.1 | 6.65 | 6.8 |
| 21 | Toluene % Vol. (0-118OC) | 10.05 | 11.5 | 13 | 12.3 | 10.4 | 11.0 |  | 12.5 | 12.2 | 11.65 | 11.3 |
| 22 | C8 Aromatics % Vol. (118-145OC) | 8.65 | Xylene 6.4 | 13 | Xylene 7.3 | 8.0 | 9.6 |  | 10.0 | 10.4 | 10.5 | 11.1 |
| 23 | C9 Aromatics % Vol. (145-166OC) | 2.55 | 8.5 |  |  | 3.8 | 3.95 |  | 3.5 | 3.65 | 3.25 |  |
| 24 | C10+ Aromatics % Vol. (>166OC) | 4.1 |  |  |  | 2.9 | 3.65 |  | 3.75 | 3.2 | 3.7 |  |
| 25 | Total Aromatics | 32.9 | 36 |  | 41.5 | 32.45 | 33.95 | 34.2 | 36.25 | 36.55 | 35.75 |  |
| 26 | Methyl cyclopentane | 5.8 |  |  |  | 5.9 | 5.8 |  | 5.1 | 5.05 | 5.1 |  |
| 27 | Cyclohexane | 4.2 |  |  |  | 3.75 | 2.95 |  | 3.2 | 2.75 | 3.65 |  |
| 28 | 2,2,4 Trimethyl Pentane (est.) | 13 |  |  |  | 12.5 | 13.5 |  | 13.5 | 14 | 13.5 |  |
| 29 | Higher Trimethyl Pentane (est.) | 5 |  |  |  | 7 | 7 |  | 7 | 6.5 | 7 |  |
| 30 | Total Trimethyl Pentane | 18 |  |  |  | 19.5 | 20.5 |  | 20.5 | 20.5 | 20.5 |  |
| 31 | Ethyl Benzene | 4.2 | Rough cut 5.5 |  | Rough cut 6.0 |  | 3.0 | Rough cut | 3.5 |  |  | 4.5 |
| 32 | Hydrindenes | 1.7 | 1.6 |  | 1.8 |  | 1.2 | 2 | 1.5+ |  |  | 1.6 |
| 33 | 2,2,4 Trimethyl Pentane (+2,2 Dimethyl Hexane) | 12.1 | 12.0 |  | 13.3 |  |  | 12 | 1.3 |  |  | 12 |
| 34 | Higher Trimethyl Pentane | 5.8-6.1 | 6.1 |  | 6.1 |  |  | 5 | 6 |  |  | 5.5 |
| 35 | Total Trimethyl Pentane | 18.2 | 18.1 |  | 19.4 |  |  | 17 | 19 |  |  | 17.5 |
|  | I.C.I. Report Number | G43/73 | - | - | - | G43/74 | G43/76 | - | G43/75,79 | G43/77 | G43/78 | - |

S= Analysis by Shell, V= Analysis by Vauxhall Lab. Other samples generally by I.C.I. Camb. = Cambridge

# Appendix II

AROMATIC HYDROCARBONS ANALYSIS

Considerable information about the amount and distribution of the aromatic hydrocarbons was obtained by fractionation and determination of the aromatic content of individual fractions. This method was part of the general Billingham method of analysis (see Appendix V and Refs. 3 & 4). Aromatic contents in the absence of unsaturated compounds are determined quite satisfactorily and with a fair accuracy by the simple aniline point method (before and after sulphonation) already described (Ref. 3 & 4)

This method only allows an estimate to be made of individual aromatics benzene and toluene, and gives a bulk figure for total C8 aromatics (118-145OC) and some information about the distribution of higher aromatics. Further information about the nature and amount of individual C8 and higher aromatics is obtained by ultra violet spectroscopic examination of individual fractions (see also Refs. 5, 6, 11, 12 etc.) It is possible to obtain limited information by direct spectroscopic examination without fractionation (table I and Refs. 5, 6, 7 etc.), but the results are not reliable.

## RESULTS

The aromatic composition and distribution obtained by fractionation and analysis by the aniline point method for fourteen green C3 type gasolines are collected in Table II, which also includes further information on individual aromatics obtained by spectroscopic methods applied to fractions.

In order to deduce possible methods for productions of those aromatics it was necessary to examine products from as wide a variety of known sources as possible. The results of such examinations have been given in detail in a number of separate reports (Refs. 13-55) and also in various summary reports (Refs. 1, 2, 5, 6, &7) and will be collected in a complete set of analyses to be issued in a further summary report.

Table III is included here, however to give the relevant information from fractionation of the amount and distribution of aromatics in most of the samples obtained from known sources, while Table IV records most of the additional information obtained by ultraviolet spectroscopic analysis.

## Interpretation of Results

The green C3 type of aviation gasoline has always been characterised by the high content of aromatics. In the earliest samples obtained in 1940 (G.F.28 – G.F.32) interest arose in the high aromatic content (10%) of high boiling (ca. 175OC) aromatics. Examination of those aromatics led to the discovery that 1-2 % of hydrindene was present. Hydrindene had also been found in the small amount of breakdown product obtained from naphthalene hydrogenation at 400OC for tetralin and decalin manufacture. Further Spectroscopic examination of this material showed that the monocyclic aromatics were predominantly of the type of normal mono-alkyl benzenes (methyl, ethyl, n-propyl and n-butyl) as would be expected from cracking of naphthalene via tetralin (Hall J.S.C.I. 1935 etc.). it was then conformed that ethyl benzene was the main C8 aromatic in the German green gasoline (65% of the C8 aromatics) and also that appreciable amounts of n-propyl benzene (2%) and n-butyl benzene (at least 2%) was present (Table II).

Extension of the spectroscopic method of analysis to a variety of other products (Table IV) showed that this type of aromatic composition – presence of hydrindene and predominance of normal alkyl benzenes was only found in products obtained by destructive hydrogenation of bituminous raw materials, such as naphthalene and creosote. The larger the number of steps used in the hydrogenation process the more these characteristics tend to disappear, but even in the high volatility aviation petrols made in two stages from creosote the amount of ethyl benzene is greater than that of xylenes (Table IV, line 24 etc.)

In no other products examined were these characteristics found and in general it was concluded that aromatics from all other sources were characterised by:

Absence of hydrindene

Relatively small proportion of ethyl benzene in C8 aromatics (20-30% or less – Table IV) and marked predominance of meta xylene (m/p ratio approx. 4/1)

Relatively small proportion of n-propyl benzene in C9 aromatics with marked predominance of ethyl toluenes (particularly meta) and pseudocumene.

Practically no n-butyl benzene and large amounts of ethyl xylenes etc.

Calling the two types of aromatic composition:

A = hydrindene and predominance of normal mono alkyl benzenes

B = no hydrindene and predominance of poly alkyl benzenes (particularly meta)

It will be seen from Tables II & IV that the samples examined can be divided into these classes:

Type A 	hydrindene and predominance of normal mono alkyl benzenes

Products from destructive hydrogenation of:

Naphthalene (in tetralin manufacture (Table IV line 26)

Creosote (in various stages of gasoline manufacture) (Table IV line 22-25)

German green C3 type aviation gasoline (Table II)

Types B 	no hydrindene and predominance of meta poly alkyl benzenes

Probably most natural virgin gasolines and naphthas from petroleum (Table IV lines 1-8)

Products from destructive hydrogenation of petroleum gas oils (Table IV lines 19-21)

Products from direct hydroforming of petroleum naphthas (Table IV lines 12-14)

Products from dehydrogenation (hydrofining) of petroleum naphthas of aviation boiling range

Products from dehydrogenation (hydrofining) of hydro-naphthas of aviation boiling range whether these have been produced from petroleum or bituminous raw materials (Table IV lines 16-18)

Aromatic oils from coal carbonisation (Table IV lines 9-11)

Table III lists the amount of aromatics and the distribution for 43 materials from various known sources, largely arranges in ascending order of aromatic content.

It will be seen that the most natural naphthas are immediately excluded from consideration as possible sources of aromatics in green fuel, on the ground that the total aromatic content is too low (Table III) and the distribution differs from that in the green fuels (Tables III and II). The above evidence would also exclude any aromatic extract obtained from such sources, while Borneo and Venezuelan ‘B’ naphthas which contain more aromatics, have the wrong composition (Table IV).

Most normal sensitive catalyst type of hydrogenation products are also excluded on the ground of too low total aromatic content, although Edeleanu extraction of a low aromatic content hydro-product from a bituminous raw material would possibly fit the analytical evidence.

Apart from the Edeleanu extraction other methods of producing a highly aromatic fuel are:

Catalytic cracking (e.g. hydrogenated brown coal tar))

‘Aromatisation’ or Fischer-Tropsch products

Hydrofining or selective dehydrogenation of cyclohexane naphthenes

Hydroforming of natural or hydro-naphthas

Direct addition of aromatics from coal carbonisation (or via hydro-refining)

Alkylation of coke oven benzole

High temperature aromatising hydro-cracking

Nothing is known about the composition of the aromatics from catalytic cracking, particularly if this were applied to a suitable hydrogenated fraction of brown coal tar; but it is thought that this process is not being used by the Germans.

Similarly, little is known about the possibility of aromatising Fischer-Tropsch products, but it is felt that unless products of high naphthene contents were available this method would not yield a naphtha rich enough on aromatics and poor enough in paraffins to use for the green fuel.

The hydrofining process of selective dehydrogenation of naphthenes to aromatics could certainly be used on a suitably naphthenic naphtha to yield material of high enough aromatic content. The process does not entail any noticeable amount of ‘splitting’ and would thus have to deal with naphtha already boiling in the aviation range (e.g. 80-170OC). If such a naphtha were from a petroleum source it would yield aromatics of the wrong composition and even if it were a product from hydrogenation of a bituminous material our experience shows that aromatic composition would be wrong; since in destructive hydrogenation down to the aviation boiling range the cyclohexane naphthenes tend to be of the poly-alkyl type and yield xylenes etc. on dehydrogenation. Thus, a hydro gasoline from creosote containing 12% aromatics and 3% of C8 aromatics will have 50-60% of ethyl benzene in the C8 aromatics. Dehydrogenation will raise the aromatics to 23% and the C8 aromatic content to 7½%, but the new C8 aromatics formed are mainly xylenes and the ethyl benzene in C8 aromatics falls to 30% (cf. Table IV lines 24 and 16).

The hydroforming process is similar to the hydrofining process, but is more severe with a small amount of cracking and more aromatisation by some cyclisation and isomerisation. The same arguments apply to this process, namely that the treatment of a hydro product already boiling with in the gasoline range will give aromatics rich in xylenes.

Examination of several samples of coke oven aromatics have shown these to be of type ‘B’ (cf. Tables IV lines 12-15) deficient in ethyl benzene with m-xylene predominating. Coal tar aromatics could not provide for the n-propyl and n-butyl benzene. It is possible that some coal tar benzole is being added to bring the benzene content to about 7%, since it is difficult to account for such a high benzene content by the proposed hydro-aromatisation process proposed below. This would be in keeping with the additions of large amounts (25%) of benzole to motor spirit. The low (0.02%) sulphur content of the green fuel may imply that the benzole is being given a hydro-refining treatment (to reduce sulphur content). The remarkably constant ratio of benzene to toluene and other aromatics (Table II) rather suggests that these are both formed by the same process and that benzene is not being added separately.

Alkylation of coke oven benzole with olefines derived either from Fischer-Tropsch or as ‘by products’ from dehydrogenation processes designed to produce synthetic octanes or synthetic rubber could be used to produce a higher boiling aromatic mixture. Such a process could be used for the production of ethyl benzene but it is very unlikely that ethyl benzene produced as such would be allowed to go to fuel when this is in demand for styrene. Alkylation would produce isopropyl benzene (Cumene) and tertiary and secondary butyl benzenes, but would not yield the normal derivatives. Neither isopropyl benzene nor tertiary butyl benzene have ever been detected in samples of green fuel.

The only process known to produce n-propyl benzene and n-butyl benzene as well as a preponderance of ethyl benzene and little xylenes is that involving the cracking of material containing polycyclic aromatics such as naphthalene. It is known that high temperature (500OC or more) hydrogenation of creosote oil from bituminous coal can give low volatility highly aromatic naphthas containing aromatics of this type. The hydrindene will be produced from indene present in the creosote.

NOTES ON SUGGESTED PROCESS FOR PRODUCTION OF AROMATICS BY HIGH TEMPERATURE “HYDRO-AROMATISING”

U.K. experience with this type of process is very limited, but it is known for many years before the war I.G Farbenindustrie were keen on so called “aromatising” processes which all produce highly aromatic material of low volatility.  It is known that the Welheim plant was designed to hydrogenate pitch in the liquid phase at a temperature of 490OC, and a high pressure of 700 atmospheres and that this process gave a ‘liquid phase’ petrol which contained as much as 70% of aromatics.

U.K. had limited experience some years ago of vapour phase aromatisation of aromatic middle oil (170-300OC) at 200 atmosphere and 530OC, indicated that the conversion to a 180OC end point naphtha was about 45%. The naphtha contained 38% aromatics and had a very low volatility (5% to 70OC, 22% to 100OC). The difficulty of dealing with the still bottoms on a straight through process in view of the apparent absence of any ‘normal’ vapour phase operation must be used. This would suggest that pressures are being used to overcome catalyst deterioration. It is known that I.G. were doing much work doing much work at pressures up to 800 atmospheres before the war and it is known that the plants at Stettin, Gelsenkirchen, Schelven, Welheim, Magdeburg and Nordstern were constructed to work at pressures up to 700 atmospheres on bituminous coal.  Such high-pressure high temperature converters will require special steels.

Since no brown coal hydrogenation products or brown coal tars have been available for treatment and examination it is not known whether brown coal would yield “aromatised” products with the characteristics described, but it is felt that the probability is very high that products from bituminous coal or any of its products such as creosote, or pitch.

The essential criterion of the process used is that the aromatics are formed directly by cracking of dicyclic (or poly cyclic) aromatics in the presence of some hydrogen to prevent olefine and carbon formation and loss of catalyst activity, but at a temperature high enough to prevent any appreciable hydrogenation to cyclohexane. This process is essentially different from the dehydrogenation of naphthenes which have been formed in another hydrogenation process (hydroforming or selective dehydrogenation) since the latter two-stage process gives the wrong type of aromatics.

## Evidence from the Non-Aromatic Hydrocarbon-Naphthenes

The composition of the non-aromatics is considered in Appendix III (octanes) and IV (naphthenes and other paraffins). It is sufficient to point out here that the green fuels are deficient in naphthenes particularly above 85OC. This is considered to be in keeping with the above view of the method pf production of the aromatics. The green fuel contains the normal amount of methyl cyclopentane (5%) and cyclohexane (4%) which would be associated with an average gasoline from petroleum, but is very deficient in higher boiling naphthenes, particularly the dimethyl cyclopentanes boiling at 90OC. This and other factors are considered to be evidence that the green fuel consists of low volatility highly aromatic naphtha of boiling range of 95-180OC produced as above, blended with a light-cut 0-85OC from some other sources.

## Recent Change in Composition of Green Fuel Aromatics

At the end of 1942 or early 1943, progressive changes occurred in the green fuel:

Addition of 3% more octane and more light petrol causing a reduction in aromatic content from 37% to 33% by dilution, but actually accompanied by a rise in weak (Lean) mixture rating

Still further increase of 2% in octane content with a return to the previous aromatic content of 37% and further rise in weak and rich mixture ratings.

The second part of this change was also accompanied by a marked change in the composition of the aromatics (samples AIR285-AIR328, table II). There was an increase of 1½-2% in toluene, about 2% increase in higher aromatics, but if anything, a decrease in the benzene content. Spectroscopy brought to light even more marked changes: -

Ethyl benzene fall from 6.2% to 3% or less in spite of overall rise in C8 aromatics, so that ethyl benzene in C8 aromatics fell from about 65% to about 30%.

The increase in C8 aromatics was partly due to meta-xylene and para-xylene but more remarkable was the sudden appearance of ortho-xylene at up to 1.8% where previously none had been detected.

Marked fall in n-propyl benzene from 2% to as low as 0.6% coupled with a rise in ethyl toluene (particularly meta).

Apparently also a reduction in amount of n-butyl benzene.

These changes are all away from the mono alkyl benzenes previously considered as characteristics of this type of aromatics produced by many other processes.

These changes have also been accompanied with a marked decrease in the amount of higher boiling naphthenes present in this fuel: -

Table 9. Composition of samples from captured sources

| OC |  |  | Naphthenes |  |  |  | Aromatics |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | AIR189 | AIR317 |  | AIR389 | AIR317 |
| C5 | 40-60 | Cyclopentanes | 0.7 |  | 1.0 |  |  |  |
| C6 | 60-75 | Methyl Cyclopentane | 4.1 |  | 5.1 | C6 Benzene | 8.8 | 6.5 |
| C6 | 75-84 | Cyclohexane | 5.35 |  | 3.25 |  |  |  |
| C7 | 84-95 | Dimethyl Cyclopentane | 0.9 |  | 1.2 | C7 Toluene | 12.3 | 12.5 |
| C7 | 95-110 | Methyl Cyclohexane | 4.65 |  | 1.6 |  |  |  |
| C8 | 110-125 | Dimethyl Cyclohexane | 1.55 |  | 1.1 | C8 Ethyl benzene m & p Xylene o-Xylene | 6.4 3.0 0 | 3.5 5.35 1.7 |
| C8 | 125-137 | Ethyl Cyclohexane | 2.95 |  | 1.1 |  |  |  |
|  | >137 |  | 5.8 |  | 2.55 | C9 n-Propyl Benzene Ethyl toluene | 2.0 1.4 | 0.6 2.75 |
| Total |  |  | 26.0 |  | 16.8 | Total Aromatic | 30½ | 36½ |
| Naphthenes >95OC |  |  | 16 |  | 6 |  |  |  |

In view of the similarity of composition of aromatics in the most recent green fuels with that of hydroformed products, particularly as regards the ortho-xylene and the decrease in naphthenes, it is suggested that the aromatised naphtha as made by the high temperature process described above, is now being given an additional treatment, probably by hydroforming which produces xylenes by conversion of the naphthenes  The decrease in benzene content may imply that benzene is not coming from the aromatising processes, but is being added as benzole. Dehydrogenation is still not complete since some cyclohexanes added as benzole. Dehydrogenation is still not complete since some cyclohexanes are still present in latest fuels (e.g. 1.65% methyl cyclohexane in sample AIR317)

Table 10. Aromatic hydrocarbons C8.

| Hydrocarbon | Green Fuel |  | Hydroformed Iranian Naphtha |
| --- | --- | --- | --- |
|  | 1942 | 1943 |  |
| % of C8 aromatics as ethyl benzene | 65 | 33 | 23 |
| % of C8 aromatics as m-xylene | 29 | 39 | 48 |
| % of C8 aromatics as p-xylene | 6 | 12 | 10 |
| % of C8 aromatics as o-xylene | 0 | 16 | 19 |

The view that the recent increase in weak (lean) and rich mixture ratings has been achieved by removal of a large part of the higher boiling naphthenes by hydroforming received some conformation from the A.I.O.C. (Anglo-Iranian Oil Company) experiment in which a sample of green gasoline of the previous type was subjected to hydroforming and considerable increases in ratings achieved.

Table 11. Analysis of captured samples

|  | 1941 |  | 1943 |
| --- | --- | --- | --- |
| Sample No. | AIR141 |  | AIR317-330 |
|  | As received | Hydroformed | As received |
| % Aromatics | 37½ |  | 36½ |
| Ethyl Benzene in C8 aromatic | 65 |  | 35 |
| Total % naphthenes | 27 |  | 17 |
| Knock Rating |  |  |  |
| Weak (Lean) C.F.R. motor | 94 | 95 | 97 (1) |
| Rich 3-C %grade 130 | 100-110 | 140 | >125 |

Due also to addition of further 5% octane and more light petrol

Photo 7. Dornier Do-17Z refuelling

Table II

DISTRIBUTION & COMPOSITION OF AROMATICS IN GERMAN C3 (GREEN) AVIATION GASOLINE

Table 12, Distribution and composition of aromatics in German C3 Aviation Gasoline

| Green Gasoline Ref. No | GF 28 | No. 40/41 | GF31 | GF32 | AIR141 | AIR144 | AIR176 | AIR109 | AIR285 | AIR305 | AIR311 | AIR317 | AIR318 | AIR328 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | 1940 |  |  |  | 1941 |  | 1942 |  | 1943 |  |  |  |  |  |
| Vol.% on fuel (Aniline Point Method) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene 0-90OC | 6.2 | 6.7 | 6.35 | 6.55 | 8.05 | 10.1 | 8.5 | 8.8 | 7.5 | 7.35 | 5.75 | 6.5 | 7.1 | 6.65 |
| Toluene 90-118OC | 7.8 | 7.55 | 8.6 | 7.9 | 12.2 | 11.5 | 12.5 | 12.3 | 10.05 | 10.4 | 11.0 | 12.5 | 12.2 | 11.65 |
| C8 Aromatics 118-145OC | 6.65 | 6.3 | 7.55 | 6.95 | 9.25 | 8.55 | 9.8 | 9.2 | 8.65 | 8.0 | 9.6 | 10.0 | 10.4 | 10.5 |
| C9 Aromatics 145-166OC | 3.2 | 2.5 | 4.35 | 3.5 | 4.15 | 4.0 | 4.0 | 4.4 | 2.55 | 3.8 | 3.95 | 3.3 | 3.65 | 3.25 |
| C10 Aromatics >166OC | 9.55 | 9.9 | 10.5 | 10.6 | 4.05 | 4.15 | 2.6 | 3.9 | 4.1 | 2.9 | 3.65 | 3.8 | 3.2 | 3.7 |
| TOTAL | 33.4 | 32.55 | 37.55 | 35.5 | 37.7 | 37.4 | 37.4 | 38.6 | 32.85 | 32.45 | 33.95 | 36.3 | 36.55 | 35.75 |
| Distribution (% on total aromatics) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C6 | 18 ½ | 19 ½ | 17 | 18 ½ | 21 ½ | 26 ½ | 23 | 23 | 23 | 22 ½ | 17 | 18 | 19 ½ | 18 ½ |
| C7 | 23 ½ | 23 | 23 | 22 | 32 | 31 | 33 | 32 | 30 ½ | 32 | 32 ½ | 34 ½ | 33 ½ | 33 |
| C8 | 20 | 19 ½ | 20 | 19 ½ | 24 ½ | 23 | 26 | 24 | 26 | 24 ½ | 27 | 27 ½ | 28 ½ | 29 ½ |
| C9 | 9 ½ | 8 | 11 ½ | 10 | 11 | 8 ½ | 11 | 11 | 8 | 12 | 11 ½ | 9 ½ | 10 | 9 |
| C10+ | 28 ½ | 30 | 28 ½ | 30 | 11 | 11 | 7 | 10 | 12 ½ | 9 | 11 | 10 ½ | 8 ½ | 10 |
| Distribution (% on C6+C7+C8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C6 | 30 | 31 ½ | 28 ½ | 30 ½ | 27 ½ | 33 ½ | 28 | 29 | 28 ½ | 28 ½ | 22 | 22 ½ | 24 | 23 |
| C7 | 38 | 37 ½ | 38 | 37 | 41 | 38 | 40 | 41 | 38 ½ | 40 ½ | 42 | 43 | 41 | 40 ½ |
| C8 | 32 | 31 | 33 ½ | 32 ½ | 31 ½ | 28 ½ | 32 | 30 | 33 | 31 | 36 | 34 ½ | 35 | 36 ½ |
| UV Spectroscopic Analysis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ethyl Benzene |  |  |  |  | 6.1 | 5.6 | 6.2 | 6.4 | 4.2 |  | 2.45 | 3.5 |  |  |
| m-Xylene |  |  |  |  | 3.15 | 2.65 | 2.9 | 2.3 | 2.5 |  | 3.35 | 4.15 |  |  |
| p-Xylene |  |  |  |  | 0.35 | 0.35 | 0.7 | 0.7 | 0.7 |  | 1.25 | 1.2 |  |  |
| o-Xylene |  |  |  |  | 0 | 0 | 0 | 0 | 0.95 |  | 1.85 | 1.7 |  |  |
| % Ethyl benzene in C8 aromatics |  |  |  |  | 63 | 65 | 63 | 69 | 50 |  | 28 | 33 |  |  |
| n-Propyl benzene |  | 2 |  |  | 2.0 | 2.0 | 2.4 | 2.0 | 0.9 |  | 0.9 | 0.6 |  |  |
| m-Ethyl benzene |  | 0.5 |  |  |  |  | 1.2 | 1.0 | 1.75 |  | 1.55 | 2.1 |  |  |
| p-Ethyl benzene |  | 0.3 |  |  |  |  | 0.4 | 0.3 | 0.5 |  | 0.5 | 0.65 |  |  |
| pseudo Cumene |  |  |  |  |  |  | 0.6 |  | 0.8 |  | 1.05 | 0.8 |  |  |
| n-Butyl benzene |  | 0.8 |  |  | 1.9 | 2 | 2 | present |  |  |  |  |  |  |
| Hydrindenes |  | 1.1 |  |  | 1.4 | 2.1 | 1.4 | 1.4 | 1.7 |  | 1.4 | 1.5 |  |  |
| Naphthalene |  |  |  |  |  |  |  |  |  |  | 0.08 |  |  |  |

Table III

AMOUNT & DISTRIBUTION OF AROMATICS IN AVIATION PRODUCTS FROM KNOWN SOURCES

Table 13. Distribution of Aromatics (All figures Vol. % by Aniline Point Method)

| Source | End Pt. OC | Total aromatics Vol.% | Distribution Vol.% on fuel |  |  |  |  | Distribution as % of Aromatics |  |  |  |  |  | % Ethyl Benzene in C8 Aromatics |  | % Hydrindenes |  | % Naphthenes in fuel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0-90 | 90-118 | 118-145 | 145-166 | 166 | C6 | C7 | C8 | C9 | C10 |  |  |  |  |  |  |
|  |  |  | C6 | C7 | C8 | C9 | C10+ |  |  |  |  |  |  |  |  |  |  |  |
| Natural petroleum gasolines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Russian Fuel AIR151 | 152 | 1.75 | 0.25 | 0.5 | 0.8 | 0.2 | - | 15 | 29 | 44½ | 11½ | - | 50 |  | Abs. |  | 64½ |  |
| Midway light cut | 103 | 2.05 | 1.35 | 0.7 | - | - | - | 65 | 35 | - | - | - |  |  |  |  | 39½ |  |
| Reitbrook (Germany) | 149 | 2.6 | 0.1 | 0.85 | 1.65 |  | - | 4½ | 32½ | 63 |  | - | 48 |  | Abs. |  | 48½ |  |
| Trinidad high octane | 95 | 3.3 | 2.4 | 0.9 | - | - | - | 72 | 28 | - | - | - |  |  |  |  | 39½ |  |
| Miri aviation | 111 | 3.3 | 1.15 | 1.9 | 0.25 | - | - | 35 | 58 | 7 | - | - |  |  |  |  | 64 |  |
| Lagunillas light naphtha | 140 | 3.5 | 0.75 | 1.75 | 1.0 |  | - | 21½ | 50 | 28½ |  | - | 30 |  | Abs. |  | 45 |  |
| Venezuelan ‘A’ | 153 | 4.1 | 0.5 | 1.15 | 1.85 | 0.55 |  | 12½ | 28 | 45½ | 15½ |  |  |  |  |  | 47 |  |
| Kettleman light cut | 111 | 4.3 | 1.55 | 2.75 | -- | - | - | 36 | 64 | - | - | - |  |  |  |  | 37 |  |
| Californian (8127) | 124 | 5.25 | 1.2 | 3.3 | 0.75 | - | - | 23 | 63 | 14 | - | - |  |  |  |  | 45 |  |
| Sumatra (Palembang) | 96 | 5.5 | 5.2 | 0.3 | - | - | - | 95 | 5 | - | - |  |  |  |  |  | 24 |  |
| Venezuelan (8080) | 148 | 5.9 | 0.7 | 2.35 | 2.25 | 0.6 |  | 12 | 40 | 38 | 10 |  |  |  |  |  | 43 |  |
| Shell light cut (SR.164) | 112 | 6.6 | 2.15 | 4.45 | - | - | - | 33 | 67 | - | - | - |  |  |  |  | 31½ |  |
| Roumanian (8247) | 126 | 6.95 | 1.4 | 5.0 | 0.6 |  | 0 | 20 | 71½ | 8½ |  | - | 33 |  | Abs. |  | 48½ |  |
| Roumanian (P.B.) | 150 | 7.35 | 1.2 | 3.05 | 2.4 | 0.6 | 0.1 | 16 | 41½ | 33 | 8½ | 1 |  |  |  |  | 40½ |  |
| Lagunillas gasoline | 214 | 7.5 | 0.5 | 1.2 | 2.05 | 1.05 | 2.75 | 6 | 16 | 27½ | 14 | 36½ |  |  |  |  | 47½ |  |
| Shell 73 Octane | 151 | 10.9 | 1.5 | 4.6 | 3.7 | 1.1 | - | 14 | 42 | 34 | 10 | - |  |  |  |  | 44½ |  |
| Venezuelan ‘B’ | 163 | 21.0 | 1.4 | 3.5 | 8.8 | 2.6 | 4.7 | 6½ | 17 | 42 | 12 | 22½ | 10 |  | Abs. |  | 29½ |  |
| Borneo ‘C’ grade | 178 | 28.3 | 3.9 | 9.4 | 8.1 | 3.95 | 2.95 | 14 | 33 | 29 | 14 | 10 | 17 |  | Abs. |  | 35 |  |
| ‘Normal’ hydrogenation gasolines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Creosote ‘saturated’ product | 290 | 21.6 | 0.7 | 1.0 | 1.85 | 1.0 | 17.05 | 3½ | 4½ | 8½ | 4½ | 79 | 65 |  | 0.9 |  | 75 |  |
| From creosote Middle Oil | 200 | 6.65 | 1.25 | 1.2 | 1.35 | 0.5 | 2.3 | 18½ | 18½ | 20½ | 8 | 35 | 56 |  | 0.29 |  | 63½ |  |
| From creosote Middle Oil | 150 | 2.8 | 0.75 | 0.95 | 0.7 | 0.25 | 0.15 | 26½ | 34 | 25 | 9 | 5½ | 50 |  | 0.05 |  | 54 |  |
| From creosote Middle Oil | 145 | 4.25 | 1.7 | 1.35 | 1.1 | 0.1 | - | 39 | 32 | 26 | 3 | - |  |  |  |  | 50 |  |
| From creosote Middle Oil | 130 | 12.9 | 4.45 | 5.4 | 3.05 | - | - | 34½ | 42 | 23½ | - | - | 50 |  | (Low F.B.P.) |  | 41 |  |
| From creosote Heavy Oil | 130 | 15.35 | 4.7 | 8.25 | 2.4 | - | - | 31 | 54 | 15 | - | - | 59 |  | (Low F.B.P.) |  | 44½ |  |
| From petroleum gas oil | 200 | 10.2 | 0.7 | 2.05 | 2.9 | 2.1 | 2.45 | 7 | 20 | 28½ | 20½ | 24 | 20 |  | Abs. |  | 41½ |  |
| From petroleum gas oil | 130 | 6.45 | 1.15 | 3.65 | 1.65 |  | - | 18 | 56½ | 25½ |  | - | 25 |  | Abs. |  | 33½ |  |
| From petroleum gas oil | 130 | 6.55 | 1.4 | 4.4 | 0.75 | - | - | 21½ | 67 | 11½ | - | - | 28 |  | Abs. |  | 32½ |  |
| From petroleum gas oil (hydro Albanian) | 144 | 9.6 | 1.1 | 3.45 | 4.35 | 0.7 |  | 11½ | 37 | 45 | 7½ |  |  |  |  |  | 32 |  |
| From petroleum gas oil (Intava) | 137 | 3.2 | 0.55 | 1.3 | 1.3 | 0.05 |  | 17 | 40½ | 40 ½ | 1½ |  |  |  |  |  | 42 |  |
| Hydrofined (Catalytic Dehydrogenation) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hydro creosote gasoline | 140 | 22.4 | 4.3 | 10.4 | 7.5 | - | - | 20 | 45½ | 33½ | - | - | 31 |  | Abs. |  | 31½ |  |
| Hydro petroleum gasoline | 140 | 13.8 | 1.4 | 3.3 | 4.1 | - | - | 10 | 60 | 30 | - | - | 17 |  | Abs. |  | 25 |  |
| Hydro petroleum (SO2) Extract | 160 | 70 | 2 | 23 | 34.5 | 10.5 |  | 3 | 32 | 50 | 15 |  | 8 |  | Abs. |  |  |  |
| Hydroformed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| East Texas Naphtha | 150 | 31.3 | 4.2 | 11.7 | 11.15 | 4.23 |  | 13½ | 37 | 35½ | 14 |  | 20 |  | Abs. |  | 25.1 |  |
| Iraqi naphtha | 153 | 58.8 | 4.9 | 20.5 | 26.9 | 3.5 | 2.6 | 8 | 35 | 46 | 6½ | 4½ |  |  |  |  | 6.2 |  |
| Iranian naphtha No.1 | 162 | 40.3 | 3.2 | 12.8 | 15.8 | 8.0 |  | 8 | 32 | 40 | 20 |  | 23 |  | Abs. |  | 12.5 |  |
| Iranian naphtha No.2 | 188 | 50.4 | 2.5 | 14.9 | 21.6 | 5.5 | 4.4 | 5 | 30 | 43 | 13 | 9 | 24 |  | Abs. |  | 7.0 |  |
| Hydro creosote 130-200OC naphtha | 220 | 61.5 | 0.8 | 3.4 | 7.1 | 9.7 | 40.5 | 1½ | 5½ | 11½ | 15½ | 66 | 28 |  | 5 |  | 29 |  |
| High Temperature Aromatising |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘Aromatised’ creosote | 186 | 38.2 | 4.5 | 9.8 | 10.2 | 9.5 | 4.2 | 12 | 25½ | 26½ | 25 | 11 |  |  |  |  | 52.7 |  |
| Fores ex tot & dec. | 187 | 38.6 | 0.3 | 1.1 | 9.8 | 27.4 |  | 1 | 3 | 25 | 71 |  | 87 |  | 8.8 |  | 61 |  |
| Aromatics from Coal Carbon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gas spirit |  | 67.9 | 0 | 9.5 | 51.5 | 4.5 | 1.4 | 0 | 14 | 76 | 6½ | 3½ | 11 |  | Abs. |  |  |  |
| Coke Oven spirit |  | Ca95 | 0 | 32 | 63 | 5 | 0 | 0 | 32 | 63 | 5 | 0 | <1 |  | Abs. |  |  |  |
| Coke Oven ‘Xylenes’ |  | 100 |  |  | >95 |  |  |  |  |  |  |  | <10 |  | Abs. |  |  |  |
| Catalytically Cracked Petroleum | 251 | 13.0 | 1.5 | 5.25 | 5.1 | 1.1 | Tr. | 11½ | 40½ | 39 | 8½ |  |  |  |  |  | 32½ |  |

Table IV

COMPOSITION OF INDIVIDUAL AROMATICS IN A VARIETY OF GASOLINES & BLENDING AGENTS FROM KNOWN SOURCES

Table 14. Composition of Aromatics in various gasolines and blendstocks

| Source | By fraction & Aniline Point |  |  |  | U.V. Spectroscopic Results |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total Aromatics | Benzene 0-90Oc | Toluene 90-90-118OC | C8 Aromatics |  |  |  |  | C9 Aromatics |  |  |  |  | C10 n-Butyl Benzene | Hydrindenes |  | % Ethyl Benzene in C8 Aromatics |
|  |  |  |  | Ethyl Benzene |  | Xylenes |  |  | n-Propyl Benzene | Ethyl Toluene |  | Trimethyl Benzene |  |  |  |  |  |
|  |  |  |  |  |  | meta | para | ortho |  | m | p | 1,2,4 | 1,3,5 |  |  |  |  |
| Straight Run Petroleum Naphthas |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Russian fuel AIR151 | 1.8 | 0.3 | 0.5 | 0.4 |  | 0.3 | 0.1 | 0 | 0.2 | No higher aromatics present |  |  |  |  |  | Abs. | 50 |
| 2. Reitbrook gasoline (Germany) | 2.6 | 0.1 | 0.85 | 0.55 |  | 0.3 | 0.3 | 0 | 0.36 (i) |  | 0.11 |  |  |  | Abs. |  | 48 |
| 3. Lagunillas light naphtha | 3.5 | 0.75 | 1.75 | 0.25 |  | 0.45 | 0.15 | 0 |  | No higher aromatics present |  |  |  |  |  |  | 30 |
| 4. Roumanian (8247) | 7.0 | 1.4 | 5.0 | 0.2 |  | 0.3 | 0.1 | 0 |  |  |  |  |  |  | Abs. |  | 33 |
| 5. Reitbrook naphtha | 12.2 | 0 | 0 | 0.3 |  | 0.15 | 0.15 | 0 | 0.2 |  | 1.2 | 3.6 | 1.0 |  | Abs. |  | 50 |
| 6. Lagunillas heavy naphtha | 17.6 | 0 | 0.15 | 0.5 |  | 0.65 | 0.2 | 0.3 | 0.4 | 0.2 | 0.2 | 0.7 | 0.5 |  | Abs. |  | 30 |
| 7. Venezuelan ‘B’ | 21.0 | 1.4 | 3.5 | 0.9 |  | 5.9 | 0.8 | 1.3 | Tr. | 1.4 | 0.4 | 0.65 | 0.35 |  | Abs. |  | 10 |
| 8. Borneo ‘C’ grade | 28.3 | 3.9 | 9.4 | 1.3 |  | 5.4 | 0.9 | 0 | 0.55 (i) | 1.6 | 0.4 |  | 1.2 |  | Abs. |  | 17 |
| Aromatic Oils from Coal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9. Gas Spirit | 67.9 | 0 | 9.5 | 5.9 |  | 28.8 | 8.2 | Ca. 8.5 |  | 0.1 | 0.05 |  | 0.05 |  | Abs. |  | 11 |
| 10. Coke Oven Spirit | Ca95 | 0 | 32 | 0.3 |  | 39.0 | 9.15 | Ca. 12.6 |  |  |  | 0.2 |  |  | Abs. |  | <1 |
| 11. Coke Oven ‘Xylenes’ | >95 |  |  | >5 |  | Ca. 75 | Ca. 23 |  |  |  |  |  |  |  | Abs. |  | <10 |
| Hydroformed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12. East Texas Naphtha | 31.3 | 4.2 | 11.7 | 2.3 |  | 6.05 | 1.3 | 2.0 |  | 0.9 | 0.5 | 1.0 | 0.6 |  | Abs. |  | 20 |
| 13. Iranian naphtha No.1 | 40.3 | 3.2 | 12.8 | 3.6 |  | 7.4 | 1.65 | 2.85 | 1.3 | 3.6 | 0.9 | 0.8 | 0.4 |  | Abs. |  | 23 |
| 14. Iranian naphtha No.2 | 50.4 | 2.5 | 14.9 | 5.5 |  | 11.75 | 3.05 | 2.5 | Present | 1.1 | 0.65 | 3.5 | 1.5 |  | Abs. |  | 24 |
| 15. Hydro creosote 130-200OC naphtha | 61.5 | 0.8 | 3.4 | 2.0 |  | 3.75 | 0.95 | 0.4 | 1.4 | 6.8 | 2.25 | 2.3 | Present | 8.0 | 5.0 |  | 28 |
| Selective Dehydrogenation (Hydrofining) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16. From Hydrogenated Creosote (130O end pt.) | 22.4 | 4.5 | 10.4 | 2.35 |  | 4.1 | 1.05 | 0 | No higher aromatics |  |  |  |  |  | Abs. |  | 31 |
| 17. From Hydrogenated petroleum (130O end pt.) | 13.8 | 1.4 | 8.3 | 0.9 |  | 2.5 | 0.9 | 0.2 | No higher aromatics |  |  |  |  |  |  |  | 17 |
| 18. From Hydro petroleum (SO2) Extract) | 70 | 2 | 23 | 2.9 |  | 18.55 | 6.95 | 6.3 |  | 1.9 | 0.7 | 3.8 | 1.1 |  | Abs. |  | 8 |
| Hydrogenation (a) from petroleum gas oil |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19. 70% volatility petrol (6/1536) | 1.45 | 1.15 | 3.65 | 0.4 |  | 0.9 | 0.25 | 0.05 | No higher aromatics |  |  |  |  |  | Abs. |  | 25 |
| 20. 200OC F.B.P. petrol (6/1954) | 10.2 | 0.7 | 2.05 | 0.6 |  | 1.6 | 0.5 | 0.3 |  | 0.8 | 0.53 | 0.65 | 0.25 |  | Abs. |  | 20 |
| 21. 75% volatility petrol (43/1/37) | 6.55 | 1.4 | 4.4 | 0.2 |  | 0.4 | 0.1 | 0.05 | No higher aromatics |  |  |  |  |  | Abs. |  | 29 |
| Hydrogenation (b) from creosote & naphthalene |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22. 150OC F.B.P. petrol | 2.8 | 0.75 | 0.95 | 0.3 |  | 0.25 | 0.05 | 0.01 | 0.14 | 0.03 | 0.01 |  |  |  | 0.05 |  | 50 |
| 23. 200OC F.B.P. petrol | 6.65 | 1.25 | 1.2 | 0.75 |  | 0.5 | 0.1 | 0 | 0.15 | 0.2 | 0.05 |  | 0.1 | 0.45 | 0.29 |  | 56 |
| 24. 130OC F.B.P. petrol (42/9/5) | 12.9 | 4.45 | 5.4 | 1.35 |  | 1.25 | 0.3 | 0 | No higher aromatics |  |  |  |  |  |  |  | 50 |
| 25. Total Saturated prod. ex creosote | 21.6 | 0.7 | 1.0 | 1.2 |  | 0.55 | 0.1 | 0 | 0.25 | 0.15 | 0.05 |  |  | 0.4 | 0.9 |  | 65 |
| 26. Forerunnings ex Tet. & dec. | 38.6 | 0.3 | 1.1 | 6.6 |  | 0.9 | 0.05 | Trace | >2 | 1.0 | 0.4 |  | 2.0 | 8.2 | 8.8 |  | 87 |

# Appendix III

COMPOSITION OF SYNTHETIC OCTANE IN GERMAN GREEN AVIATION GASOLINES

- FRACTIONATION & ANALYSIS BY ANILINE POINT

The results obtained by fractionation in 30-40 plate stills and analysis of fractions by the aniline point method (Ref. 3,4) are collected in Table IX, Appendix IV. This method applied to the first samples of the green fuel received in 1940 showed that some 10% of the fuel consisted of paraffins boiling at about 99OC. These fractions, after freeing from small amounts of toluene had high aniline point of above 72OC (see Table XIV Appendix V) and of high octane number.

The presence of 2,2,4 Trimethyl Pentane was confirmed by infra-red spectroscopic analysis.

It was also found that a further 4% of paraffins distilled at about 108-111OC and when freed from toluene these were also found to have a high octane number. Spectroscopic analysis showed this fraction to consist largely of 2,3,4 Trimethyl Pentane (which apparently distils at a lower temperature in the presence of much toluene).

Two rough methods for estimation of the amount of trimethyl pentanes from these figures have been used: -

From octane numbers of aromatic-free fractions by calculation

By assuming figures for the small amount of normal heptane and dimethyl hexanes present in the base fuel.

In this way it has been possible to arrive at a rough estimate of the amount of 2,2,4 Trimethyl Pentane and the amount of “higher boiling trimethyl pentanes” (TMP) present in such fuels. The figures deduced in this way are given in Table I lines 28-30 (Appendix I) and are to be compared in a number of instances with the results obtained by infra-red spectroscopy. (Table I lines 33-35). In general, the fractionation method gives a useful rough figure for total trimethyl pentanes and an approximate figure for the proportion of 2,2,4 Trimethyl Pentane. This method of course gives no information about the individual higher boiling trimethyl pentanes or dimethyl hexanes and would be badly in error if there were large changes in the amount of n-heptane present.

These rough figures are:

Table 15. TMP content of captured samples.

|  | 1940 |  |  |  | 1941 |  | 1942 |  | 1943 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | GF 28 | No. 40/41 | GF31 | GF32 | AIR141 | AIR 144 | AIR 176 | AIR180 | AIR285 | AIR305 | AIR 311 | AIR 317 | AIR 318 | AIR 328 |
| 2,2,4 Trimethyl Pentane | 9 | 9 | 10½ | 10 | 12½ | 12 | 11 | 7 | 13 | 12½ | 13½ | 13½ | 14 | 13½ |
| Higher Boiling TMP | 3 | 3 | 3½ | 3½ | 4½ | 5 | 4 | 8 | 5 | 7 | 7 | 7 | 6½ | 7 |
| Total | 12 | 12 | 14 | 13½ | 17 | 17 | 16 | 15 | 18 | 19½ | 20½ | 20½ | 20½ | 20½ |
| % 2,2,4 in total TMP | 75 | 75 | 75 | 74 | 73 | 71 | 74 | 46 | 72 | 64 | 66 | 66 | 67 | 66 |

These rough results shown that there have been small increases in total trimethyl pentanes on several occasions, the most marked increase being at the end of 1942 or early in 1943. With the sole exception of sample AIR189 these rough figures would indicate that the same type of octane containing 75% 2,2,4 trimethyl pentane had been used throughout.

B.	ANALYSIS BY INFRA RED SPECTROSCOPY

It was considered that a detailed knowledge of the composition of the octane components might enable a conclusion to be reached as to the type of process being used. The method which has been used mostly has been used to use the fractions produced in the 30-plate distillation of 6-8 litres of material done at Billingham for analysis by aniline point method.

Since toluene interferes it is necessary to analyse the toluene free fractions. Generally, toluene has been removed by shaking with 98-99% sulphuric acid. It has been realised that this method may cause decomposition and isomerisation of the trimethyl pentane and time of contact with acid was consequently reduced to a minimum (5-10 minutes) when it was felt that little decomposition occurred, at least as regards 2,2,4 trimethyl pentane. Recently it has been possible to remove toluene by azeotropic distillation with methanol followed by refractionation. The composition found for the octanes by this method (Ref. 81) is practically identical with that found by the much more rapid method of acid washing, so that it is concluded that the rapid acid washing method is fairly satisfactory provided time of contact is kept short.

C.	Reference Synthetic Octanes-Characteristics

In order to identify the method of manufacture it would be desirable to examine a very wide field of alkylates and hot and cold hydro-codimers made under all varieties of conditions, but unfortunately relatively few samples representatives of different types of processes have been available, but the samples examined appear to show sufficiently well defined differences in composition for these to be assumed to be characteristics of the processes in question. Table V gives the infra-red analysis of only five samples, but the general conclusions appear to be well supported by other analyses reported in its literature.

The main points are:

C4 Alkylates are characterised by:

A preponderance of 2,3,3 trimethyl pentane as compared with 2,3,4 TMP (1.4/1)

Little or no 2,2,3 trimethyl pentane (loss than 1%)

Relatively little 2,3 dimethyl pentane

The presence of appreciable amounts of dimethyl hexanes (10-15%)

The presence of appreciable amounts of 2,2,5 Trimethyl Hexane (5%)

C4 hydrocodimers (U.O.P. (Universal Oil Products) phosphoric acid type process and possibly hot sulphuric acid type process) are characterised by:

A preponderance of 2,3,4 trimethyl pentane as compared with 2,3,3 TMP

More 2,2,3 trimethyl pentane than alkylates (5-10%)

Appreciably more 2,3 dimethyl pentane than alkylate (10%)

Much less dimethyl hexanes than in alkylates (generally less than 2%) and generally present as 2,2 dimethyl hexane, whereas alkylates contain more of the 2,4 and 2,5 isomers

Much less 2,2,5 trimethyl hexane than alkylate (less than 1% or not detected)

Sometimes if a sharp cut has not been made, some hydro-trimer may be present (such as a pentamethyl heptane B.P. 170-180OC)

C4 hydrocodimers (cold acid type) characterised by:

Main component is 2,2,4 trimethyl pentane which generally amounts to 85-95% of the products. Other components probably 2,3,4 TMP.

The content of 2,2,4 trimethyl pentane is little guide to identification unless this is above 80% which would indicate a cold-acid type of octane. The amount of 2,2,4 TMP is known to vary from as low as 10% up to as high as 60% or more in hot acid octanes and alkylates, depending very much on temperature of polymerisation etc.

The appreciable content of 2,3 dimethyl pentane in U.O.P. type octane from the Heysham Plant is possibly due to the presence of some propane in the butane and may not be entirely characteristic of this process.

Other alkylates (not reported here) have apparently had a very similar composition and several authors have noted that the presence of dimethyl hexanes and 2,2,5 trimethyl hexane is characteristic of this process.

Nothing is known of the composition of possible cross-polymers such as C3-C5 alkylate, C4-C5 alkylate, C4-light petrol alkylate, etc. It is considered that these will have a more complex composition and will probably contain more single branch paraffins such as are beginning to appear in C3-C4 alkylates

Photo 8. Heinkel He-111 refuelling in Russian winter

Table V

REFERENCE SYNTHETIC OCTANES (VOL. %)

(Characteristic Hydrocarbons which differentiate two types are marked ‘C’)

Table 16. Reference Synthetic Octanes

| Hydrocarbon | B.P.OC | Alkylates |  |  | Hydrocodimers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Aruba I.A.A. 119 | Baton Rouge I.A.A. 122 | Texas P.P.F 798 | Standard Oil P.P.F. 795 | Aruba A204 | Heysham HG.6 | Heysham 43/3/60 | Standard Indiana N.A.C.A. |
| Trimethyl Pentanes |  |  |  |  |  |  |  |  |  |
| 2,2,4 | 99.2 | 30.95 | 22.5 | 22.4 | 30.7 | 12 | 30.0 | 26.5 | 9.9 |
| 2,2,3 | 100.8 | 2.7 | 1.0 | 11.5 | 1.6 | 18 | 8.25 | 8.5 | 9.2 |
| 2,3,4 | 113.5 | 13.0 | 9.5 | 14.8 | 9.0 | 28 | 36.75 | 34.35 | 43.9 |
| 2,3,3 | 114.7 | 18.0 | 14.5 | 10.4 | 14.5 | 28 | 5.5 | 4.1 | 8.6 |
| Total |  | 64.7 | 47.5 | 49.1 | 55.8 | 86 | 80.5 | 73.5 | 71.6 |
| % Distribution of trimethyl pentanes |  |  |  |  |  |  |  |  |  |  |
| 2,2,4 |  | 48 | 47½ | 45½ | 55 | 14 | 37 | 36 | 14 |
| 2,2,3 |  | 4 | 2 | 3 | 3 | 21 | 10 | 11½ | 13 |
| 2,3,4 |  | 20 | 20 | 30 | 16 | 32½ | 45½ | 46½ | 61 |
| 2,3,3 |  | 28 | 30½ | 21½ | 26 | 32½ | 7½ | 5½ | 12 |
| Other characteristic components |  |  |  |  |  |  |  |  |  |  |
| C8 |  |  |  |  |  |  |  |  |
| 2,2 dimethyl hexane | 106.8 | - | - | - | - | 2 | 2.0 | 1.95 | 1.2 |
| 2,5 dimethyl hexane | 109.1 | 5.2 | 4.0 | 5.2 | 1.0 | - | - | - | 2.7 |
| 2,4 dimethyl hexane | 109.4 | 3.3 | 2.5 | 5.1 | 7.1 | - | - | - | 4.0 |
| 3,3 dimethyl hexane | 111.9 | - | - | 0.9 | 0.5 | - | - | - |  |
| 2,3 dimethyl hexane | 115.6 | 0.95 | 1.4 | 3.4 | 3.7 | - | - | 2.15 | 5.6 |
| C7 |  |  |  |  |  |  |  |  |
| 2,2 dimethyl pentane | 79.2 | - | <0.05 | - | - | - | 1.25 | 1.2 | 1.2 |
| 2,4 dimethyl pentane | 80.6 | 3.65 | 3.3 | 6.2 | 3.6 | - | 2.2 | 1.6 |  |
| 2,3 dimethyl pentane | 89.7 | 2.5 | 2.1 | 3.4 | 1.6 | - | 11.65 | 7.9 | 1.1 |
| C6 |  |  |  |  |  |  |  |  |
| 2,2 dimethyl butane | 49.8 | - | - | 0.5 | - | - |  | - |  |
| 2,3 dimethyl butane | 58.0 | 4.55 | 5.35 | 8.1 | 6.1 | - | 0.9 | 0.75 |  |
| C9 |  |  |  |  |  |  |  |  |
| 2,2,5 trimethyl hexane | 124.1 | 5.65 | 5 | 8.8 | 12.7 | - | v small | v small | Ca 2 |
| 2,3,5 trimethyl hexane | 131.4 | - |  | 1.9 | 1.9 |  |  |  |  |
| C10 etc. |  |  |  |  |  |  |  |  |
| 2,2,6 trimethyl heptane |  | Ca 2.5 | 9 |  |  | - | small |  |
| Other high boiling paraffins |  | 3 |  | 1.7 | 2.4 |  |  |  |  |
| Single branch |  |  |  |  |  |  |  |  |
| 2 & 3 methyl pentanes | 60-63 | 1.45 | 1.4 | 3.9 | 2.7 |  | - | 0.1 |  |
| 2 & 3 methyl hexanes | 69-91 | 0.4 | 0.2 | 1.7 | 0.9 |  | - | 0.1 | 0.4 |
| Butanes & pentanes | <40 | 4.4 | 18.7 | 18.7 | 20.0 |  | 1 |  | 1 |

A.P.I. N.B.S (American Petroleum Institute – National Bureau of Standard) analyses by Augustus R. Glasgow by fractionation and analysis by reference index.

All other figures for Aruba and Baton Rouge alkylates, Aruba and Heysham hydrocodimers, determined by fractionation and analysis by infra-red spectroscopy

D.	German Aviation Gasolines – C3 Green Type

14 samples have been fractionated and analysed by aniline point method which gives fairly satisfactory figured for total 2,2,5 T.M.P. and total octanes 105-110OC (2,2,3  2,2,4 and 2,2,3 etc.)

5 Samples have been analysed in detail by infra-red methods applies to separate aromatic–free fractions and also applied to one sample freed from toluene by azeotropic distillation.

4 samples have been analysed without or with only rough fractionation.

Spectroscopic analysis of individual fractions is the best method. But the other data is included as confirmatory evidence.

Table VI

Table 17. Analysis of TMP Content of various captured samples.

| Sample |  |  | Fine Fractionation & Aniline Point Method |  |  |  |  | Fine Fractionation & Infra-red analysis |  |  |  | Uncut or rough Fractionation & Infra-red |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Date | 2,2,4 TMP |  | Higher TMP | Total TMP’s | %2,2,4 in TMP | 2,2,4 TMP |  | 2,3,4 TMP etc. | Total TMP’s | %2,2,4 in TMP | 2,2,4 TMP xx | 2,3,4 TMP etc.+ | Total TMP’s | %2,2,4 in TMP |
| GF 28 | Summer ‘40 | 9 |  | 3 | 12 | 75 |  |  |  |  |  |  |  |  |  |
| 40/41 | Sept. 40 | 9 |  | 3 | 12 | 75 |  |  |  |  |  |  |  |  |  |
| GF 31 | Autumn ‘40 | 10½ |  | 3½ | 14 | 75 |  |  |  |  |  |  |  |  |  |
| GF 32 | Autumn ‘40 | 10 |  | 3½ | 13½ | 74 |  |  |  |  |  |  |  |  |  |
| AIR141 | June ‘41 | 12½ |  | 4½ | 17 | 73 | 11 |  | 5 | 16 | 69 |  |  |  |  |
| AIR144 | July ‘41 | 12 |  | 5 | 17 | 71 | 12 |  | 5 | 17 | 70 |  |  |  |  |
| AIR176 | April ‘42 | 11 |  | 4 | 15 | 74 | 11½ |  | 3½ | 15 | 76 |  |  |  |  |
| AIR189 | June ‘42 | 7 |  | 8 | 15 | 46 | 8 |  | 8 | 16 | 50 |  |  |  |  |
| AIR285 | May ‘43 | 13 |  | 5 | 18 | 72 | 12 |  | 6 | 18 | 66 |  |  |  |  |
| AIR287 |  |  |  |  |  |  |  |  |  |  |  | 12 | 6.1 | 18.1 | 66 |
| AIR291 |  |  |  |  |  |  |  |  |  |  |  | 13.3 | 6.1 | 19.4 | 68 |
| AIR305 | June ‘43 | 12½ |  | 7 | 19½ | 64 |  |  |  |  |  |  |  |  |  |
| AIR311 | June ‘43 | 13½ |  | 7 | 20½ | 66 |  |  |  |  |  |  |  |  |  |
| AIR312 | June ‘43 |  |  |  |  |  |  |  |  |  |  | 12 | 5 | 17 | 71 |
| AIR317 | Middle East | 13½ |  | 7 | 20½ | 66 | 13.0x |  | 6.0x | 19.0x | 68½ x |  |  |  |  |
| AIR318, 321, 327 | Middle East | 13½ -14 |  | 6½ | 20½ | 67 |  |  |  |  |  |  |  |  |  |
| AIR328-330 | Middle East | 13½ |  | 7 | 20½ | 66 |  |  |  |  |  |  |  |  |  |
| AIR331 | Middle East |  |  |  |  |  |  |  |  |  |  | 12 | 5.5 | 17.5 | 68 |

xx	Included 2,2 dimethyl hexanes (ca. 0.9%) with 2,2,4 Trimethyl Pentane

+	Included 2,2,3, & 2,3,3, Trimethyl Pentanes with 2,3,4 Trimethyl Pentane

x	On material freed from toluene by azeotropic distillation.

Table VII

COMPOSITION OF TRIMETHYL PENTANES (Infra-red Method)

Table 18. TMP Composition

| Fuel | AIR141 | AIR144 | AIR176 | AIR189 | AIR285 | AIR317x |
| --- | --- | --- | --- | --- | --- | --- |
| Vol.% on Fuel |  |  |  |  |  |  |
| 2,2,4 TMP | 11.0 | 12.0 | 11.5 | 8.0 | 12.1 | 13.05 |
| 2,2,3 TMP | - | Trace? | - | - | 0.6 | 1.2 |
| 2,3,4 TMP | 5.0 | 5.0 | 4.0 | 8.0 | 4.6 | 4.1 |
| 2,3,3 TMP | Trace? | - | 0.5 | - | 0.6 | 0.7 |
| 2,2 Dimethyl Hexane |  |  |  |  |  | 0.25 |
| 2,4 Dimethyl Hexane |  |  |  |  | 0.33 | 0.1 |
| 2,3 Dimethyl Hexane |  |  |  |  |  | 0.23 |
| 2,2,5 Trimethyl Hexane |  |  |  |  | absent |  |

x	On material freed from toluene by azeotropic distillation. (Ref. 81)

Table VIII

DETAILED COMPOSITION OF PARAFFINS OF FUEL AIR285 IN COMPARISON WITH KNOWN OCTANES.

(Fractionated in 40-plate still and toluene removed by acid washing)

Table 19. AIR285 Composition

|  | Vol. % on fuel | Vol. % on added octane (calc.) | C4 Alkylate | C4 Hot Acid (U.O.P.) | C4 Cold Acid |
| --- | --- | --- | --- | --- | --- |
| C8 |  |  |  |  |  |
| 2,2,4 Trimethyl pentane | 12.07 | 55 | 20-30 | 30 | 90 |
| 2,2,3 Trimethyl pentane | 0.62 | 3 | 1 | 10 |  |
| 2,3,4 Trimethyl pentane | 4.60 | 21 | 10-15 | 35 | 5-10 |
| 2,3,3 Trimethyl pentane | 0.56 | 2.5 | 15-20 | 5 |  |
| 2,4 Dimethyl hexane | 0.33 | 1½ | 8-10 | <2 |  |
| 2,2 Dimethyl hexane | Not detected | 0 |  |  |  |
| C7 |  |  |  |  |  |
| 2,3 Dimethyl pentane | 1.82 | 8-9 | 2 | 10 |  |
| 2,2 Dimethyl pentane | Not detected | 0 | 4 | 2-3 |  |
| 2,4 Dimethyl pentane | Not detected | 0 |  |  |  |
| 3,3 Dimethyl pentane | 0.14 | ? | 0 | 0 |  |
| 2 Methyl hexane | 0.69 | Nil (?) | 0.5 | 0 |  |
| 3 Methyl hexane | 0.50 | Nil (?) |  |  |  |
| C6 |  |  |  |  |  |
| 2,3 Dimethyl butane | 0.95 | 4 (?) | 4-5 | 1 |  |
| 2,2 Dimethyl butane | 0.1 | 0.5 (?) |  |  |  |
| 2 Methyl pentane | 0.97 | Nil (?) | 2 | 0 |  |
| 3 Methyl pentane | (0.4+) | Nil (?) |  |  |  |
| C9 |  |  |  |  |  |
| 2,2,5 Trimethyl hexane | Not detected | 0 | 5 | <1 |  |
| 2,2 branched paraffins | Present (say) |  |  |  |  |
| B.P. 175OC (hydro-trimers) | (say 2½ %) | (say 5%) |  | May be present |  |
|  | Say 22% | 100% |  |  |  |

Fig 1. Formula of 2,2,4 Trimethyl Pentane

Discussion and Conclusions

It is concluded from the small amounts of dimethyl hexanes and the preponderance of 2,3,4 trimethyl pentane with relatively little 2,3,3 trimethyl pentane and the absence of any 2,2,5 trimethyl hexane that the added octanes are not alkylate.

The preponderance of 2,2,3 trimethyl pentane and 2,3 dimethyl hexane as well as the presence of more 2,3 dimethyl butane and 2,3 dimethyl pentane than would be expected to be in the base fuel, all point to the material being a hydrocodimers. This is also supported by the presence of about 2% of multi-branched paraffins boiling about 175-180OC which is probably a small amount of hydrogenated trimer.

The close similarity with hydrocodimer is brought out in Table VIII above where it would appear that fuel AIR285 contains about 22% of octane material, 81% of which is actually trimethyl pentanes.

The main difference between this material and the Heysham octane is that the German material is richer in 2,2,4 trimethyl pentane and is thus nearer to a cold acid octane.

It is suggested that the process used is actually involving phosphoric acid type catalyst operated at a low temperature and high throughput with incomplete conversion and a high proportion of 2,2,4 TMP and a low make of trimer, so that the whole product and be hydrogenated directly and put into the green fuel together with the 10-15% trimer. The unpolymerised olefines possibly with the other olefines and propanes, butanes and pentanes are then passed to alkylation to yield a fairly poor quality alkylate which goes to the B4 blue aviation fuel.

It is possible that this process uses butanes from the hydro-aromatisation process which operated at high temperature would be richer in normal butane. It is also possible that the addition of a dehydrogenation (hydroforming?) process to the aromatised naphtha (as had been suggested for recent samples of green fuel) would produce more butanes and so increases the availability of octane.

Figure 2. Formula of 2,2,3 Trimethyl Pentane

Figure 3. Formula of 2,3 Dimethyl Hexane

# Appendix IV

COMPOSITION OF COMPLETE FUEL – POSSIBLE MAKE-UP ANALYSIS:

Table IX gives details of the distribution of paraffins and naphthene hydrocarbons in fourteen samples of the green fuel as determined by the method of fractionation in 30-40 plate columns and analysis of the ‘smooth-curve’ aniline point method.

The absolute accuracy of such a method is not high due to deviations of the aniline points of actual paraffins and naphthenes present in narrow cuts from the smooth curve relating average aniline point and boiling point. The magnitude of the errors introduced by this method and other factors tending to cause incorrect results are discussed in Appendix V. The results are, however, reasonably consistent and reproducible and hence marked differences in composition will have a real meaning.

INTERPRETATION

To interpret the results in a comparative sense it is necessary to consider the results of similar analyses of a wide variety of products from known sources. These analyses, which include some 20 natural petroleum gasolines, a wide variety of hydrogenation gasolines, and a number of other products are to be tabled in detail in a future report. The outstanding points about the composition of the non-aromatic hydrocarbons in the earlier samples of the green fuel emphasised in Table IX and X are:

Large amounts of paraffins distilling in the range 97-112OC and shown to include about 15-20% of trimethyl pentanes (Table IX).

Low content of pentanes and hexanes as compared with most natural gasolines from petroleum or ‘normal’ hydrogenation petrols (Table X).

Relatively high proportion of normal paraffins in the pentanes and hexanes (this is really a deficiency in iso-paraffins).

Relatively small amounts of paraffins boiling in the range 113-153OC (methyl heptanes, n-octanes, nonanes, etc.) and an unusually high proportion of paraffins distilling at about 175-180OC (probably hydrogenated trimer).

Normal content of C6 naphthenes as compared with natural petroleum gasolines, and also a similar ratio of methyl cyclopentane to cyclohexane (approx. 1-1.5 to 1), but a deficiency of methyl cyclopentane and excess of cyclohexane when compared with medium temperature ‘normal’ vapour phase hydrogenation gasolines (ratio 6:1 of higher).

Deficiency of naphthenes boiling at 90OC (dimethyl cyclopentanes) when compared with ‘normal’ hydrogenation gasolines or with such straight run petroleum spirits as 73 octane Roumanian (Table X).

Octane Content and Composition of octane –free base naphtha

Fractionation and spectroscopic analysis of fractions 90-125OC has shown (Appendix III) that the green fuel has always contained from 13% to 20% of trimethyl pentanes. Detection and estimation of smaller amounts of other paraffins which probably originate in the synthetic octane added to the base fuel suggest that some 16% to 24% by volume of total synthetic octane material has been added (containing about 80% trimethyl pentanes).

There have been progressive changes in composition with small increases in octane content, which are discussed more fully below, but generally roughly four variations in the composition can be distinguished. Combining all the analytical evidence it is possible to allow for the synthetic octane components added and deduce the composition of the base gasoline to which this has been added.

Table X gives figures obtained in this way for the composition of the variations in base material (octane-free) in comparison with 73 octane virgin Roumanian gasoline and with several types of hydrogenation gasolines.

The obvious points of difference between the base fuel and Roumanian or normal vapour phase hydro-petrol are:

Much higher aromatic content of green fuel base

Lower ‘front-end’ volatility

Table X also shows that whereas the green fuel compares with Roumanian or some of the lower volatility of ‘vapour phase’ type hydro-petrol as regards amount of C5 and C6 naphthenes it is very deficient in C7 and C8 naphthenes. The small amount of such naphthenes as methyl cyclohexane, dimethyl cyclohexane, and ethyl cyclohexane in what is probably a hydro product from a bituminous raw material, are considered to support the view that such naphthenes have been destroyed by dehydrogenation or were never formed as in an aromatising splitting process which largely produces aromatics.

The comparative absence of such naphthenes is also considered as evidence against the supposition that green fuel consists of a blend of aromatics added to a base fuel of lower aromatic content and more normal naphthene distribution.

If the composition of the base material is expressed on the basis of absence of synthetic octane and aromatics, it is apparent that the distribution of the remaining 45-40% of paraffins and naphthenes is very different from either Roumanian or normal hydro petrols in that the proportion of C6 paraffins and naphthenes is much higher (Table XI).

This fact of the presence of a fairly normal amount of C6 hydrocarbons relative to C7 and C8, and the abnormally low content of 90OC flat (iso-heptane and dimethyl cyclopentane) and the presence of an appreciable amount of cyclohexane compared with the low content of methyl cyclohexane all suggest that the lower part of the fuel has not been produced by the same process as that used for the higher boiling range.

These anomalies suggested that a low boiling fraction containing C5 and C6 hydrocarbons of end-point about 85OC was being added to a low volatility highly aromatised naphtha containing comparatively little material boiling below 95OC.

On the grounds it was suggested that the earlier type of green gasoline final blend was made up from:

25-30%	Light cut C5 and C6 hydrocarbons, mainly boiling below 85OC. Possibly with additional pentane.

55-50%	Low volatility, highly aromatised, naphtha made by destructive hydro aromatisation of a bituminous material. Such as naphtha would have a composition of about 60% aromatics, 30% naphthenes, 10% paraffins and would have a boiling range essentially 90-175OC.

5%	Refined motor benzole

20%	Synthetic octane of the phosphoric acid type, but rich in 2,2,4 TMP

It was also argued that the light-cut fraction was not made by ‘normal’* hydrogenation since:

This would yield more iso-pentane and less normal pentane

More iso-hexanes and less n-hexane would be present.

More methyl cyclopentane and much less cyclohexane would be present.

The arguments regarding proportions of iso and normal paraffins would certainly exclude the normal medium temperature hydrogenation process from supplying any appreciable portion of this fuel. These analyses are considered to be fairly reliable.

* The word ‘normal’ hydrogenation is used to distinguish the medium temperature (ca. 400OC) destructive hydrogenation carried out in the vapour phase with sensitive catalyst from the liquid phase process or high temperature ‘aromatising’.

Table 20. Comparison of various gasolines

| Vol % on Octane free basis | German Green Gasoline |  | Hydrogenation (Bituminous) |  |  |  |  |  | Roumanian 73 Octane |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1941-42 | 1943 | Medium temp. Catalytic Vapour Phase |  |  |  | Liquid Phase | ‘Aromatised’ Vapour Phase |  |
| End Point OC | 175 | 175 | 200 | 160 | 145 | 130 | 220 | 180 | 126 |
| Iso-pentane | 4 | 8 | 7 | 9 | 12½ | 18 | 2.5 | 0.8 | 4½ |
| n-pentane | 3 | 5 | 2 | 2 | 2½ | 4 | 1.7 | 1 | 3½ |
| % Iso- in Pentanes | 55 | 60 | 75 | 82 | 83 | 82 | 60 | Ca. 50 | 55 |
| Iso-hexane | 3 | 6½ | 4½ | 8½ | 11 | 11 | 2.5 | 1 | 7½ |
| n-hexane | 3 | 4½ | 1½ | 2 | 2½ | 2 | 1.4 | 1 | 3½ |
| % Iso- in hexane | 50 | 60 | 90 | 83 | 82 | 85 | 65 | Ca. 50 | 70 |
| Methyl cyclopentane (MCP) | 6 | 6½ | 6½ | 10 | 13½ | 12.2 | 2.8 | 3 | 5½ |
| Cyclohexane | 6 | 4½ | 2 | 1.8 | 2 | 2.3 | 1.4 | 5 | 5½ |
| % MCP in C6 naphthenes | 50 | 60 | 80 | 85 | 87 | 85 | 65 | 40 | 47 |
| % Aromatics | 45 | 50 | 6½ | 5½ | 4 | 13 | 30 | 38 | 7 |
| % Naphthenes | 34 | 20 | 63 | 57½ | 50 | 40 | 62 | 53 | 48½ |
| % Paraffins | 21 | 30 | 30½ | 37 | 46 | 47 | 8 | 9 | 44½ |

The estimation of methyl cyclopentane is also considered satisfactory (Appendix V), but the aniline point method tends to over-estimate cyclohexane, and this error is increased by anomalous behaviour on distillation in the presence of large amounts of benzene. Even as there is certainly more cyclohexane present than in ‘normal’ medium temperature hydro-petrols particularly in the earlier samples of the green aviation fuel.

It has already been pointed out (Appendix II) that more benzene is present than would be expected (unless this is formed from phenol in the raw material). A probable explanation is that since benzol is being added to many other types of fuels some will also go into the aviation fuel. Motor benzole may be added to the raw material being treated for hydro-aromatisation in such a way that the benzole is refined and also partially hydrogenated to cyclohexane. This explanation would account for the larger ratio of C6 naphthenes to C6 paraffins than is normally the case.

The earlier type of green gasoline contains more naphthenes than paraffins (when corrected for added octane) and if it is also assumed that the fraction up to 95OC, if from a different source the aromatised naphtha fraction 95-175OC will have some such composition as 61% aromatics, 31% naphthenes and only 8½% paraffins and is similar to the low volatility product from high temperature “aromatisation” of creosote. (Hydro gasoline (4) in Table X).

The absence of any appreciable amounts of alkyl cyclopentanes as compared with normal medium temperature hydro-petrols is also considered to support this view of the process used. The naphthenes remaining in the aromatised naphtha will be largely alkyl cyclohexanes formed by direct hydrogenation of aromatics with little or none of the isomerisation to cyclopentanes which is so characteristic of medium temperature splitting hydrogenation processes.

Table 21. Cyclopentanes

|  | Green fuel (octane free) | Typical medium temperature hydro-petrol |
| --- | --- | --- |
| Methyl cyclopentane | 6 | 13 |
| Dimethyl cyclopentane | 1½ | 6 |
| Ethyl cyclopentane | ½ | 2 |

Little information is available regarding the composition of hexanes and pentanes made from higher temperature high pressure destructive aromatisation, but it is known that such conditions and also operation of liquid phase processes with less selective catalysts would yield products with higher proportions of cyclohexane relative to methyl cyclopentane and normal paraffins relative to iso-paraffins (see table above and Table X). It is thus possible that such high temperature or liquid phase hydrogenation processes would provide the whole of the base material in the green fuel, although it is still considered that light-cut material from another source is being added to increase the front end volatility of the aromatised product.

Figure 4. Formula of Cyclopentane

Changes in Composition of Non-Aromatics in German Green C-3 Aviation Gasolines.

Table IX lists all the samples examined and shows that several changes have occurred. These are:

End of 1940

End-point reduced from 185OC to 175OC by cutting back the aromatised naphtha. The mainly had the effect of reduction the amount of high boiling aromatics without much change in composition of non-aromatics although octane content was raised from about 16% to about 19%.

1941-1942

Little change, apart from slight further increase in octane content.

During early part of 1943

A number of progressive changes occurred:

Further increase in octane content by 5% up to 24% vol.

Increase in pentane content from 5-6% vol. up to 10% vol., and also an increase in hexane content from about 4% vol. up to about 8% vol.

These changes at first resulted in a reduction in aromatic content due to the dilution effect of adding 10-15% more material, but later during 1943 the aromatic content was restored to the previous figure of about 36% vol. and this last change was accompanied by:

Marked decrease of 8-10% in the amount of higher boiling naphthenes, and considerable change over in the composition of the aromatics, away from such mono alkyl benzenes as ethyl, n-propyl and n-butyl towards the meta poly methyl benzenes such as m-xylene and m-ethyl toluene.

These changes are clearly shown in the following table:

Table XII CHANGES IN COMPOSITION OF GREEN AVIATION FUELS

Table 22. Changes in Green aviation fuel composition 1942-1943

| Date | 1941-42 | 1943 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample No. | AIR176 | AIR285 | AIR305 | AIR311 | AIR317 | AIR318 | AIR328 |
| Knock Ratings: |  |  |  |  |  |  |  |  |
| Weak (Lean rating) | 95 | 96 | 96 ½ | 97 | 97 | 97 | 97 |
| Rich rating, % Grade 130 | Ca. 110 | 118 ½ | 124 | >125 | >125 | >125 | >125 |
| Vol.% octane material (total) | 18 | 21 | 22 | 24 | 25 | 25 | 25 |
| Vol.% pentane (+ butane) | 6.55 | 9.45 | 10.2 | 10.0 | 10.5 | 10.25 | 9.95 |
| Vol.% hexanes | 4.45 | 5.85 | 6.2 | 8.3 | 8.4 | 7.85 | 8.1 |
| Vol.% total aromatics | 37.4 | 32.9 | 32.45 | 33.95 | 36.3 | 36.55 | 35.75 |
| Total naphthenes vol.% | 26.9 | 25.65 | 23.5 | 18.8 | 16.8 | 16.6 | 17.6 |
| Boiling below 95OC | 11.2 | 12.6 | 12.1 | 11.15 | 10.4 | 9.9 | 10.95 |
| Boiling 95OC-180OC | 15.7 | 13.0 | 11.3 | 7.65 | 6.4 | 6.7 | 6.7 |
| Aromatics below 95OC (benzene) | 8.5 | 7.5 | 7.35 | 5.75 | 6.5 | 7.1 | 6.65 |
| Cyclohexane | 5.3 | 4.25 | 3.75 | 2.95 | 3.25 | 2.75 | 3.65 |
| Methyl cyclopentane | 3.9 | 5.8 | 5.9 | 5.8 | 5.1 | 5.05 | 5.1 |
| Cyclopentane | 0.85 | 1.0 | 0.75 | 1.0 | 0.9 | 1.0 | 1.0 |
| Aromatics above 95OC | 28.9 | 25.4 | 25.1 | 28.2 | 29.8 | 29.45 | 29.1 |
| C7 | 12.5 | 10.05 | 10.4 | 11.0 | 12.5 | 12.2 | 11.65 |
| C8 | 9.8 | 8.65 | 8.0 | 9.6 | 10.0 | 10.4 | 10.5 |
| C9+C10 | 6.6 | 6.7 | 5.9 | 7.6 | 7.3 | 6.85 | 6.95 |
| Naphthenes above 95OC | 15.7 | 13.0 | 11.3 | 7.65 | 6.4 | 6.7 | 6.7 |
| C7 | 4.4 | 2.35 | 2.8 | 1.8 | 1.6 | 1.85 | 1.8 |
| C8 | 4.9 | 4.5 | 3.7 | 1.75 | 2.2 | 2.2 | 2.3 |
| C9+C10 | 6.4 | 6.15 | 4.9 | 3.1 | 2.6 | 2.65 | 2.6 |
| Ethyl benzene | 6.2 | 4.2 |  | 2.45 | 3.5 |  |  |
| m-xylene | 2.9 | 2.5 |  | 3.35 | 4.15 |  |  |
| p-xylene | 0.7 | 0.7 |  | 1.25 | 1.2 |  |  |
| o-xylene | 0 | 0.95 |  | 1.85 | 1.7 |  |  |
| Ethyl benzene % on C8 aromatics | 63 | 50 |  | 28 | 33 |  |  |
| n-propyl benzene | 2.4 | 0.85 |  | 0.9 | 0.6 |  |  |
| Ethyl toluenes | 1.6 | 2.25 |  | 2.05 | 2.8 |  |  |

The most interesting changes are those occurring early in 1943 when apparently a further manufacturing step was introduced to dehydrogenate the higher boiling naphthenes present in the aromatised naphtha by some such process as hydroforming or possibly recycle of the aromatised naphtha.

Most of this decrease in naphthene content has occurred in the range above 115OC:

Table 23.  Changes due to Dehydrogenation of naphthenes

|  | >115OC fraction of recent green fuels |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1942 | 1943 |  |  |
| Vol.% of fuel >115OC (T.B.P.) | 33 | 30 | 2.9 | 27½ |
| Composition % on >115OC fraction |  |  |  |  |
| Aromatics | 51 | 52 | 56½ | 64 |
| Naphthenes | 32 | 32 | 27 | 17 |
| Paraffins | 17 | 16 | 16½ | 19 |

It should be noted that when 5% more octane and 8% more pentanes and hexanes are present in the most recent 1943 fuels there has actually been a decrease in cyclohexane and in benzene contents. This may be a dilution effect or may indicate that less benzol is being added and that cyclohexane is now being dehydrogenated in the last process. The increase in pentanes and hexanes with a decrease in benzene and cyclohexane would appear to confirm the view that the light cut is a separate component of the blend.

(Changes in true boiling point distribution on fractionation are also apparent. These are discussed in Appendix V and Table XIII).

CONCLUSION:

The most recent type of green fuel is now considered to be made up from the following components: Vol.%

29%	Light cut, possible from hydrogenation providing most of the pentanes, hexanes, and methyl cyclopentane and boiling up to 85OC. This cut would have composition 7% benzene, 29% naphthenes, 64% paraffins.

5%	Hydro-refined benzole, providing 4 parts of benzole and possibly 1 part of the cyclohexane.

42%	Doubly aromatised naphtha. Made in two stage process:

By high temperature destructive aromatising hydrogenation of a bituminous raw material to give a low volatility highly aromatic naphtha, possibly of some such composition as

61% Aromatics

31% Naphthenes

8% Paraffins

and boiling 95-175OC with little below 95OC

This aromatised naphtha is given a second aromatising treatment (probably different conditions) or is hydro-formed (dehydrogenated) whereby the composition becomes:

74% Aromatics

16% Naphthenes

10% Paraffins

A certain amount of gas and low boiling hydrocarbons made in this process may supplement components (1) and (4) (see Ref. 40).

24%	Synthetic octane material made by low temperature operation of phosphoric acid process. The composition is approx.:

80% Trimethyl pentanes

12% other paraffins

8% Hydro trimer

2,2,4 trimethyl pentane from 70% and 2,3,4 trimethyl pentane about 22-25% of the total trimethyl pentanes.

Table IX

NON-AROMATIC HYDROCARBON COMPOSITION OF 14 SAMPLES OF C3 (GREEN) TYPE OF ENEMY AVIATION GASOLINE

Table 24. Non-Aromatic content of German aviation gasolines (Analysis by fractionation and aniline point method. Volume %)

| Probable Chief Component | Range OC | GF28 | 40/41 | GF31 | GF32 | AIR141 | AIR144 | AIR176 | AIR189 | AIR285 | AIR305 | AIR311 | AIR317 | AIR318-27 | AIR328-30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1940 |  |  |  | 1941 |  | 1942 |  | 1943 |  |  |  |  |  |
| Paraffins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C4 Iso-butane | POD analysis | 0 | 0 | 0 | 0 | 0 | 0 | 0.25 | 0 | 0 | 0.05 | 1.1 | 0 | 0 | 0 |
| C4 normal butane |  | 1.1 | 2.05 | 1.2 | 1.8 | 2.1 | 2.1 | 0.7 | 1.55 | 1.65 | 2.55 | 3.45 | 0 | 0.9 | 0.45 |
| C5 Iso-pentane |  | 3.0 | 5.0 | 2.75 | 3.6 | 2.55 | 3.0 | 2.35 | 3.6 | 4.35 | 4.8 | 3.0 | 6.9 | 5.3 | 5.95 |
| C5 normal pentane |  | 2.3 | 2.8 | 1.6 | 2.3 | 2.4 | 1.75 | 3.25 | 2.35 | 3.5 | 2.75 | 2.45 | 3.6 | 4.05 | 3.55 |
| C6 methyl pentanes | 40-65 | 3.4 | 3.95 | 3.2 | 3.2 | 1.3 | 2.15 | 1.55 | 2.5 | 2.9 | 3.5 | 5.1 | 5.1 | 4.6 | 4.7 |
| C6 normal hexane | 65-80 (75) | 2.6 | 2.45 | 2.55 | 2.55 | 2.7 | 2.15 | 2.9 | 1.8 | 2.9 | 2.7 | 3.2 | 3.3 | 3.25 | 3.4 |
| C7 2,2 & 2,4 dimethyl pentanes | (75-85) |  |  |  |  |  |  |  |  | 0.45 | 0.7 | 0.6 | 0.6 | 1.0 | 0.6 |
| C7 methyl hexanes | 80 (85)-95 | 4.1 | 3.45 | 3.4 | 3.55 | 2.15 | 2.45 | 2.6 | 1.95 | 3.0 | 3.75 | 3.4 | 3.2 | 3.4 | 3.35 |
| C7 normal heptane | 95-102 | 9.75 | 9.9 | 11.4 | 10.45 | 13.35 | 13.1 | 11.9 | 7.95 | 12.95 | 11.9 | 12.9 | 13.1 | 13.35 | 13.45 |
| C8 2,2,4 trimethyl pentane |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C8 dimethyl hexanes (& TMP’s) | 102-113 | 4.1 | 3.55 | 4.5 | 4.55 | 5.3 | 6.55 | 4.75 | 8.9 | 4.8 | 6.3 | 5.9 | 5.75 | 5.45 | 6.05 |
| C8 methyl heptanes | 113-123 | 1.25 | 1.4 | 1.15 | 1.4 | 0.3 | 1.0 | 0.75 | 2.1 | 0.65 | 0.7 | 0.9 | 0.5 | 0.4 | 0.3 |
| C8 n-Octane (& TM hexanes) | 123-129 | 0.4 | 0.45 | 0.55 | 0.55 | 0.15 | 0.35 | 0.25 | 0.4 | 0.1 | 0.3 | 0.5 | 0.4 | 0.3 | 0.4 |
| C9 | 129-137 | 1.05 | 0.75 | 1.0 | 1.0 | 0.6 | 0.55 | 0.8 | 0.4 | 0.8 | 0.5 | 0.9 | 0.65 | 0.95 | 1.1 |
| C9 | 137-153 | 0.4 | 0.45 | 0.55 | 0.35 | 0.4 | 0.2 | 0.35 | 0.4 | 0.5 | 0.5 | 0.6 | 0.8 | 0.8 | 0.4 |
| C10 (some hydro trimer) etc. | >153 | 2.25 | 2.6 | 4.15 | 3.7 | 1.6 | 0 | 3.3 | 1.45 | 2.95 | 3.05 | 3.25 | 3.0 | 3.05 | 2.9 |
| Total Paraffins |  | 35.7 | 38.8 | 38.0 | 39.0 | 34.9 | 35.35 | 35.7 | 35.35 | 41.5 | 44.05 | 47.25 | 46.9 | 46.85 | 46.6 |
| Naphthenes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C5 cyclopentane | 40-60 | 0.65 | 0.65 | 0.75 | 0.7 | 0.7 | 0.75 | 0.85 | 0.7 | 1.0 | 0.75 | 1.0 | 0.9 | 1.0 | 1.0 |
| C6 methyl cyclopentane | 60-75 | 5.1 | 5.95 | 5.0 | 5.5 | 3.5 | 5.45 | 3.9 | 4.1 | 5.8 | 5.9 | 5.8 | 5.1 | 5.05 | 5.1 |
| C6 cyclohexane | 75-84 | 5.3 | 4.05 | 4.0 | 4.5 | 5.35 | 4.55 | 5.3 | 5.35 | 4.25 | 3.75 | 2.95 | 3.25 | 2.75 | 3.65 |
| C7 dimethyl cyclopentane | 84-95 | 2.4 | 2.0 | 1.55 | 1.75 | 0.85 | 0.95 | 1.15 | 0.9 | 1.6 | 1.7 | 1.4 | 1.2 | 1.1 | 1.2 |
| C7 methyl cyclohexane + etc. | 95-110 | 5.15 | 4.0 | 3.3 | 3.4 | 4.2 | 3.65 | 4.45 | 4.65 | 2.35 | 2.75 | 1.8 | 1.6 | 1.85 | 1.8 |
| C8 dimethyl hexanes + etc. | 110-125 | 2.8 | 2.25 | 2.3 | 2.05 | 1.0 | 1.9 | 1.9 | 1.55 | 2.4 | 1.85 | 1.3 | 1.1 | 1.05 | 1.15 |
| C8 ethyl cyclohexane etc. | 125-137 | 3.1 | 3.25 | 2.25 | 2.5 | 3.65 | 2.65 | 3.0 | 2.95 | 2.1 | 1.85 | 1.45 | 1.1 | 1.15 | 1.15 |
| C9 | 137-153 | 1.95 | 1.85 | 1.5 | 1.2 | 1.85 | 1.45 | 1.55 | 1.65 | 2.0 | 1.2 | 0.8 | 0.4 | 1.2 | 0.7 |
| C10+ | >153 | 4.45 | 3.75 | 4.0 | 3.0 | 6.3 | 5.8 | 4.85 | 4.15 | 4.15 | 3.75 | 2.3 | 2.15 | 1.45 | 1.9 |
| Total Naphthenes |  | 30.9 | 28.65 | 24.65 | 25.5 | 27.4 | 27.15 | 26.9 | 26.0 | 25.65 | 23.5 | 18.8 | 16.8 | 16.6 | 17.65 |

Table X

COMPOSITION OF TYPICAL GERMAN GREEN GASOLINES COMPARED WITH NATURAL PETROLEUM AND HYDROGENATION GASOLINES

(Figures mainly by aniline point method – supplemented by spectroscopic methods)

Table 25. Composition of German aviation gasoline with other gasolines

| Probable components | Type | True B.P.OC |  |  | B.P. Range in Fraction |  | German Green C3 Gasolines |  |  |  |  |  |  |  |  |  | Natural Roumanian 73 Oct. 130OC End Point | Hydrogenation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 1940 | 1941-42 |  |  |  |  | 1943 |  |  |  | From Type FBP OC | Creosote Middle Oil |  |  |  |  |  |  |  |  |  |  |  |  |  | Creosote Heavy Oil Liq. Phase |  | Petroleum Gas Oil. Med. Temp. V.P. |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Early |  |  | Late |  | Med. Temp. Normal V.P. |  |  |  |  |  |  |  |  | High Temp. V.P. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 135* | 160 |  |  |  | 200 |  |  |  | 180 |  |  |  |  | 220 |  | 135* |  |  |
| Estimated octanes added as vol.% of final blend |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ref No. | (1) |  | (2) |  |  |  | (3) |  |  |  | (4) |  |  |  | (5) |  | (6) |  |  |
| 2,3 dimethyl pentane etc. | C7 |  |  |  | 88-95 |  | 1 | 1 |  |  |  |  | 1½ |  |  | 2 | - | - | - |  |  |  | - |  |  |  | - |  |  |  |  | - |  | - |  |  |
| 2,2,4 trimethyl pentane | C8 |  |  |  | 95-110 |  | 9½ | 12 |  |  |  |  | 12½ |  |  | 13½ | - | - | - |  |  |  | - |  |  |  | - |  |  |  |  | - |  | - |  |  |
| 2,3,4 trimethyl pentane | C8 |  |  |  | 105-112 |  | 3½ | 5 |  |  |  |  | 5 |  |  | 5½ | - | - | - |  |  |  | - |  |  |  | - |  |  |  |  | - |  | - |  |  |
| Hydro trimer (dodecanes) | C12 |  |  |  | Ca. 175 |  | 2½ | 1 |  |  |  |  | 2 ½ |  |  | 2 | - | - | - |  |  |  | - |  |  |  | - |  |  |  |  | - |  | - |  |  |
|  | Total |  |  |  |  |  | 16½ | 19 |  |  |  |  | 21½ |  |  | 23½ | Nil | nil | nil |  |  |  | nil |  |  |  | nil |  |  |  |  | nil |  | nil |  |  |
| Calculated vol.% composition of octane-free base: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Iso-butane | C4 | -11.5 |  |  | 0-40 |  | 0 | 0 |  |  |  |  | 0 |  |  | 0 | 0.6 | 0 | 0.85 |  |  |  | 4.05 |  |  |  | 0.75 |  |  |  |  | 1.55 |  | 0 |  |  |
| n-butane | C4 | -0.5 |  |  |  |  | 2.0 | 2.2 |  |  |  |  | 2.3 |  |  | 0.65 | 1.3 | 0.2 | 0.8 |  |  |  | 2.5 |  |  |  | 0 |  |  |  |  | 2.35 |  | 0 |  |  |
| Iso-pentane | C5 | 28.0 |  |  |  |  | 4.25 | 3.3 |  |  |  |  | 5.8 |  |  | 7.9 | 4.35 | 18.0 | 9.05 |  |  |  | 7.1 |  |  |  | 0.8 |  |  |  |  | 3.5 |  | 23.3 |  |  |
| n-pentane | C5 | 36.1 |  |  |  |  | 2.7 | 2.9 |  |  |  |  | 4.0 |  |  | 5.15 | 3.35 | 4.5 | 2.0 |  |  |  | 2.15 |  |  |  | 0.95 |  |  |  |  | 2.2 |  | 3.35 |  |  |
| Cyclopentane | C5 | 49.3 |  |  | 40-57 |  | 0.85 | 0.95 |  |  |  |  | 1.15 |  |  | 1.3 | 0.6 | 0.9 | 0.55 |  |  |  | 0.35 |  |  |  | 0.7 |  |  |  |  | 0.6 |  | 0.9 |  |  |
| Methyl pentane | C6 | 58-63 |  |  | 57-65 |  | 4.15 | 2.1 |  |  |  |  | 4.1 |  |  | 6.3 | 7.65 | 10.8 | 8.55 |  |  |  | 4.45 |  |  |  | 1.15 |  |  |  |  | 2.5 |  | 14.95 |  |  |
| n-hexane | C6 | 68.8 |  |  | 65-75 |  | 3.0 | 3.15 |  |  |  |  | 3.55 |  |  | 4.3 | 3.4 | 2.1 | 1.85 |  |  |  | 1.55 |  |  |  | 1.05 |  |  |  |  | 1.5 |  | 2.0 |  |  |
| Methyl cyclopentane | C6 | 71.8 |  |  | 65-75 |  | 6.3 | 5.3 |  |  |  |  | 7.45 |  |  | 6.65 | 4.9 | 13.2 | 10.15 |  |  |  | 6.55 |  |  |  | 2.85 |  |  |  |  | 2.9 |  | 8.9 |  |  |
| Cyclohexane | C6 | 80.8 |  |  | 75-84 |  | 5.5 | 6.3 |  |  |  |  | 5.05 |  |  | 4.2 | 5.45 | 2.5 | 1.85 |  |  |  | 2.1 |  |  |  | 5.55 |  |  |  |  | 1.5 |  | 1.1 |  |  |
| 2.2 & 2, 4 dimethyl pentane | C7 | 79-80 |  |  | 75-84 |  | trace | small |  |  |  |  | 0.4 |  |  | 0.2 | ? | small | trace |  |  |  | small |  |  |  | trace |  |  |  |  | small |  | 0.45 |  |  |
| Benzene | C6 | 80.1 | 65-90 |  |  | 7.65 |  |  | 11.0 | 9.45 |  |  |  | 8.8 |  |  | 1.4 | 4.4 | 1.65 |  |  | 1.25 |  |  |  | 4.5 |  |  |  | 1.8 |  |  |  |  |  | 1.4 |
| Methyl hexanes | C7 | 89-92 |  | 84-95 |  | 2.7 |  |  | 1.75 |  | 2.2 |  |  |  | 1.5 |  | 8.3 | 5.4 | 5.55 |  |  | 3.3 |  |  |  | 0.6 |  |  |  |  | 2.25 |  |  |  |  | 8.65 |
| Dimethyl cyclopentanes | C7 | 90-92 |  | 84-95 |  | 2.0 |  |  | 1.25 |  | 2.1 |  |  |  | 1.6 |  | 6.45 | 5.4 | 6.95 |  |  | 4.0 |  |  |  | 1.65 |  |  |  |  | 1.85 |  |  |  |  | 6.2 |
| n-heptane | C7 | 98.4 |  | 95-102 |  | 0.9 |  |  | 0.8 |  | 0.6 |  |  |  | 0.5 |  | 1.4 | 0.25 | 0.25 |  |  | 0.3 |  |  |  | 0.75 |  |  |  |  | 0.7 |  |  |  |  | 0.6 |
| Methyl cyclohexane (+ ethyl cyclopentane) | C7 | 101-103 |  | 95-110 |  | 5.7 |  |  | 5.1 |  | 3.25 |  |  |  | 2.2 |  | 18.25 | 10.15 | 9.95 |  |  | 7.95 |  |  |  | 12.55 |  |  |  |  | 4.0 |  |  |  |  | 9.5 |
| Dimethyl hexanes, trimethyl cyclopentanes | C8 | Ca. 108 |  | 102-111 |  | 0.9 |  |  | 0.75 |  | 0.75 |  |  |  | 0.65 |  | 3.6 | 1.75 | 1.1 |  |  | 0.55 |  |  |  | 1.05 |  |  |  |  | 1.2 |  |  |  |  | 3.5 |
| Toluene | C7 | 110.6 |  | 90-118 |  | 9.55 |  |  | 14.9 |  | 13.0 |  |  |  | 15.85 |  | 5.0 | 5.5 | 1.75 |  |  | 1.2 |  |  |  | 9.3 |  |  |  |  | 4.05 |  |  |  |  | 4.4 |
| Methyl heptanes | C8 | 117-120 |  | 111-123 |  | 1.7 |  |  | 1.2 |  | 1.0 |  |  |  | 0.55 |  | 8.0 | 2.15 | 3.55 |  |  | 2.3 |  |  |  | 0.8 |  |  |  |  | 1.45 |  |  |  |  | 3.75 |
| Dimethyl cyclohexanes (etc.) | C8 | 120-124 |  | 110-125 |  | 2.8 |  |  | 1.95 |  | 2.7 |  |  |  | 1.5 |  | 11.0 | 5.75 | 6.75 |  |  | 5.85 |  |  |  | 3.9 |  |  |  |  | 2.7 |  |  |  |  | 4.85 |
| n-octane etc. | C8 | 125.6 |  | 123-129 |  | 0.5 |  |  | 0.4 |  | 0.4 |  |  |  | 0.5 |  | 0.9 | 0.15 | 0.25 |  |  | 0.25 |  |  |  | 0.1 |  |  |  |  | 0.95 |  |  |  | 0.3 |  |
| Ethyl cyclohexane (etc.) | C8 | 132 |  | 125-137 |  | 3.15 |  |  | 3.8 |  | 2.5 |  |  |  | 1.5 |  | 1.25 | 3.0 | 4.55 |  |  | 3.7 |  |  |  | 6.05 |  |  |  |  | 3.45 |  |  |  | 1.4 |  |
| Ethyl benzene (U.V.) | C8 | 136.4 |  | 118-145 |  |  |  |  | 7.1 |  | 5.35 |  |  |  | 4.35 |  | 0.2 | 1.75 |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |  |  | 0.2 |  |
| m-xylene (U.V.) | C8 | 138.5 |  |  |  |  |  |  | 3.3 |  | 3.2 |  |  |  | 5.25 |  | 0.3 | 0.95 |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  | 0.4 |  |
| p-xylene (U.V.) | C8 | 139.2 |  |  |  |  |  |  | 0.8 |  | 0.9 |  |  |  | 1.55 |  | 0.1 | 0.25 |  |  |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  | 0.1 |  |
| o-xylene (U.V.) | C8 | 144.5 |  |  |  |  |  |  | 0 |  | 1.2 |  |  |  | 2.3 |  | 0 | 0 |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  | 0.05 |  |
| Total C8 aromatics (aniline point) |  |  |  |  |  | 8.25 |  |  | 11.4 |  |  | 10.6 |  |  |  | 13.45 | 0.6 | 2.95 | 1.6 |  |  | 1.35 |  |  |  | 10.2 |  |  |  |  |  |  | 6.2 | 0.75 |  |  |
| C9 naphthenes | C9 |  |  | 137-153 |  | 1.65 |  |  | 2.0 |  | 1.25 |  |  |  | 1.05 |  | 0.7 | 0.7 | 6.95 |  |  | 4.85 |  |  |  | 8.75 |  |  |  |  | 3.8 |  |  |  | 1.35 |  |
| m-ethyl toluene (U.V.) | C9 | 161.5 |  | 145-166 |  |  |  |  | 1.35 |  | 2.2 |  |  |  | 2.75 |  | 0 | 0 |  |  |  | 0.2 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
| p-ethyl toluene (U.V.) | C9 | 162.1 |  |  |  |  |  |  | 0.5 |  | 0.65 |  |  |  | 0.85 |  | 0 | 0 |  |  |  | 0.05 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
| Pseudo Cumene (U.V.) | C9 | 169.2 |  |  |  |  |  |  | 0.75 |  | 1.0 |  |  |  | 1.2 |  | 0 | 0 |  |  |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
| n-propyl benzene (U.V.) | C9 | 159.2 |  |  |  |  |  |  | 2.6 |  | 1.15 |  |  |  | 0.8 |  | 0 | 0 |  |  |  | 0.2 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
| Total C9 aromatics (aniline point) |  |  |  |  |  | 4.05 |  |  | 4.7 |  |  | 4.05 |  |  |  | 4.5 | Nil | Nil | 0.45 |  |  | 0.5 |  |  |  | 9.5 |  |  |  |  |  |  | 2.7 | Nil |  |  |
| C9+C10 and higher paraffins | C9, C10 etc. |  |  | >137 |  | 2.3 |  |  | 1.6 |  | 2.15 |  |  |  | 2.0 |  | 1.5 | 1.1 | 3.0 |  | 1.5 |  |  |  | 1.2 |  |  |  | 4.8 |  |  |  |  |  | Trace |  |
| C10 and higher naphthenes | C10 |  |  | >153 |  | 4.9 |  |  | 6.9 |  | 5.05 |  |  |  | 2.5 |  | Trace | Nil | 9.9 |  | 28.1 |  |  |  | 10.7 |  |  |  | 23.1 |  |  |  |  |  | Nil |  |
| n-butyl benzene (U.V.) | C10 | 183.1 |  |  | >166 | 1.5 |  |  | 2.5 |  |  |  |  |  |  |  | 0 | 0 |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
| Hydrindenes (U.V.) | C9, C10 | 176-186 |  |  |  |  |  |  | 1.8 |  | 2.1 |  |  |  | 2.0 |  | 0 | 0 |  |  | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
| Total C10 & higher aromatics (aniline point) |  |  |  |  |  | 12.3 |  |  | 4.45 |  |  | 4.45 |  |  |  | 4.65 | nil | nil | 0.15 |  |  | 2.3 |  |  |  | 4.15 |  |  |  |  | 16.4 |  |  |  | Nil |  |
| Base fuel total vol.% Paraffins |  |  |  |  |  | 25 |  |  | 20 |  |  | 27 |  |  |  | 30½ | 44½ | 46½ | 37 |  |  | 30 |  |  |  | 9 |  |  |  |  | 25 |  |  |  | 61 |  |
| Base fuel total vol.% Naphthenes |  |  |  |  |  | 33 |  |  | 33½ |  |  | 30½ |  |  |  | 22½ | 48½ | 40½ | 57½ |  |  | 63½ |  |  |  | 53 |  |  |  |  | 44 |  |  |  |  | 32.5 |
| Base fuel total vol.% Aromatics |  |  |  |  |  | 42 |  |  | 46½ |  |  | 42½ |  |  |  | 47 | 7 | 13 | 5½ |  |  | 6½ |  |  |  | 38 |  |  |  |  | 31 |  |  |  |  | 6.5 |

NOTES ON TABLE X

(Table 25.)

The composition of the green gasolines have been calculated from the figures determined by aniline point method to an octane-free basis allowing for the octane components which have been estimated in various ways. These figures are only approximate, but provide a rough basis for comparison with various gasolines of known origin.

Composition of the individual higher boiling aromatics is only known in a few cases where spectroscopic analyses have been made.

Other properties of the gasolines from known source quoted in Table X (Table 25.) are:

Table 26. Other Properties of gasolines

| Property | Roumanian Aviation | Hydrogenation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (1) | (2) | (3) | (4) | (5) | (6) |
| Final boiling point OC | 126 | 130 | 159 | 195 | 182 | 220 | 125 |
| % distillation +loss to 75OC | 11½ | 41 | 21½ | 19 | 5 | 13 | 46 |
| % distillation +loss to 100OC | 60½ | 77 | 50 | 34 | 22 | 20 | 82 |
| % distillation +loss to 140OC | 100 | 100 | 87 | 63 | 65 | 42 | 100 |
| Reid Vapour Pressure | 3.5 | 7.0 | 5.6 | 9 | (3) | 8 | 7.6 |
| Wt.% C4+C5 | 5 | 19 | 11 | 12 | (2) | 7½ | 23½ |
| CFR Motor Octane No. | 73.3 | 73 | 74 | 68 | 77 | 70 | 77½ |

The hydrogenation gasolines (1) to (6) can be distinguished as:

(1), (2), (3)	All from creosote middle oil by medium temperature sensitive catalyst ‘vapour phase’ operation with full recycle illustrates a range of end-point and operating conditions.

(4)	High temperature (550OC) ‘aromatising’ cracking of creosote middle oil at 220 atmospheres to yield a low volatility aromatic naphtha of 180OC end point.

(5)	“Liquid phase” hydrogenation of creosote heavy oil to yield a high end-point low volatility aromatic gasoline. This material requires heavy refining treatment and contains unsaturated and some tar acids and tar bases.

(6) Medium temperature vapour phase sensitive catalyst destructive hydrogenation of a petroleum gas oil to yield an aviation grade gasoline of high volatility.

Table XI

DISTRIBUTION OF NON-AROMATIC C6, C7 & C8 HYDROCARBONS

Table 27. Distribution of non-aromatic hydrocarbons

| Components | Green C3 Gasolines (octane free) |  |  |  | Natural Roumanian 73 Oct. | Hydrogenation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1940 | 1941-1942 | 1943 |  |  | Med. Temp. Creosote end point OC |  |  | High Temp. Vap. Phase | Creosote Heavy Oil Liq. Phase | Pet. Gas Oil Med. Temp. Vap. Phase |
|  |  |  | Early | Late |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 135 | 160 | 200 | 180 | 220 | 135* |
| * C6+C7+C8 paraffins % on fuel | 13.85 | 10.15 | 12.8 | 14.5 | 33.3 | 22.6 | 21.1 | 12.65 | 5.5 | 10.6 | 33.9 |
| + C6+C7+C8 naphthenes % on fuel | 25.45 | 23.7 | 23.05 | 17.65 | 47.3 | 38.9 | 40.2 | 30.1 | 27.05 | 16.4 | 31.6 |
| C6+C7+C8 P + N on fuel | 39.3 | 33.85 | 35.85 | 32.15 | 80.6 | 61.5 | 61.3 | 42.75 | 32.55 | 27.0 | 65.5 |
| As % of C6+C7+C8 P+N |  |  |  |  |  |  |  |  |  |  |  |  |
| Paraffins 40-65OC | 10.6 | 6.2 | 11.5 | 19.6 | 9.5 | 17.5 | 14.0 | 10.4 | 3.0 | 9.3 | 22.5 |
| 65-84OC | 7.6 | 9.3 | 9.9 | 13.4 | 4.2 | 3.5 | 3.0 | 3.6 | 2.8 | 5.6 | 3.1 |
| 84-95OC | 6.9 | 5.2 | 6.7 | 5.3 | 10.3 | 8.8 | 9.0 | 7.7 | 1.6 | 8.4 | 13.8 |
| 95-102OC | 2.3 | 2.4 | 1.7 | 1.5 | 1.8 | 0.4 | 0.4 | 0.6 | 1.9 | 2.5 | 0.9 |
| 102-111OC | 2.3 | 2.2 | 2.1 | 2.0 | 4.5 | 2.8 | 1.8 | 1.3 | 2.8 | 4.4 | 5.3 |
| 111-123OC | 4.3 | 3.5 | 2.8 | 1.7 | 9.9 | 3.5 | 5.8 | 5.4 | 2.1 | 5.4 | 5.7 |
| 12-129OC | 1.3 | 1.2 | 1.1 | 1.6 | 1.1 | 0.3 | 0.4 | 0.6 | 0.2 | 3.7 | 0.5 |
| Total | 35.3 | 30.0 | 35.8 | 45.1 | 41.3 | 36.8 | 34.4 | 29.6 | 14.4 | 39.3 | 51.8 |
| Naphthenes 60-75OC | 16.0 | 15.7 | 20.7 | 20.6 | 6.1 | 19.9 | 16.5 | 15.3 | 7.5 | 10.8 | 13.6 |
| 75-84OC | 14.0 | 18.6 | 14.1 | 13.1 | 6.8 | 3.8 | 3.0 | 4.9 | 14.6 | 5.6 | 1.6 |
| 84-95OC | 5.1 | 3.7 | 5.9 | 5.0 | 8.0 | 8.8 | 11.4 | 9.3 | 4.3 | 6.8 | 9.2 |
| 95-110OC | 14.5 | 15.1 | 9.0 | 6.8 | 22.7 | 16.4 | 16.2 | 18.6 | 33.0 | 14.8 | 14.5 |
| 110-125OC | 7.1 | 5.7 | 7.5 | 4.7 | 13.6 | 9.4 | 11.0 | 13.7 | 10.3 | 10.0 | 7.4 |
| 125-137OC | 8.0 | 11.2 | 7.0 | 4.7 | 1.5 | 4.9 | 7.4 | 8.6 | 15.9 | 12.7 | 1.9 |
| Total | 64.7 | 70.0 | 64.2 | 54.9 | 58.7 | 63.2 | 65.5 | 70.4 | 85.6 | 60.7 | 48.2 |
| % Methyl cyclopentane in C6 naphthenes | 54 | 45 | 60 | 61 | 47 | 64 | 85 | 76 | 35 | 66 | 89 |
| % iso-paraffins in total |  |  |  |  |  |  |  |  |  |  |  |
| Pentane | 61 | 53 | 59 | 61 | 57 | 81 | 82 | 77 | Ca. 50 | 61 | 87 |
| Hexanes | 50 | 40 | 54 | 59 | 69 | 83 | 82 | 74 | Ca. 50 | 63 | 88 |
| Heptanes (approx.) | 75 |  |  |  | 25 | 95 | 96 | 91 | Ca. 50 | 72 | 94 |

# Appendix V

NOTE ON METHODS OF ANALYSIS – ACCURACY & REPRODUCIBILITY

- GENERAL NOTES

Then method of analysis of petrols used at Billingham for some years has been fully described in various reports (see ref. 3 & 4).

A fairly simple method was designed to enable a large number of samples to be examined is a reasonably short time and with fairly simple methods for analysis. The policy was adopted because it was realised that any attempt at interpretation of the analytical results could only hope to have any real meaning:

(a)	If a sufficiently large number of samples of enemy aviation gasoline were analysed so as to give a measure of the limits of fluctuation between different samples. This is necessary so that obviously weathered or contaminated samples could be excluded, and so as to be able to arrive at a satisfactorily representative average analysis.

(b)	If a sufficiently wide range of samples were analysed of aviation gasolines or blending components from known sources or produced by known processes to be able to identify an unknown hydrocarbon mixture as being derived from a particular source by reason of some highly characteristic factors in the composition or distribution of the individual hydrocarbons.

(c)	If the differences in composition of aviation blending components and naphthas from different sources as determined by the method of analysis used were considerably greater than possible errors in the method of analysis.

Obviously, some compromise had to be made between highly accurate analysis of only a few samples or approximate analysis of as wide a selection of samples as possible.

Since it had been found that a simple fractionation analysis method (refs. 3 & 4) gave reproducible results and demonstrated that hydrogenation gasolines had characteristics in composition which apparently allowed them to be distinguished from natural petroleum gasoline, it was decided to use this method throughout and apply the results in a comparative way to determine whether the unknown material could contain large proportions of any of the known reference materials examined, or whether differences in composition were great enough to exclude such sources.

The comparative simplicity of this method has allowed it to be applied over a period of four years to:

66 samples of Enemy blue B4 aviation gasoline

14 samples of Enemy green C3 aviation gasoline

3 miscellaneous samples of Enemy Diesel oils and motor gasolines

8 synthetic octanes from various sources (and many other Heysham samples)

22 natural petroleum gasolines from a variety of sources

8 aromatic materials, including coke oven products, hydroformed, aromatised, ‘hydrofined’ etc.

Over 100 samples of hydrogenation gasolines from various processes and raw materials.

The method has later been extended considerably on the analytical side by using ultra violet spectroscopic techniques to identify and estimate the individual aromatics in separate fractions from the distillation still, which infra-red spectroscopic technique has been used successfully to identify and estimate individual paraffins and naphthenes in the fractions freed from aromatics by acid treatment.

Methods of analysis have developed very considerably over the last two years, both as regards much improved methods of separation by more efficient fractionating columns and special methods for removal of the interfering aromatics, and also by the rapid development in spectroscopic technique for analysis of the fractions. These latest developments have not yet been applied fully to the problems of analysis of a sample of enemy aviation gasoline since the time taken would be considerably longer than with the present method, and the additional and more accurate information obtained would only become of value if such elaborate methods could also be applied to the reference materials. Some scattered results are collected in Table XVI.

Recommendation

Since the simpler methods now appear to have defined fairly closely the composition of the green aviation fuel and allowed an interpretation to be put forward as to the probable method is manufacture it may now be worthwhile analysing a typically representative sample of green gasoline alongside products made by the processes suggested for its manufacture.

II	FRACTIONATION

All the samples examined have been distilled in 7ft. x1¼” glass column jacketed to be adiabatic and operated with a reflux ratio of 40 to 60/1. In the earlier distillations the columns were packed with ‘Digby’ beet eyelets and were tested as having 27 theoretical plates. More recently, these columns have been used packed with 1/8 x 1/8” gauze rings, when the test shows 40-60 theoretical plates.

In all cases some 6-8 litres of material is used as a charge to the boiler and about 95% is distilled into about 40-50 separated 2% fractions over a continuous period of about 170 hours operation.

It has been pointed out (ref 4.) that distillation at this efficiency is ample to separate individual hydrocarbons or groups of hydrocarbons, whose boiling points differ by about 5-8OC or more. This separation gives considerable information alone or when coupled with some method of analysis of the fractions. The next step in separation of individual members of such close boiling groups as occur at 90OC would require stills of at least 200 theoretical plates, and a considerably longer operating time.

True Boiling Point Distillation & Distribution

Typical distillation curves obtained in the 30-40 plate distillation stills are reproduced in Figure (1). In all such distillations these curves show clearly defined ‘plateaux’ separated by regions where the temperature is rising steeply. The mid-points of such steep slopes have often been used as ‘cut points’ (see Ref. 4.) for measuring the amount of material distilling over each plateau. Such figures for the volume percentage distilling between such ‘cut-points’ are reasonably reproducible and afford a direct means of comparing different samples, since this method of comparison only involves reproducibility of fractionation and does not involve any method of hydrocarbon analysis.

Table XIII lists the ‘true boiling’ distribution for the fourteen samples of green aviation gasoline examined:

Cut (1) 0-40OC is a measure of butanes + pentanes

Cut (2) 40-65OC is a measure of iso-hexanes + cyclopentane

Cut (3) 65-75OC is a measure of n-hexane + methyl cyclopentane + about 1½ % of benzene

Cut (4) 75-84OC is a measure of cyclohexane + the rest of the benzene (Cuts (2) – (4) thus measure the C6 hydrocarbons)

Cut (5) 84-95OC is a measure of dimethyl cyclopentanes + branched heptanes, but may include some of the 2,2,4 trimethyl pentane if such is present.

Cut (6) 95-114OC is a measure of 2,2,4 trimethyl pentane + other trimethyl pentanes and dimethyl hexanes  + C7 naphthenes (such as methyl cyclohexane) + toluene. The main components here are toluene and trimethyl pentanes.

Cut (7) 114-130OC includes such C8 paraffins as methyl heptanes and n-octanes and any trimethyl hexanes and such C8 naphthenes as dimethyl cyclohexane.

Cut (8) 130-150OC consists mainly of C8 aromatics with small amounts of C8 and C9 paraffins and naphthenes.

Cut (9) 150-170OC includes the C9 aromatics such as n-propyl benzene and ethyl toluenes, as well as some C9 naphthenes.

Cut (10) 170-180OC includes hydrindenes and other C9 and C10 aromatics as well as some hydrogenated trimer (2,2,4,6,6 pentamethyl heptane?)

Cut (11) >180OC includes some higher boiling aromatics – possibly n-butyl benzene.

Comparison of T.B.P. Figures

The figures for boiling point distribution show that certain changes have occurred while in other cases groups of samples are very similar. The main points are summarised Table XIV.

The 1940 material is characterised by the large amount boiling above 170OC. All four samples are similar in this respect. Sample 40/41 has a higher volatility and may have had more added pentane since the whole of this difference is in the 0-40OC fraction.

In 1941 the amount of material boiling above 170OC was reduced by 10% presumably by cutting back on aromatised naphtha. But benzene, toluene and C8 aromatic contents were increased considerably, presumably by increasing the severity of aromatisation. These factors account for the decrease in>170OC fraction and the increase in 75-84OC (benzene) 95-114OC (toluene) and 130-150OC (C8 aromatics). Part of the increase in 95-114OC fraction is due to about 2½ increase in octanes added. It would be expected that simple cutting back to a lower end point would increase the front-end volatility whereas actually there has been a decrease of about 4% in the C6 paraffins and naphthenes, and possibly 2% in the C7 compounds. This change again supports the view that the lower boiling fractions are produced in a separate process and that the aromatisation produces very little C6 paraffins.

In 1942 no further changes are apparent apart from possibly a slight reduction on the octane content.

In 1943 the more volatile lower gravity fuels appeared with lower aromatic content of 32-33% and about 7% more C5 and C6 paraffins and naphthenes. The octane content was also higher. Later during 1943, the amounts of C7 and C8 aromatics were increased and the amounts of higher boiling naphthenes considerably decreased.

The two extreme types of this fuel are compared in figure (1), which clearly shows the difference in boiling range distribution.

III 	ANALYSIS

The method of analysing the separate fractions for aromatics, paraffins and naphthenes by determination of the aniline point before and after removal of the aromatics by sulphonation, has already been described and the errors considered which arise from the use of any type of ‘smooth curve’ relating boiling point and the average physical property of a series of hydrocarbons. (Ref. 4)

It was shown (Ref. 4) that little improvement in accuracy is obtained by using methods based on specific refraction or refractivity intercept and often these methods are less sensitive and require considerably more time than the simple aniline point method.

It was also shown in the above report that the ‘smooth curve’ aniline point method is liable to quite large errors when applied to fractions rich in hydrocarbons which have aniline points deviating markedly from such smooth curve relationships. Such compounds as cyclohexane, methyl cyclohexane, ethyl cyclohexane all having lower aniline points are over-estimated, and the associated paraffins under-estimated. 2,2,4 trimethyl pentane may also tend to be over-estimated, while dimethyl cyclopentane will be under-estimated. These errors are obvious from a consideration of figure (6).

Such errors can be overcome by assuming that particular hydrocarbons are present according to boiling point and then using the correct physical constants for calculation of composition. This is certainly an improvement in many cases, but may lead to other errors, if the exact behaviour of the hydrocarbons in the distillation cannot be defined in relation to boiling point.

The best method appears to be to use spectroscopic methods to identify the components present and then calculate composition from accurately measured physical constants. This would certainly be the most reliable method if distillation could limit the cuts to only two components. Several authors have used the method of super-fractionation and identified and estimated the components by direct refractive index measurements. It is felt that this method may be unreliable where anomalous vapour pressure relationship exist, or where the physical constants of the pure components are not sufficiently different.

Presentation of Results

The method of presentation of the results obtained by aniline point analysis of separate fractions (see Table XV) is to express these as a percentage of the material charged to the still, and so by summation to construct three separate T.B.P. curves for paraffins, naphthenes and aromatics (see Figures (7), (8) and (9)). These are then subdivided into plateau and the results expressed as in Table IX (Appendix IV).

In comparing gasolines of different end-point and possibly with varying amounts of added pentane and also possibly with added aromatics, it has been found preferable to consider the distribution of the C6, C7 and C8 paraffins and naphthenes only. These are divided into 13 main groups which are plotted on a diagram as in figures (11), (12) etc. These ‘hydrocarbon pictures’ give at a glance the main essentials of the distribution of C6, C7 and C8 non aromatics (usually about 70% of a normal aviation gasoline) and enable a quick decision to be reached as to which gasolines are similar, and which are not.

ERRORS IN ANILINE POINT METHOD AS AFFECTING MAIN ARGUMENT & CONCLUSIONS

The aniline point method was checked against a synthetic blend made up from pure components and gave remarkably good results. (Ref. 84)

Cyclopentane, iso-hexane, n-hexane and methyl cyclopentane are all probably estimated fairly truthfully. (See Ref 5.)

Cyclohexane

If any paraffins boiling above 80OC (2, 2 and 2,4 dimethyl hexane) are present the aniline point method may tend to over-estimate cyclohexane. Actually, most gasolines contain very little of those paraffins (0.2%). If enough benzene is present cyclohexane will distil as an azeotrope with 50% benzene at 77.2OC. The separation from methyl cyclopentane is then not so good as in the absence of benzene. If very much benzene is present, some of the paraffins normally boiling at 90OC may distil at 78-79OC and will lead to marked over-estimation of cyclohexane.

Although the green gasolines contain 6-8% benzene it is doubtful whether the errors made in estimating cyclohexane are large enough to vitiate the argument on this point regarding normal hydro-petrols, which is also supported by evidence from iso/normal ratios in the paraffins and several other points.

Many figures are available for independent estimations of cyclohexane by spectroscopic methods, (see Ref. 5) which show that generally the aniline point method is not in error to such a great extent even with the green gasolines.

Table 28. Cyclohexane % content by various methods.

|  | Synthetic Blend | Lagunillas | Roumanian | Venezuelan | Normal Hydro |  | Green Gasoline |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | AIR144 | AIR176 |
| ‘Smooth curve’ aniline point | 5.0 | 5.9 | 5.5 | 4.5 | 2.5 | 0.6 | 4.6 | 5.3 |
| ‘Corrected’ aniline point | 4.9 | 5.1 | 4.7 | 3.8 | 2.5 | 0.6 | 4.1 | 5.0 |
| Infra-red | 4.5 | 5.1 | 6.1 | 3.9 | 2.2 | 0.7 | 3.9 | 5.1 |

Dimethyl cyclopentane and iso-heptane

The relatively high aniline point of the transform of 1,3 and 1,2 dimethyl cyclopentanes (which distil with the heptanes 2, 3 dimethyl pentane, 2 methyl hexane, 3 methyl hexane in an inseparable mixture at 90-92OC) cause those naphthenes to be underestimated and the iso-heptane over-estimated by use of the smooth curve method (see Fig. 6 and Ref. 4.).

The amount of ‘90OC flat’ in the green gasolines is abnormally small and this fact has been used to support the argument that such gasolines consist of a blend of two fractions with a gap in boiling range at about 90OC.

A small amount of the 90OC flat paraffins are brought down to 79OC when distilling in the presence of excess benzene.

These errors are relatively insignificant and do not affect the main argument regarding deficiency of 90OC naphthenes. This supported by spectroscopic analyses (Table XVI)

More 2,3 dimethyl pentane has been found by spectroscopy than would be expected in comparison with the other heptanes 2 methyl hexane and 3 methyl hexane. It is suggested that this has come from the added synthetic octane.

Methyl cyclohexane, n-Heptane and 2,2,4 Trimethyl pentane

The very high aniline point of 2,2,4 trimethyl pentane and the low aniline point of methyl cyclohexane results in over-estimation of 2,2,4 TMP and under-estimation of the 100OC ‘naphthenes’ (methyl cyclohexane and ethyl cyclohexane). It is impossible to estimate n-heptane in such mixtures without considerably improved fractionation. n-heptane can be estimated roughly by octane number determinations. Fairly close agreement with independent infra-red estimates of the content of 2,2,4 trimethyl pentane are reported in Table VI (Appendix III). In making such rough estimations a low figure was taken for n-heptane content, based on O.N. (octane number) figures and the relatively small amount of iso-heptanes. A single infra-red determination (Table XVI) gives n-heptane = 0.4%.

The aniline point method indicates that the amount of methyl cyclohexane is small and that there has been an appreciable decrease during 1943 (Table XI). Independent figures by infra-red (Table XVI) confirm this.

Higher boiling trimethyl pentanes & toluene

The analysis of fractions 106-112OC containing the higher boiling trimethyl pentanes is rendered difficult due to the presence of high concentrations of toluene. Removal of toluene by sulphonation introduces the risk of decomposing the trimethyl pentanes, although it has been found that with rapid acid treatment decomposition is negligible (Ref. 81). In the presence of much toluene it has been found that such trimethyl pentanes as 2,3,4- distil at about 109OC, which is several degrees below its true boiling point.

Due to the anomalous behaviour of so many non-aromatic hydrocarbons when distilled with aromatics it is obviously preferable to remove the aromatics and fractionate these separately. SO2 (Sulphur dioxide) extraction does not give sufficiently complete separation; phenol extractive distillation is not easy to apply to the whole boiling range from 70OC to over 180OC; azeotropic distillation has worked very well for particular problems such as removal of toluene from the octanes (Ref. 81), but it more laborious when applied generally and entails several distillations with difficulties in getting a complete balance; the method of selective adsorption of aromatics on silica gel offers some promise but may be difficult to apply to large volumes of spirit rich in aromatics as to get quantitative separation and recovery.

Comparison of Analytical Results

The figured obtained for distribution of aromatics have already been discussed in Appendix II (Table II), and for paraffins and naphthenes in Appendix IV (Table IX).

Progressive changes in composition have been noted (Appendix IV). These are clearly demonstrated by comparing figures (7), (8) and (9), which show the separated T.B.P. (True Boiling Point) curves for aromatics, naphthenes for the three samples GF32 (1940), AIR285 (early 1943) and AIR328 (late 1943)

Comparison of these curves show:

- The large amount of high boiling aromatics (B.P. ca 180-185OC) in the 1940 gasoline, which are not present in later types.

- The increased amount of toluene and C8 aromatics in the later samples.

- The increased amount of octanes in the later samples.

- The increased amount of hexanes in the later samples.

- The marked increase in amount of higher boiling naphthenes, which is very apparent in a comparison of AIR285 and AIR328.

The paraffins/naphthene composition of different fuels can also be expressed by plotting curves of aniline point against boiling point for the non-aromatic hydrocarbons as in Fig. (10). By plotting aniline point against B.P. rather than the volume %, no direct measure of the amounts of individual components is possible, but since the curves were all obtained by fractionation at the same efficiency they indicate whether large amounts of certain components are present by the closeness with which  the curves approach the aniline points for individual hydrocarbons.

It is seen that methyl cyclopentane is separated fairly pure from the hydro-petrol where large quantities are present, but that a purer cyclohexane is obtained from the earlier sample of green gasoline.

The main differences from normal hydro petrol are that the aniline points are generally higher indicating more paraffins and less naphthenes. Again, there is a difference between the early green gasoline GF32 and the later sample AIR328.

The more paraffinic nature of green gasolines is largely due to the presence of added octanes. The presence of high aniline point material is shown markedly in the ranges 95-100OC, 105-112OC and 170-185OC. The first peak is due to 2,2,4 trimethyl pentane of exceptionally high aniline point. This is not isolated pure, even in AIR328, presumably due to the inability of the still to separate n-heptane and methyl cyclohexane.

The trough in aniline point at 102OC is due to methyl cyclohexane. Much less is present in the green gasolines and with high aniline point octanes on each side the trough is less marked than in normal hydro-gasoline. Again, AIR328 contains more octanes and less methyl cyclohexane than GF32.

The peak at 100-112OC is due to 2,3,4 trimethyl pentane and other trimethyl pentanes. This peak is not as pronounced since these paraffins have lower aniline points, but it is probable that material in this range is largely 2,3,4 trimethyl pentane, which in the presence of toluene apparently distils below its true boiling point.

The third peak occurs at 170-185OC and is probably due to the presence of hydrogenated trimer contained in the added octanes.

Hydrocarbon Pictures

Figures (5) – (18) illustrate the value of the method in clearly demonstrating different types of hydrocarbon composition and distribution of C6, C7 and C8 non-aromatics.

Fig. (11)	Green C3 gasolines	GF28, 40/41, GF31 GF32 (1940 type)

Fig. (12)	Blue B4 gasoline	no hydro-petrol, small content of added octanes, low aromatic content (90/8/2) 7 very similar samples.

Fig. (13)	Blue B4 gasoline	no hydro-petrol, small content of added octanes with higher aromatic content (90/6A/1) 3 very similar samples.

Fig. (14)	Blue B4 gasoline	estimated 50% hydro petrol, no added octanes, no added aromatics (90/H/1) 2 very similar samples.

It is clear that these samples can be divided into a number of groups, members of which are very similar. Many other examples have been reported (Refs. 1,2 and separate reports).

Fig. (15)	Lagunillas, and Venezuelan aviation gasolines.

Fig. (16)	130OC end point normal hydro gasoline from petroleum gas oil.

Fig. (17	)	138OC end point normal hydro gasoline from creosote.

Fig. (18)	200OC end point normal hydro gasoline from creosote.

The above are a few examples of the large number of materials analysed in this way. The characteristic type of ‘picture’ for a hydrogenation gasoline is easily distinguished from petroleum gasoline Fig. (15), and it is apparent that the blue gasolines of Fig. (14) are closer in composition to a normal hydrogenation gasoline, than those in Figs. (12) and (13), whereas the green gasoline Fig. (11) is seen to be very different from normal hydrogenation spirit.

Photo 9. German Messerschmidt Bf-109 fighter being refuelled on an airfield at one of the Channel Islands, circa 1940.

The German occupation of the Channel Islands lasted for most of the Second World War, from 30 June 1940 until their liberation on 9 May 1945. The Bailiwick of Jersey and Bailiwick of Guernsey are two British Crown dependencies in the English Channel, near the coast of Normandy.

As with all Nazi German occupations, all useable/serviceable resources were pressed into the German war effort; note the company markings on the English lorry. With the limited range of the Messerschmitt Bf-109, the Channel Islands provided an ideal base of operations for Luftwaffe’s attacks on shipping in the English Channel and England itself.

TABLE XIII	“TRUE BOILING POINT’ DISTILLATION

Volume % distilled in 30-40 Plate Columns

Table 29. True Boiling Point Distillation

|  | 1940 |  |  |  | 1941 |  | 1942 |  | 1943 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | GF28 | 40/41 | GF31 | GF32 | AIR141 | AIR144 | AIR176 | AIR189 | AIR285 | AIR305 | AIR311 | AIR317 | AIR318 | AIR328 |
| Vol.% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Up to 40OC | 6.2 | 9.6 | 5.9 | 7.7 | 7.0 | 7.0 | 6.6 | 7.5 | 9.6 | 10.15 | 10.0 | 10.65 | 9.8 | 9.9 |
| Up to 65OC | 10.7 | 14.3 | 9.8 | 12.0 | 9.0 | 9.5 | 9.3 | 10.4 | 13.5 | 14.5 | 16.2 | 16.55 | 15.9 | 15.7 |
| Up to 75OC | 21.1 | 24.6 | 18.3 | 21.3 | 16.35 | 18.9 | 17.0 | 17.7 | 23.9 | 24.85 | 26.6 | 26.4 | 25.8 | 25.6 |
| Up to 84OC | 28.7 | 33.6 | 27.5 | 30.8 | 28.7 | 32.2 | 29.5 | 31.2 | 34.1 | 34.3 | 34.2 | 35.15 | 34.95 | 35.0 |
| Up to 95OC | 35.1 | 38.7 | 32.5 | 36.0 | 31.7 | 35.45 | 33.4 | 33.7 | 39.2 | 40.65 | 39.55 | 40.25 | 39.9 | 40.2 |
| Up to 114OC | 609 | 65.2 | 60.3 | 62.3 | 66.1 | 70.1 | 67.0 | 67.1 | 69.4 | 71.1 | 71.1 | 72.6 | 72.0 | 73.0 |
| Up to 130OC | 66.3 | 70.7 | 65.9 | 67.0 | 70.0 | 74.9 | 71.5 | 72.6 | 73.3 | 75.2 | 74.2 | 74.9 | 74.5 | 75.2 |
| Up to 150OC | 78.3 | 81.0 | 77.1 | 78.5 | 84.3 | 86.9 | 85.2 | 86.1 | 86.8 | 86.2 | 87.3 | 88.2 | 89.3 | 87.7 |
| Up to 170OC | 82.2 | 86.2 | 84.0 | 83.9 | 92.9 | 93.5 | 93.4 | 94.0 | 92.0 | 93.7 | 93.0 | 94.4 | 94.8 | 93.9 |
| Cut No. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 0-40OC | 6.2 | 9.6 | 5.9 | 7.7 | 7.0 | 7.0 | 6.6 | 7.5 | 9.6 | 10.15 | 10.0 | 10.65 | 9.8 | 9.9 |
| - 40-65OC | 4.5 | 4.5 | 4.3 | 4.3 | 2.0 | 2.5 | 2.7 | 2.9 | 3.9 | 4.35 | 6.2 | 5.9 | 6.1 | 5.8 |
| - 65-75OC | 10.4 | 10.3 | 10.5 | 9.3 | 7.35 | 9.4 | 7.3 | 7.3 | 10.4 | 10.35 | 10.4 | 9.95 | 9.9 | 9.9 |
| - 75-84OC | 7.6 | 9.0 | 9.2 | 9.5 | 12.35 | 13.3 | 12.5 | 13.5 | 10.2 | 9.45 | 7.6 | 8.75 | 9.15 | 9.4 |
| - 84-95OC | 6.4 | 5.1 | 5.0 | 5.2 | 3.0 | 3.25 | 3.9 | 2.5 | 5.1 | 6.35 | 5.35 | 5.1 | 4.95 | 4.8 |
| - 95-114OC | 25.8 | 26.5 | 27.8 | 26.3 | 34.4 | 34.65 | 33.6 | 33.4 | 30.2 | 30.45 | 31.55 | 32.35 | 32.1 | 32.8 |
| - 114-130OC | 5.4 | 5.5 | 5.6 | 4.7 | 3.9 | 4.8 | 4.5 | 5.5 | 3.9 | 4.1 | 3.1 | 2.3 | 2.5 | 2.2 |
| - 130-150OC | 12.0 | 10.3 | 11.2 | 11.5 | 14.3 | 12.0 | 13.7 | 13.5 | 13.5 | 11.0 | 13.1 | 13.3 | 14.8 | 12.5 |
| - 150-170OC | 3.9 | 5.2 | 6.9 | 5.4 | 6.6 | 6.6 | 8.2 | 7.9 | 5.2 | 7.5 | 5.7 | 6.2 | 5.5 | 6.2 |
| - >170OC | 17.8 | 14.8 | 16.0 | 16.1 | 7.1 | 6.5 | 6.6 | 6.0 | 8.0 | 6.3 | 7.0 | 5.6 | 5.2 | 6.1 |
| - >180OC | 12.1 | 10.0 | 8.1 | 10.9 | <3 | <3 | <3 | <3 |  |  | <3 |  |  | 3.2 |
| Vol.% Aromatics | 35 |  |  |  | 37½ |  |  |  | 33 ------------------------------------------ > 36 |  |  |  |  |  |

Table XIV

Table 30. Summary of Main Differences in Distillation Distribution

|  | 1940 | 1941,1942 | 1943 |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Early | Later |  |
| C5 0-40OC | 7 | 7 | 10 |  | Butanes & pentanes increased in 1943 |
| C6 |  |  |  |  |  |
| 40-65OC | 4½ | 3 | 4 | 6 | Iso-hexanes |
| 65-75OC | 10 | 7½ | 10½ | 10 | n-hexane + methyl cyclopentane +benzene |
| 75-84OC | 9 | 12½ | 10 | 9 | Cyclohexane + benzene |
| Total C6 | 23½ | 23 | 24½ | 25 | Increased during 1943 |
| Benzene | 6½ | 8½ | 7½ | 6½ | Increase in 1941, 1942 now down again |
| C6 paraffins + naphthenes | 17 | 14½ | 17 | 18½ | Increase in 1943 |
| C7 (& octanes) |  |  |  |  |  |
| 84-95OC | 5 | 3 | 5 | 5 | Iso-heptanes, dimethyl cyclopentane |
| 95-114OC | 26½ | 34 | 30 | 32½ | Trimethyl pentanes, toluene, etc. |
| Total C7 (+octanes) | 31½ | 37 | 35 | 37½ |  |
| Toluene | 8 | 12 | 10 | 12 | Toluene increased again during 1943 |
| Octanes +C7 (95-114OC) | 16½ | 22 | 20 | 20½ | Includes methyl cyclohexane & n-heptane. |
| C8 (& C9) |  |  |  |  |  |
| 114-130OC | 5 | 5 | 4 | 2 | C8 naphthenes decreased during 1943 |
| 130-150OC | 11½ | 13½ | 13½ | 13½ |  |
| C8 aromatics | 6½ | 9 | 8½ | 10½ | Increased during 1943 |
| 130-150OC (P+N) | 5 | 4½ | 5 | 2 | Naphthenes decreased during 1943 |
| C9 |  |  |  |  |  |
| 150-170OC | 5½ | 8 | 6 | 6 |  |
| Aromatics (150-170OC) | 3½ | 4 | 3½ | 4 | Increased during 1943 |
| 150-170OC (P+N) | 2 | 4 | 2½ | 2 | Decreased during 1943 |
| C10 & higher |  |  |  |  |  |
| >170OC | 16 | 6½ | 8 | 6 |  |
| Aromatics >170OC | 10 | 4 | 3½ | 3½ | Decreased for 1941 |
| >170OC (P+N) | 6 | 2½ | 4½ | 2½ | Naphthenes decreased during 1943 |

Table XV

PROPERTIES OF FRACTIONS OF ENEMY AVIATION GASOLINE (GREEN C3 TYPE) BULKED SAMPLE NO. AIR328, 329, 330

(6,000 mls. distilled. No. 4 Still Ref. 43/9/71)

Table 31. Properties of Fractions of Enemy Aviation Gasoline

| B.P.OC (Corr.) | Total Dist. Vol.% | Original fractions |  |  |  |  | Aromatic- free fractions |  | Total H/Cs as vol.% on Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Density 15/15 | A.P.OC/ n 20D | H/Cs Vol.% |  |  | Density 15/15 | A.P.OC | Arom. +Unsat. | Naphthenes | Paraffins |
|  |  |  |  | Aromatics +Unsat. | Naphthenes | Paraffins |  |  |  |  |  |
| 0-30 | 5.53 |  | 1.3510 | 0 | 0 | 100 |  |  | 0 | 0 | 5.530 |
| 30-40 | 9.90 |  | 1.3570 | 0 | 0 | 100 |  |  | 0 | 0 | 9.900 |
| 40-57.0 | 11.90 |  | 1.3892 | 0 | 51.0 | 49.0 |  |  | 0 | 1.020 | 10.880 |
| 61.1 | 13.90 |  | 1.3750 | 0.4 | 0 | 99.6 |  |  | 0.008 | 1.020 | 12.872 |
| 65.0 | 15.70 |  | 1.3770 | 2.1 | 0 | 97.9 | 0.659 | 71.0 | 0.046 | 1.020 | 14.634 |
| 69.1 | 17.70 |  |  | 6.1 | 8.4 | 85.5 | 0.670 | 65.4 | 0.168 | 1.188 | 16.344 |
| 70.4 | 19.70 | 0.711 | 46.9 | 9.7 | 53.7 | 36.6 | 0.718 | 48.1 | 0.561 | 3.363 | 17.826 |
| 71.6 | 21.75 | 0.750 | 29.4 |  |  |  |  |  |  |  |  |
| 72.3 | 23.75 | 0.768 | 19.2 | 21.5 | 72.5 | 6.0 | 0.747 | 37.9 | 1.378 | 6.118 | 18.054 |
| 75.0 | 25.55 | 0.775 | 11.9 |  |  |  |  |  |  |  |  |
| 77.2 | 27.55 | 0.808 | <5 | 49.3 | 49.4 | 1.3 | 0.758 | 37.3 | 3.350 | 8.094 | 18.106 |
| 78.3 | 29.55 | 0.824 | <5 |  |  |  |  |  |  |  |  |
| 79.0 | 31.55 | 0.830 | <5 | 58.2 | 32.0 | 9.8 | 0.745 | 41.4 | 4.514 | 8.734 | 18.302 |
| 79.6 | 33.55 | 0.833 | <5 | 58.8 | 30.6 | 10.6 | 0.735 | 46.0 | 6.514 | 9.774 | 18.662 |
| 84.0 | 34.95 | 0.807 | <5 |  |  |  |  |  |  |  |  |
| 90.7 | 36.95 | 0.724 | 55.2 | 6.3 | 24.4 | 69.3 | 0.716 | 61.8 | 6.640 | 10.262 | 20.048 |
| 92.9 | 38.95 | 0.720 | 60.8 | 1.6 | 20.6 | 77.8 | 0.715 | 63.9 | 6.692 | 10.927 | 22.561 |
| 95.0 | 40.18 | 0.714 | 65.0 |  |  |  |  |  |  |  |  |
| 98.6 | 42.18 | 0.708 | 69.9 | 2.9 | 0 | 97.1 | 0.705 | 72.9 | 6.750 | 10.927 | 24.503 |
| 99.0 | 46.18 | 0.705 | 74.2 | 2.4 | 0 | 97.6 | 0.681 | 76.9 | 6.846 | 10.927 | 28.407 |
| 99.1 | 46.18 | 0.702 | 74.6 |  |  |  |  |  |  |  |  |
| 99.2 | 48.25 | 0.708 | 72.3 | 4.1 | 0 | 95.9 | 0.689 | 75.5 | 7.013 | 10.927 | 32.310 |
| 99.3 | 50.25 | 0.714 | 69.5 |  |  |  |  |  |  |  |  |
| 99.6 | 52.25 | 0.719 | 66.4 | 4.3 | 0 | 95.7 | 0.710 | 71.0 | 7.099 | 10.927 | 34.224 |
| 102.9 | 54.25 | 0.741 | 52.2 | 9.0 | 30.0 | 61.0 | 0.731 | 61.6 | 7.279 | 11.527 | 35.444 |
| 107.8 | 56.25 | 0.779 | 22.2 | 41.4 | 17.4 | 41.2 | 0.733 | 63.0 | 8.935 | 12.223 | 37.092 |
| 108.5 | 58.25 | 0.794 | 9.1 |  |  |  |  |  |  |  |  |
| 108.7 | 60.25 | 0.801 | <5 | 56.9 | 5.8 | 37.3 | 0.723 | 67.3 | 11.211 | 12.455 | 38.584 |
| 109.0 | 62.25 | 0.808 | <5 |  |  |  |  |  |  |  |  |
| 109.1 | 64.25 | 0.809 | <5 | 63.5 | 3.7 | 32.8 | 0.717 | 68.2 | 13.751 | 12.603 | 39.896 |
| 109.4 | 66.25 | 0.818 | <5 |  |  |  |  |  |  |  |  |
| 109.9 | 68.25 | 0.833 | <5 | 72.7 | 2.3 | 25.0 | 0.755 | 67.6 | 17.749 | 12.730 | 41.271 |
| 110.0 | 70.25 | 0.837 | <5 |  |  |  |  |  |  |  |  |
| 110.0 | 71.75 | 0.839 | <5 |  |  |  |  |  |  |  |  |
| 122.4 | 73.75 | 0.820 | 24.0 | 26.2 | 45.8 | 28.0 | 0.781 | 56.5 | 18.412 | 13.889 | 41.979 |
| 125.0 | 74.28 |  | 50.4 |  |  |  |  |  |  |  |  |
| 133.9 | 76.28 | 0.790 | 25.0 | 31.4 | 38.8 | 29.8 | 0.755 | 59.7 | 19.040 | 14.665 | 42.575 |
| 135.8 | 78.28 | 0.840 | <5 | 72.1 | 11.2 | 16.7 | 0.752 | 64.5 | 20.482 | 14.889 | 42.909 |
| 136.8 | 80.28 | 0.855 | 1.4860 | 84.5 | 4.7 | 10.8 | 0.749 | 66.8 | 23.862 | 15.077 | 43.341 |
| 137.9 | 82.28 | 0.861 | 1.4900 |  |  |  |  |  |  |  |  |
| 138.2 | 84.28 | 0.862 | 1.4920 | 91.5 | 5.5 | 3.0 | 0.770 | 60.1 | 27.522 | 15.297 | 43.461 |
| 143.4 | 86.28 | 0.864 | 1.4902 |  |  |  |  |  |  |  |  |
| 139.4-145.0 | 87.06 | 0.861 | <5 | 64.2 | 27.8 | 8.0 | 0.786 | 58.3 | 29.326 | 16.078 | 43.686 |
| 156.1 | 88.20 | 0.840 | <5 |  |  |  |  |  |  |  |  |
| 160.3 | 89.09 | 0.838 | <5 |  |  |  |  |  |  |  |  |
| 161.4 | 89.98 | 0.850 | <5 | 74.1 | 16.3 | 9.6 | 0.792 | 63.2 | 32.031 | 16.673 | 44.036 |
| 162.2 | 90.87 | 0.852 | <5 |  |  |  |  |  |  |  |  |
| 164.3 | 91.76 | 0.852 | <5 |  |  |  |  |  |  |  |  |
| 166.0 | 92.74 | 0.852 | <5 |  |  |  |  |  |  |  |  |
| 167.7 | 93.63 | 0.849 | <5 | 60.8 | 10.7 | 28.5 | 0.785 | 71.1 | 33.654 | 16.959 | 44.797 |
| 172.3 | 94.52 | 0.850 | <5 |  |  |  |  |  |  |  |  |
| 172.3 | 95.41 | 0.846 | <5 |  |  |  |  |  |  |  |  |
| 175.3 | 96.30 | 0.844 | <5 | 50.3 | 2.8 | 46.9 | 0.771 | 76.1 | 34.549 | 17.009 | 45.632 |
| 181.6 | 97.19 | 0.852 | <5 |  |  |  |  |  |  |  |  |
| 184.0 | 98.15 | 0.838 |  | 43.2 | 22.1 | 34.7 | 0.803 | 70.2 | 35.763 | 17.630 | 46.607 |
| >184 Res | 100.0 | 0.842 |  |  |  |  |  |  |  |  |  |

SPECTROSCOPIC ANALYSES OF NON-AROMATIC HYDROCARBONS

In no cases have a complete set of fractions of a German green gasoline been examined spectroscopically since interest has mainly centred on the two problems of octanes and higher aromatics. A few figures have been obtained, however, which are of interest in comparison with other fuels and to compare with other methods such as the aniline point (Billingham) or refractive index methods (U.S.A.)

Table XVI

Table 32. Further Analyses of German Aviation Gasolines

| True B.P. | CRC ID Additional information | B.P OC CRC (additional information) | Hydrocarbon | German Green C3 Gasolines |  |  |  |  | Blue B4 Gasoline |  |  | Roumanian 73 Octane | Hydrogenation (normal) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | AIR144 | AIR176 | AIR189 | AIR285 | AIR317 | AIR177 | AIR203 | AIR209 |  | Creosote | Pet. Gas Oil |
| 49.3 | 4774 | 49.3 | Cyclopentane |  |  |  |  |  |  |  | 1.0 | 0.6 | 1.0 | 0.9 |
| 49.7 | 3289 | 49.7 | 2,2, dimethyl butane (neohexane) |  |  |  | 0.1 |  |  | - | - |  | 0.15 | 0.5 |
| 58.6* | 3290 | 57.9 | 2,3 dimethyl butane |  |  |  | 0.95 |  |  | - | 0.9 | 7.8 | 1.2 | 2.4 |
| 60.25 | 8823 | 60.2 | 2 methyl pentane (iso-hexane) |  | 1.4 | 0.5 | 0.97 |  | 3.7 | 3.85 | 3.1 |  | 5.6 | 7.9 |
| 63.2 | 8824 | 63.2 | 3 methyl pentane |  | 0.7 | 0.5 | 0.4+ |  | 2.1 | 2.6 | 3.1 |  | 3.8 | 4.6 |
| 68.75 | 6731 | 68.7 | n-hexane | 2.3 | 2.4 | 2.0 |  |  | 3.8 | 3.85 | 3.6 | 3.0 | 1.6 | 2.0 |
| 71.85 | 4826 | 71.8 | Methyl cyclopentane | 5.5 | 4.1 |  |  |  | 5.6 | 6.45 | 5.75 | 4.95 | 12.5 | 9.0 |
| 80.6 | 4305 | 80.7 | Cyclohexane | 3.9 | 5.1 |  |  |  | 5.8 | 4.9 | 4.45 | 0.15 | 2.5 | 1.2 |
| 80.1 | 867 | 80.0 | Benzene | 12.1 | 8.6 | 8.8 | 7.5 | 0.5 | 4.1 | 7.8 | 5.7 | 1.4 | 4.4 | 1.4 |
| 79-80.6 | 8760 8762 | 79.2 80.4 | 2,2 & 2,4 dimethyl pentane |  | - | - |  |  | - | - | - |  | 0.25 | 0.5 |
| 86.0 | 8763 | 86.0 | 3,3 dimethyl pentane |  | - | - | 0.14 |  | - |  |  |  | - | - |
| 88.6 | 4800 | 87.5 | 1,1 dimethyl cyclopentane |  | - | - | 0.12 |  | - |  |  |  | - | - |
| 89.7 * | 8761 | 89.7 | 2,3 dimethyl pentane |  | 0.6+ |  | 1.02 |  | - |  |  |  | 0.3 | 0.5 |
| 89.9 | 6834 | 90.0 | 2 methyl hexane |  | 0.6+ | 0.6 | 0.09 |  | 1.8 | 1.3 | 1.6 |  | 1.5 | 3.3 |
| 90.6 | 4803 (cis) 4804 (trans) | 90.8 91.7 | 1,3 dimethyl cyclopentane |  | 0.6+ |  | 1.4 |  |  |  | 3.1 |  | 5.8 | 4.7 |
| 91.8 | 6835 | 92 | 3 methyl hexane |  |  | 0.4 | 0.5 |  | 1.2 | 2 | 2.8 |  | 3.1 | 4.2 |
| 93.4 | 8809 | 93.5 | 3 ethyl pentane |  | - | - | - | - | - | - | - |  | - | - |
| 98.4 | 6355 | 98.5 | n-heptane |  |  |  | Trace | 0.25 |  |  |  |  | 0.4 | 0.9 |
| 99.2* | 8848 | 99.2 | 2,2,4 trimethyl pentane | 12 | 11.5 | 8.0 | 12.1 | 13.1 | 2.5 | Trace |  |  | - | - |
| 91.8, 99.4 | 4802 (trans) 4801 (cis) | 91.9 99.5 | 1,2 dimethyl cyclopentane |  |  |  |  |  |  |  |  |  | 3.2 | 2.5 |
| 100.6 | 4426 | 100.9 | Methyl cyclohexane |  |  | 3.4 | 2.8 | 1.65 | 8.5 |  |  |  | 0.6 | 5.1 |
| 103.4 | 4812 | 103.5 | Ethyl cyclopentane |  |  |  |  | 0.4 |  |  |  |  | 1.5 | 1.9 |
| 106.6 * | 6771 | 106.8 | 2,2 dimethyl hexane |  |  |  |  | 0.25 |  |  |  |  | - | Trace |
| 109.4 * | 6773 | 109.5 | 2,4 dimethyl hexane |  |  |  | 0.32 | 0.10 |  |  |  |  | 0.7 | 0.6 |
| 115.6 * | 6772 | 115.6 | 2,3 dimethyl hexane |  |  |  |  | 0.23 |  |  |  |  | 0.4 | 0.7 |
| 110.6 | 1947 | 110.6 | Toluene (methyl benzene) | 11.5 | 12.5 | 12.5 | 10.1 | 10.4 | 7.3 | 6.7 | 5.8 | 4.95 | 5.5 | 4.4 |
| 109.8 * | 8847 | 110 | 2,2,3 trimethyl pentane | trace | - | - | 0.6 | 1.2 | - |  |  |  | - | - |
| 113.5 * | 8850 | 113.5 | 2,3,4 trimethyl pentane | 5 | 4.6 | 8.0 | 4.6 | 4.1 | 2.0 |  |  |  | - | - |
| 114.8 * | 8849 | 114.8 | 2,3,3 trimethyl pentane |  | 0.5 | - | 0.6 | 0.7 | 1.8 |  |  |  | - | - |
| 109-115 | 4860 | 109.3 | Trimethyl cyclopentane (1,2,4) |  |  |  | 0.3 |  |  |  |  |  | 1.8 | 2.0 |

The detailed analyses of the two ‘normal’ hydrogenation gasolines (Nos. 1 & 6 of Table X) can be expressed to show the type of non-aromatic hydrocarbons. The following figures clearly show that the bulk of the paraffins are single branch or the 2 methyl type and about two-thirds of the total naphthenes are cyclopentanes. The proportionate composition is very similar for two widely different raw materials.

Table 33. Hydrogenation Gasoline Composition non-aromatic compounds

| Hydrocarbon | Ex Creosote middle oil | Ex Petroleum gas oil |
| --- | --- | --- |
| Paraffins |  |  |
| n-butane | 0.2 | 0 |
| n-pentane | 4.6 | 4.0 |
| n-hexane | 1.8 | 2.0 |
| n-heptane | 0.4 | 0.9 |
| n-octane | 0.2 | 0.3 |
| Total | 7.2 | 7.2 |
| 2 methyl butane | 17.9 | 22.9 |
| 2 methyl pentane | 5.6 | 7.9 |
| 2 methyl hexane | 1.5 | 3.3 |
| 2 methyl heptane | 0.75 | 0.7 |
| 3 methyl pentane | 3.8 | 4.6 |
| 3 methyl hexane | 3.1 | 4.2 |
| 3 methyl heptane | 1.5 | 2.0 |
| 4 methyl heptane | 0.1 | 0.5 |
| 3 ethyl pentane | Absent | Absent |
| 3 ethyl hexane | Absent | Absent |
| Total | 34.2 | 46.1 |
| 2,2 dimethyl propane | Absent | Absent |
| 2,2 dimethyl butane | 0.15 | 0.3 |
| 2,3 dimethyl butane | 1.2 | 2.4 |
| 2,2 dimethyl pentane | 0.05 | 0.1 |
| 2,4 dimethyl pentane | 0.2 | 0.4 |
| 3,3 dimethyl pentane | Absent | Absent |
| 2,3 dimethyl pentane | 0.3 | 0.5 |
| 2,2 dimethyl hexane | Absent | Trace |
| 2,5 dimethyl hexane |  | 0.66 |
| 2,4 dimethyl hexane | 0.7 |  |
| 2,3 dimethyl hexane | 0.4 | 0.7 |
| 3,3 dimethyl hexane |  |  |
| 3,4 dimethyl hexane |  |  |
| Dimethyl heptanes | Ca 0.5 | Ca 1.1 |
| Methyl ethyl pentanes | Absent | Absent |
| Total | 3.5 | 5.15 |
| Total Paraffins | 47% | 61% |
| Approx. % of total paraffins as: |  |  |
| Straight chain | 16 | 12 |
| Single branch | 76 | 79 |
| Double branch | 8 | 9 |
| Triple branch | 0 | 0 |
| Naphthenes |  |  |
| Cyclopentane | 0.8 | 0.9 |
| Methyl cyclopentane | 12.0 | 9.0 |
| Ethyl cyclopentane | 1.5 | 1.9 |
| n-propyl cyclopentane | Traces? | Traces? |
| Total cyclopentanes | 14.3 | 11.8 |
| Cyclohexane | 2.5 | 1.2 |
| Methyl cyclohexane | 6.5 | 5.2 |
| Ethyl cyclohexane | 0.8 | 0.6 |
| n-propyl hexane | Traces | Traces |
| Total cyclohexanes | 9.8 | 7.0 |
| 1,1 dimethyl cyclopentane | Absent | Absent |
| 1,3 di methyl cyclopentane trans | 5.2 | 4.5 |
| 1,3 di methyl cyclopentane cis (?) | 0.6 | 0.5 |
| 1,2 di methyl cyclopentane trans | 2.6 | 2.4 |
| 1,2 di methyl cyclopentane cis | 0.35 | 0.2 |
| 1 methyl 3 ethyl cyclopentane | 2.4 | 2.0 |
| 1 methyl 2 ethyl cyclopentane |  |  |
| Total (DiMe, MeEt cyclopentanes) | 11.15 | 9.6 |
| 1,1 dimethyl cyclohexane | Absent | Absent |
| 1,2 di methyl cyclohexane trans | - | 0.2 |
| 1,2 di methyl cyclohexane cis | Traces |  |
| 1,3 di methyl cyclohexane trans | 2.7 | 2.0 |
| 1,3 di methyl cyclohexane cis | 1.4 | 0.9 |
| 1,4 dimethyl cyclohexane trans | 0.65 | 0.4 |
| 1,4 dimethyl cyclohexane cis | 0.35 | 0.2 |
| Isopropyl cyclohexane | Absent | Absent |
| Total (DiMe Cyclohexanes) | 5.1 | 3.7 |
| Trimethyl cyclopentanes (1,2,4?) | Ca 1.8 | Ca 2.0 |
| Total Naphthenes |  |  |
| % of total naphthenes as: | 40% | 32 ½ % |
| Cyclopentanes | 65 | 69 |
| Cyclohexanes | 35 | 31 |
| Non-substituted | 8 | 6 |
| Mono-alky | 49 | 49 |
| di-alkyl | 39 | 39 |
| Tri-alkyl | 4 | 3 |

Figure 5. German Aviation Gasoline Green C3 – T.B.P. curves.

Figure 6. German Blue B4 Composition by Aniline Point.

Figure 7. German Aviation Gasoline Green C3 GF32

Figure 8. German Aviation Gasoline AIR285

Figure 9. German Aviation Gasoline AIR 328-330

Figure 10. Aniline Point- Boiling Point Curve for Aromatic-Free Fractions.

Figure 11. Boiling point fractions German Aviation Gasolines Green C3- various.

Figure 12. Boiling point fractions German Aviation Gasolines Blue B4 – various.

Figure 13. Boiling point fraction German Aviation Gasolines GF29, GF30, No.67

Figure 14. Boiling point fractions German Aviation Gasolines No.58 & ex Junkers JU-88 crashed.

Figure 15. Boiling point fractions Lagunillas & Venezuelan Aviation Gasolines.

Figure 16. Boiling point fractions of 130 deg. C end point Petrol from Hydrogenation of Gas Oil

Figure 17. Boiling point fractions of 138 deg. C end point Petrol from Hydrogenation of Creosote.

Figure 18. Boiling Point Fractions of 200 deg. C end point Petrol from Hydrogenation of Creosote.

# REFERENCES

| Ref. No | General Analytical Reports | Author | Date Issue |
| --- | --- | --- | --- |
| 1. | Report G.40/2 Report on Examination of Enemy Aviation Fuels (Summary to end of No. 1940) | R. Holroyd D. Peel | 7/12/40 |
| 2. | Report G.40/2 Report on Examination of Enemy Aviation Fuels. Second Report | R. Holroyd D. Peel | 12/3/41 |
| 3. | Detailed examination of Enemy Fuels – Discussion of Analytical Methods | R. Holroyd D. Peel | 11/2/41 |
| 4. | Report VI. “The Billingham Method of Analysis of Petrols” | D.H. Peel | 31/5/43 |
| 5. | “Examination of Fuels by Infra-Red & Ultra Violet Spectroscopic Methods. Summary of results obtained up to July 1st 1942” | D. Peel | 10/8/42 |
| 6. | “Report on Examination of Fuels by Infra-Red & Ultra Violet Spectroscopic Methods. Second Summary covering period July 1st 1942 – 1st December 1942” | D. Peel | 29/12/42 |
| 7. | Summary of Work on Infra-Red Panel |  | Dec 1942 |
| 8. | Report II Analysis of Synthetic Octanes by Spectroscopic Methods | G.B. Sutherland | June 1943 |
| 9. | Report III, Part I Analysis of Aviation Fuels by Spectroscopic Methods Pt. I range 20O to 100OC | H.W. Thompson | June 1943 |
| 10. | Report III, Part II Analysis of Aviation Fuels by Spectroscopic Methods Pt. II range above 95OC | G.B. Sutherland | June 1943 |
| 11. | Report IV. Detection & Estimation of Aromatics in Hydrocarbon Mixtures. | W.C. Price G.B. Sutherland H.W. Thompson | June 1943 |
| 12. | Report V. Analysis of Mixtures containing Ethyl Benzene & the Three Xylenes by U.V. Method. | A.I.O.C. No.2196 | June 1943 |

Individual Fractionation Analysis Reports

Reference Samples of Known Origin	(a) Virgin – Straight Run

| Ref. No. | Report No. |  |  |  |
| --- | --- | --- | --- | --- |
| 13. | G.40/1 | Roumanian aviation gasoline | (see ref. 1) |  |
| 14. | G.40/1 | Roumanian aviation gasoline (8247) | (see ref. 1) |  |
| 15. | R.41/44 | Roumanian aviation gasoline (8247) | D. Peel | 20/12/41 |
| 16. | G.40/1 | Venezuelan aviation gasoline (8080) | (see ref. 1) |  |
| 17. | R.41/10 | Venezuelan ‘A’ | D. Peel | 19/5/41 |
| 18. | R.41/11 | Venezuelan ‘B’ | D. Peel | 19/5/41 |
| 19. | R.41/32 | Lagunillas gasoline | D. Peel | 29/8/41 |
| 20. | R.41/43 | Lagunillas light naphtha | D. Peel | 18/12/41 |
| 21. | R.42/49 | Lagunillas heavy naphtha | D. Peel | 8/9/42 |
| 22. | R.41/12 | Trinidad high octane light cut | D. Peel | 27/6/41 |
| 23. | G.40/1 | Californian aviation (8127) | (see ref. 1) |  |
| 24. | R.41/15 | Midway light blending stock | D. Peel | 30/6/41 |
| 25. | R.41/14 | Kettleman light blending stock | D. Peel | 30/6/41 |
| 26. | G.40/1 | Shell Aviation 73 octane | (see ref. 1) |  |
| 27. | - | Shell light blending cut S.R.164 |  | - |
| 28. | R.41/31 | Miri aviation petrol |  | 28/8/41 |
| 29. | R.41/35 | Borneo aviation |  | 1/9/41 |
| 30. | - | Product from Sumatra (Palembang) |  | (not reported) |
| 31. | R.41/28 | Reitbrook gasoline (Hamburg, Germany) |  | 20/8/41 |
| 32. | R.42/50 | Reitbrook naphtha (Hamburg, Germany) | D. Peel | 8/9/42 |
| 33. | R.41/41 | Russian fuel AIR151 | D. Peel | 12/11/41 |
| Ref. No. | Report No. |  | Author | Date issue |
| (b) Miscellaneous |  |  |  |  |  |
| 34. | R.43/71 | Catalytically cracked I.A.A. 110 (Intava) | D. Peel | 4/2/43 |
| 35. | R.41/40 | Fischer-Tropsch | D. Peel | 13/10/41 |
| (c) Aromatised & other aromatic products |  |  |  |  |  |
| 36. | R.41/36 | Hydroformed East Texas naphtha | D. Peel | 10/10/41 |
| 37. | R.41/37 | Hydroformed Iraqi naphtha | D. Peel | 10/10/41 |
| 38. | R.41/38 | Hydroformed Iranian No.1 Naphtha | D. Peel | 10/10/41 |
| 39. | R.41/39 | Hydroformed Iranian No.2 Naphtha | D. Peel | 10/10/41 |
| 40. | R.42/54 | Hydroformed hydrocreosote 130-200OC naphtha | D. Peel | 6/11/42 |
| 41. | R.42/62 | Forerunnings from naphthalene hydrogenation | (not yet issued |  |
| 42. | R.42/63 | High temp. hydro-aromatised product from creosote | D. Peel | 5/10/42 |
| 43. | R.42/51 | Gas spirit from bituminous coal carbonisation | D. Peel | 8/9/42 |
| 44. | R.42/52 | Coke oven spirit from bituminous coal carbonisation | D. Peel | 8/9/42 |
| (d) Hydrogenation gasolines & naphthas from coal & creosote |  |  |  |  |  |
| 45. | R.42/58 | 30% volatility 200OC end point | D.H. Peel | 15/9/42 |
| 46. | G.40/1 | 50% volatility 160OC end point | (see ref. 1) |  |
| 47. | G.40/1 | 70% volatility 140OC end point | (see ref. 1) |  |
| 48. |  | 75% volatility 130OC end point | (see ref. 4, 9&10) |  |
| 49. | R.42/61 | Total saturated product | D. Peel | 24/9/42 |
| From petroleum gas oil |  |  |  |  |  |
| 50. | R.42/59 | 40% volatility 200OC end point | D. Peel | 5/10/42 |
| 51. | G.40/1 | 70% volatility 130OC end point | (see ref. 1) |  |
| 52. | R.42/60 | 70% volatility 120OC end point | D. Peel | 16/9/42 |
| 53. |  | 70% volatility 130OC end point (42/1/37) | (see ref. 4, 9&10) |  |
| 54. | G.40/1 | 67% volatility (ex Intava) | (see ref. 1) |  |
| 55. | R.43/13 | 56% volatility (ex hydrogenated Albanian oil) | D. Peel |  |
| (e) Synthetic octanes |  |  |  |  |  |
| 56,57 | R.42/64 &70 | Aruba Alkylate (also see Refs.) | D. Peel | 5/10/42 |
| 58. | R.42/67 | Baton Rouge Alkylate (also se refs. 6,7) | D. Peel | 8/11/42 |
| 59. |  | Baton Rouge hydrocodimers (see refs. 6,7) | (see Ref. 7) |  |
| 60. |  | U.K. Phosphoric acid Hydrocodimer (H.G.G.) | (see Refs. 6, 7) |  |
| 61. |  | U.K. Phosphoric acid Hydrocodimer (43/3/69) | (see Ref. 7) |  |
| 84. | R.42/48 | Synthetic Phenol, issued 25/1/42 |  |  |

# Appendix – Additional information

PONA and distillation curves for German B4 and C3 aviation gasolines.

Graph 1. PONA Distribution of German Blue B4 Aviation Gasoline

Note olefins <1% and the high levels of paraffins 43% and naphthenes 46%, while aromatics are only 11%.

Graph 2. PONA Distribution of German Green C3 Aviation Gasoline

Note olefins <1% and the high level of aromatics 41%, with paraffins 33%, and naphthenes 26%. The high level of aromatics was required to meet the higher octane ratings.

.

Graph 3. Distillation Curves for German Blue B4 and Green C3 aviation gasolines.

German Blue B4 is sample AIR118 from a crashed Messerschmitt Me-109 on 8th May 1941.

German Green C3 is the sample from a crashed Junkers Ju-88 on 16th Feb. 1941.

# Index

1

1 methyl 2 ethyl cyclopentane	82

1 methyl 3 ethyl cyclopentane	82

1,1 dimethyl cyclohexane	82

1,1 dimethyl cyclopentane	79, 82

1,2 di methyl cyclohexane cis	82

1,2 di methyl cyclohexane trans	82

1,2 di methyl cyclopentane cis	82

1,2 di methyl cyclopentane trans	82

1,2 dimethyl cyclopentane	69, 80

1,2,4 Trimethyl cyclopentane	80

1,3 di methyl cyclohexane cis	82

1,3 di methyl cyclohexane trans	82

1,3 di methyl cyclopentane cis	82

1,3 di methyl cyclopentane trans	82

1,3 dimethyl cyclopentane	80

1,4 dimethyl cyclohexane cis	83

1,4 dimethyl cyclohexane trans	83

100 Octane	6

100 octane grade	7

2

2 methyl butane	81

2 methyl heptane	81

2 methyl hexane	48, 69, 80, 81

2 methyl pentane	48, 79, 81

2, 3 dimethyl pentane	69

2,2 branched paraffins	48

2,2 dimethyl butane	44, 48, 81

2,2 dimethyl hexane	21, 24, 42, 44, 46, 47, 80, 81

2,2 dimethyl pentane	44, 48, 81

2,2 dimethyl propane	81

2,2, dimethyl butane	79

2,2,3 trimethyl pentane	42, 47, 49, 80

2,2,4 trimethyl pentane		5, 6, 10, 11, 18, 16, 21, 24, 41, 42, 46, 47, 56, 57, 59, 67, 68, 70, 71, 80

2,2,4,6,6 pentamethyl heptane	67

2,2,5 trimethyl hexane	42, 43, 44, 48, 49

2,2,5 trimethyl pentane	11

2,2,6 trimethyl heptane	44

2,3 dimethyl butane	44, 48, 49, 79, 81

2,3 dimethyl hexane	44, 49, 80, 81

2,3 dimethyl pentane	11, 42, 43, 44, 48, 49, 59, 70, 79, 81

2,3,3 trimethyl pentane	11, 42, 46, 47, 49, 80

2,3,4 trimethyl pentane	10, 11, 16, 41, 42, 46, 47, 49, 56, 59, 71, 80

2,3,5 trimethyl hexane	44

2,4 dimethyl hexane	44, 47, 69, 80, 81

2,4 dimethyl pentane	44, 48, 57, 60, 79, 81

2,5 dimethyl hexane	44, 81

3

3 ethyl hexane	81

3 ethyl pentane	80, 81

3 methyl heptane	81

3 methyl hexane	69, 80, 81

3 Methyl hexane	48

3 methyl pentane	79, 81

3 Methyl pentane	48

3,3 dimethyl hexane	44, 82

3,3 dimethyl pentane	79, 81

3,3 Dimethyl pentane	48

3,4 dimethyl hexane	82

3-C Rich Mixture rating	6, 9, 15, 16

3-C Rich Mixture Supercharge rating	3

3-C Rich Rating	12

4

4 methyl heptane	81

7

73 octane Roumanian	50

9

90 C.F.R. motor octane	8

A

acid treatment	66, 70

acid washing	47

adsorption	70

aero engine tests	6

aero engines	12

Air Ministry	4

Alcohols	13, 18

alkyl cyclohexanes	53

alkyl cyclopentanes	53

alkylate	5, 42, 49

alkylates	10, 11, 42, 44

alkylation	27, 28, 49

Allies	3

American Petroleum Institute	44

Anglo-American Oil Co.	4

Anglo-Iranian Oil Company	31

Aniline	13

aniline point	14, 23, 25, 38, 41, 59, 68, 69, 70, 71, 78, 84, 86

aniline point method	32, 34, 41, 42, 45, 50, 52, 62, 70

anti-knock additives	18

Anti-knock Blending Additives	13

aromatic	12, 14, 15, 16, 25, 26, 29, 51, 54

aromatic amines	13, 18

aromatic extract	27

aromatic free	9

aromatic gasoline	62

aromatic hydrocarbons	6, 9

aromatic materials	65

aromatic naphtha	5, 6, 10, 11, 12, 29, 56

Aromatic oils	26, 38

aromatic–free	45

Aromatic-free Aniline Point	14

aromatic-free fractions	41

aromatics	6, 7, 9, 10, 16, 17, 20, 23, 25, 27, 28, 29, 31, 38, 51, 52, 53, 54, 55, 56, 61, 66, 68, 70, 74, 76, 78

aromatisation	10, 27, 28, 67

aromatised	65

Aromatised creosote	36

aromatised naphtha	30, 49, 51, 53, 54, 55, 56, 67

aromatising	5, 6, 7, 11, 28, 30, 51, 52, 62

aromatising hydrogenation	56

aromatising-splitting	13

Aruba	44

Asiatic Petroleum Co.	4

aviation fuel	6, 7, 9, 11, 52

aviation gasoline	3, 5, 6, 7, 8, 11, 25, 65, 66, 68

aviation gasolines	72

aviation grade gasoline	62

aviation petrols	26

Axis	3

azeotrope	69

azeotropic distillation	42, 45, 47, 70

B

B.M.W. 801 C	7

B4 blue	8

B4 blue aviation fuel	49

B4 blue gasoline	9

B4 gasoline	8

Barton	4

base fuel	41, 49, 51, 61

base gasoline	51

Base Spirit	9

Bass	4

Baton Rouge	44

Battle of Britain	5, 7, 18

benzene	6, 10, 16, 17, 20, 23, 25, 27, 29, 30, 32, 38, 52, 55, 56, 60, 67, 69, 75, 79

Benzocyclopentane	17

benzol	52, 56

benzole	6, 28, 30, 51

Billingham	3, 16, 18, 20, 23, 25, 42, 65, 78

bituminous	10, 26, 27, 51, 56

bituminous coal	28

bituminous material	5, 10

blending agents	6

Blue B4	84, 87

blue B4 aviation gasoline	65

blue B4 fuel	6

Blue B4 gasoline	71, 79

BMW 801D	19

boiling point	50, 71, 86

bomber	8

Borneo	27

Borneo ‘C’	35, 38

Brake Mean Effective Pressure	7

Britain	13

British	7

British standard test	6

British War Cabinet	3

Bromine number	14, 20, 23

brown coal	28

brown coal tar	27, 28

butane	6, 9, 14, 16, 43, 44, 49, 54, 67, 75

butyl benzene	17

butylene polymerisation	5

C

C.F.R. Motor	9, 12, 16, 20, 23, 31

C.F.R. motor method	6

C.F.R. Motor Rating	3, 8, 12, 15, 62

C10 aromatics	32, 67

C10+ Aromatics	20, 23

C3 Aviation Gasoline	7

C3 gasoline	10, 11

C3 Green	45

C3 type	9, 10, 13

C3-C5 alkylate	43

C4 Alkylate	11, 42, 47

C4 Cold Acid	47

C4 gases	11

C4 Hot Acid	47

C4 hydrocodimers	42, 43

C4-C5 alkylate	43

C4-light petrol alkylate	43

C6 hydrocarbons	51, 67

C6 naphthenes	50, 51, 52, 53

C6 paraffins	53, 67, 75

C7 naphthenes	67

C8 aromatics	16, 17, 20, 23, 25, 27, 29, 32, 38, 67, 71, 75

C8 naphthenes	51, 67, 75

C8 paraffins	67

C9 aromatics	20, 23, 26, 32, 38, 67

C9 naphthenes	61, 67

C9 paraffins	67

Californian	34

Cambridge	24

carbon formation	28

catalyst	27, 28, 49, 62

catalyst vapour phase	6, 12

catalyst vapour phase hydrogenation	10

catalysts	10, 53

catalytic cracking	27

Catalytic Dehydrogenation	36

catalytically cracked gasoline	10

Catalytically Cracked Petroleum	37

Channel Islands	72

Chilvers	4

coal	10, 38

Coal Carbon	37

coal carbonisation	10, 26, 27

coal creosote	10

coal tar aromatics	11

coal tar benzole	27

Coke Oven ‘Xylenes’	37, 38

coke oven aromatics	27

coke oven benzole	27, 28

coke oven products	65

Coke Oven spirit	37, 38

coke-oven aromatics	10

cold acid	10

cold acid type	43

cold sulphuric acid process	11

Colour	9, 13, 19, 22

cracking	25, 27, 28

creosote	14, 18, 26, 27, 28, 53, 63, 7279, 89, 90

Creosote ‘saturated’	35

Creosote Heavy Oil	59, 62, 63

Creosote Middle Oil	35, 59, 62, 81

creosote oil	28

creosote-hydrogenation naphthas	10

cross-polymers	43

Cumene	13, 18, 28

cyclisation	27

cyclohexane	10, 17, 24, 28, 29, 30, 50, 51, 52, 53, 55, 56, 58, 60, 67, 68, 71, 75, 79, 82

cyclohexane naphthenes	27

cyclopentane	30, 53, 55, 58, 60, 67, 69, 79, 81, 82

D

D.B. 601 E	8

Daimler Benz 605B	8

Daimler Benz DB 601 N	7

DB 601	19

decalin	25

decomposition	42

dehydrogenate	55

dehydrogenated	56

dehydrogenation	5, 10, 11, 12, 26, 27, 28, 29, 30, 39, 49, 51

Density	9, 12, 13, 19, 22, 76

destructive aromatisation	53

destructive hydro aromatisation	51

destructive hydrogenation	5, 10, 26, 52

Dewdney	4

dicyclic	28

Diesel oils	65

dimethyl cyclohexane	30, 51, 60, 67

dimethyl cyclopentane	29, 30, 50, 51, 53, 58, 67, 68, 75

Dimethyl heptanes	82

dimethyl hexanes	11, 41, 42, 43, 49, 57, 58, 60, 67

distillation	19, 22, 52, 62

Distillation	19, 22

distillation still	66

dodecanes	59

Dornier Do-17Z	31

E

East Texas Naphtha	36, 39

Edeleanu extraction	27

Egypt	19

End point	9

Engine	22

ethers	13, 18

ethyl benzene	10, 17, 21, 24, 25, 26, 27, 28, 29, 30, 31, 33, 38, 55, 60

ethyl cyclohexane	30, 51, 58, 60, 68, 70, 82

ethyl cyclopentane	53, 60, 80, 82

ethyl toluene	17, 26, 29, 30, 38, 55, 67

ethyl xylenes	26

F

fighter grade fuel	8

Final Boiling Point	12, 13, 19, 22, 62

Fischer-Tropsch	6, 10, 27, 28

fluorescent	9, 13, 22

Focke Wolf	7, 8

Foreign Office	4

fractionating columns	66

fractionation	5, 7, 16, 18, 25, 44, 45, 50, 56, 66, 70, 71

fractionation analysis	65

fractionation method	41

freezing	17

FW 190	7, 8, 19, 22

G

Garner	4

gas oil	10, 26, 35, 39, 59, 62, 63, 72, 79, 81, 89

Gas spirit	37, 38

gasoline	26, 29

Gelsenkirchen	28

general purpose fuel	8

German Air Force	8

German C3 (Green)	3

German C3 (Green) Aviation Gasoline	32

German green C3 type	26

Germany	6

Glasgow	44

Grade C3	3

Green (C3) aviation gasoline	7

green aviation fuel	52, 54, 66

Green C3	83, 84, 86

green C3 aviation gasoline	65, 67

green C3 fuel	6, 12

Green C3 gasoline	8, 59, 63, 71, 79

green C3 type	25, 76

green fuel	6, 7, 14, 15, 18, 27, 28, 29, 30, 41, 49, 50, 53, 55, 56

green gasoline	31, 51, 53, 62, 69, 71, 72, 78

Guernsey	72

gum deposits	14

H

Hardiman	4

Heinkel He-111	43

heptanes	64, 67

Herbert	4

hexane	10, 50, 53, 54, 56, 64, 71

Heysham	43, 44, 49, 65

Holroyd	4

hot phosphoric acid polymerisation	11

Howes	4

hydrindene	17, 21, 24, 25, 26, 28, 33, 38, 61, 67

hydro Albanian	36

Hydro creosote	36, 39

hydro gasoline	27, 53, 72

hydro petrol	72

Hydro petroleum (SO2) Extract	36, 39

Hydro petroleum gasoline	36

hydro product	27, 51

hydro trimer	56, 58, 59

hydro-aromatisation	27, 49, 53

hydrocodimers	10, 42, 44, 49

hydro-cracking	27

hydrofined	65

hydrofining	10, 11, 26, 27, 39, 27

hydroform	11

hydroformed	30, 31, 36, 39, 56, 65

hydroforming	7, 11, 12, 26, 27, 29, 30, 31, 55

hydro-gasoline	71

hydrogen	6, 28

hydrogen carbon ratio	7

hydrogenate pitch	28

hydrogenated co-polymer	11

Hydrogenated Creosote	39

Hydrogenated petroleum	39

hydrogenated trimer	49, 50, 67, 71

hydrogenation	5, 6, 7, 10, 11, 18, 25, 26, 27, 28, 39, 52, 53, 56, 59, 62, 63, 79, 89, 90

Hydrogenation (Bituminous)	52

hydrogenation gasoline	50, 51, 62, 65, 72, 81

hydrogenation naphthas	10

hydrogenation petrols	50

hydrogenation plants	10, 11

hydrogenation process	52

hydrogenation-splitting process	13

hydro-naphtha	11, 26, 27

hydro-petrol	18, 51, 52, 53, 69, 71

hydro-product	27

hydro-refined benzene	10

hydro-refined benzole	56

hydro-refining	6, 27

hydroreformed	10

hydro-trimer	43, 48

I

I.C.I.	21

I.G Farbenindustrie	28

Imperial Chemical Industries	3, 4

infra-red	41, 42, 44, 69, 70

infra-red method	45, 47

infra-red spectroscopic	66

Intava	36

Iranian Naphtha	31

Iranian naphtha No.1	36, 39

Iranian naphtha No.2	36, 39

Iraqi naphtha	36

Iron Carbonyl	13, 18

iso-butane	19, 22, 57, 59

isobutylene	11

iso-heptane	51, 69, 70, 75

iso-hexane	52, 67, 69, 75, 79

isomerisation	27, 42, 53

Iso-octane	6

iso-paraffins	50, 53, 64

iso-pentane	6, 12, 14, 19, 52, 58, 60

isopropyl benzene	28

Isopropyl cyclohexane	83

J

Jersey	72

Junker JU-87 Stuka	3

Junkers JU-88	18, 19, 22, 88

K

Kettleman light cut	34

Knock Rating	15, 16, 20, 23, 31, 54

L

Lagunillas	69, 72, 88

Lagunillas gasoline	35

Lagunillas heavy naphtha	38

Lagunillas light naphtha	34, 38

Lead	9, 13, 15

lean mixture	6

Lean rating	3, 5, 29, 54

light cut gasoline	5, 10

light gasoline	10

light paraffinic petrol	15

light petrol	5, 12, 13, 14, 15, 29, 31

light-cut	29, 52, 53, 56

liquid phase	52, 53

low volatility	5, 9, 10

Lt.-Col. S.J.M. Auld	4

Luftwaffe	72

M

Magdeburg	28

Me 109-E7	19

Me 109-F	8, 19

Me 110	19

Messerschmitt Me-110	2

Messerschmitt Bf-109	8, 72

Messerschmitt Bf-109-G	1

meta poly methyl benzenes	54

meta-xylene	26, 29

methanol	42

methyl aniline	13

methyl benzene	80

m-Ethyl benzene	33

methyl cyclohexane	30, 51, 58, 60, 67, 68, 70, 71, 80, 82

methyl cyclopentane	 17, 20, 23, 29, 30, 50, 52, 53, 56, 58, 60, 64, 67, 69, 71, 75, 79, 82

methyl ethyl pentanes	82

methyl heptanes	50, 57, 60, 67

methyl hexanes	44, 57, 60

methyl pentane	44, 57, 60

m-ethyl toluene	54, 61

Middle East	8, 13, 22

middle oil	28

Midway light cut	34

Ministry of Aircraft Production	4

Ministry of Economic Warfare	4

Ministry of Fuel and Power	4

Ministry of Supply	4

Miri aviation	34

mono-alkyl benzenes	25, 26, 29, 54

monocyclic aromatics	25

Motor benzole	53

motor gasoline	6, 65

motor spirit	28

m-xylene	27, 31, 33, 54, 55, 61

N

naphtha	10, 12, 13, 26, 27, 28, 51, 65

naphthalene	10, 25, 26, 28, 33

naphthenes	5, 6, 9, 10, 11, 12, 15, 16, 20, 23, 27, 28, 29, 30, 31, 50, 51, 52, 53, 54, 55, 56, 58, 61, 63, 64, 66, 67, 68, 69, 70, 71, 75, 76, 81

naphthenic	11

naphthenic naphtha	27

National Bureau of Standard	44

natural naphthas	26

natural petroleum gasoline	34, 50, 65

natural petroleum gasolines	50

n-butane	19, 22, 60, 81

n-butyl benzene	10, 25, 26, 27, 28, 29, 33, 38, 61, 67

neohexane	79

n-heptane	60, 70, 71, 80, 81

n-hexane	52, 60, 67, 69, 75, 79, 81

n-octane	50, 58, 60, 67, 81

nonanes	50

non-aromatics	54

Nordstern	28

normal alkyl benzenes	26

normal butane	49, 57

normal heptane	41, 57

normal hexane	57

Normal Hydro	69

Normal hydrogenation gasolines	35

normal paraffins	53

normal pentane	57

Normandy	72

n-pentane	19, 52, 60, 81

n-propyl benzene	10, 25, 26, 28, 29, 30, 33, 38, 55, 61, 67

n-propyl cyclopentane	82

n-propyl hexane	82

O

octane	5, 6, 13, 14, 29, 31, 42, 43, 47, 49, 53, 54, 56, 62, 67

octane number	41

Octane Rating	9

octanes	12, 15, 18, 29, 42, 45, 47, 49, 67, 70, 71, 78

olefine	28

olefines	28, 49

Oriel	4

ortho-xylene	17, 29, 30

Oxford	21

o-xylene	30, 31, 33, 55, 61

P

p Xylene	30

Palembang	34

paraffinic	6, 11

paraffins	15, 18, 20, 23, 27, 29, 41, 43, 44, 47, 49, 50, 51, 52, 53, 55, 56, 61, 63, 66, 68, 69, 71, 76, 81, 82

para-xylene	29

Peel	4

pentamethyl heptane	43

pentane	9, 10, 12, 13, 14, 16, 49, 51, 54, 34, 44, 50, 53, 56, 67, 68, 75

p-Ethyl benzene	33

p-ethyl toluene	61

petrol	28, 39

Petroleum Attaché’s Office	4

Petroleum Board	4

petroleum gasoline	72

petrols	65

phenol	52, 70

phosphoric acid	6, 42, 49

phosphoric acid process	56

phosphoric acid type	52

pitch	28

poly alkyl benzenes	26

poly cyclic	28

polycyclic aromatics	28

polymerisation	6, 43

propane	43, 49

pseudo Cumene	26, , 61

p-xylene	31, 33, 55, 61

R

Randolph	4

recycle	62

refractionation	42

refractive index	68, 78

Reid Vapour Pressure	9, 13, 20, 23, 62

Reitbrook	34

Reitbrook gasoline	38

Reitbrook naphtha	38

Research ON	16

Rich 3-C	12, 31

rich mixture	6, 8

rich mixture rating	3, 5, 7, 29, 31, 54

Ridler	4

Roumanian	18, 35, 38, 69

Roumanian 73 Octane	52, 59, 63, 79

Roumanian Aviation	62

Roumanian gasoline	51

Russia	1, 19

Russian	8, 13, 34, 38, 43

S

Schelven	28

secondary butyl benzenes	28

Shell	21, 24

Shell 73 Octane	35

Shell light cut	35

silica gel	70

Specification	8

spectroscopic	45

spectroscopic analyses	9, 18, 41, 50, 62, 69, 78

spectroscopic method	25, 26, 59, 68, 69

spectroscopy	29, 44

splitting	27, 51

Standard Indiana	44

Standard Oil	44

Stettin	28

still bottoms	28

storage stability	14

Straight Run Petroleum	18, 38

straight run petroleum spirits	50

sulphonation	25, 68, 70

sulphur	11, 28

Sulphur dioxide	70

sulphuric acid	42

Sumatra	34

super-fractionation	9, 68

synthetic blend	13, 69

synthetic octane	5, 6, 9, 10, 11, 28, 42, 44, 50, 51, 52, 56, 65, 70

synthetic rubber	28

T

T.E.L.	12, 15, 16, 20, 23

tar acids	62

tar bases	62

tertiary butyl benzene	13, 18

Tetra Ethyl Lead	3, 9

tetralin	25, 26

Tett	4

Texas	44

theoretical plates	66

Thole	4

Thompson	4

toluene	16, 20, 23, 25, 28, 29, 30, 32, 41, 42, 45, 47, 60, 67, 70, 71, 75, 80

trimer	49

Trimethyl Benzene	38

trimethyl cyclopentanes	60, 83

trimethyl hexanes	67

trimethyl pentane	18, 21, 24, 41, 42, 44, 47, 49, 50, 56, 67, 70, 71, 75

Trinidad high octane	34

Tripoli	13

true boiling point	13, 56, 70, 71

Tunis	13

U

U.K.	28

U.S.A. Embassy	4

ultra violet spectroscopic	25, 66

ultra violet spectroscopic analysis	17

Universal Oil Products	42

University of Birmingham	4

unpolymerised	49

unsaturated	14, 25, 62

UV Spectroscopic Analysis	33, 38

V

vapour phase	28, 50, 51, 52, 62

vapour phase hydrogenation	14

Vapour Pressure	12, 14, 68

Vauxhall Lab	21, 24

Venezuelan	27, 35, 69, 72, 88

Venezuelan ‘A’	34

Venezuelan ‘B’	35, 38

virgin gasolines	26

virgin petroleum gasolines	10

volatility	9, 12, 13, 51, 53, 56, 62, 67

W

weak mixture	6

weathered	15, 22, 65

Welheim	11, 28

Wilford	4

Wright	4

X

xylenes	17, 26, 27, 28, 30, 38

xylidene	13, 18

# Research Sources

Research Studies Institute U.S.A.F. Historical Division Archives Branch - Enemy Oil Committee Report 1944 USAF A5464-0560-0635 Item 5 German Avgas – British War Cabinet Technical Committee - Enemy Oils and Fuels